| NPRR Number | 768 | NPRR Title | Revisions to Real-Time On-Line Reliability Deployment Price Adder Categories |
| --- | --- | --- | --- |
| Date of Decision |  | July 27, 2017 |  |
| Action |  | Tabled |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | Upon system implementation |  |
| Priority and Rank Assigned |  | Priority – 2017; Rank – 2050 |  |
| Nodal Protocol Sections Requiring Revision |  | 6.3.2, Activities for Real-Time Operations 6.5.7.3.1, Determination of Real-Time On-Line Reliability Deployment Price Adder |  |
| Related Documents Requiring Revision/ Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) revises the categories of ERCOT initiated actions that trigger the Real-Time On-Line Reliability Deployment Price Adder pricing run in order for prices to reflect current system conditions. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) |  |
| Business Case |  | This NPRR will address the impacts of ERCOT initiated actions that may cause prices to not reflect system conditions in Real-Time. By including these additional categories of ERCOT actions, the pricing signals in times of scarcity will be more accurate and have a positive impact on Resource adequacy. |  |
| Credit Work Group Review |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR768 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| PRS Decision |  | On 5/12/16, PRS unanimously voted to table NPRR768 and refer the issue to WMS. All Market Segments were present for the vote. On 6/15/17, PRS voted to recommend approval of NPRR768 as amended by the 6/1/17 TIEC Comments and as revised by PRS. There was one opposing vote from the Consumer Market Segment and one abstention from the Independent Power Marketer (IPM) Market Segment. All Market Segments were present for the vote. On 7/20/17, PRS voted to endorse and forward to TAC the 6/15/17 PRS Report and Revised Impact Analysis for NPRR768, with a recommended priority of 2017 and a rank of 2050. There was one opposing vote from the Consumer Market Segment. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 5/12/16, participants noted an open rulemaking at the Public Utility Commission of Texas (PUCT) relating to this issue and requested tabling of NPRR768 until that process is resolved. On 6/15/17, participants reviewed the 6/1/17 TIEC comments and determined additional desktop edits were needed to clarify Block Load Transfers (BLTs) as ‘registered’ in paragraph (h) of Section 6.5.7.3.1. On 7/20/17, participants reviewed the Business Case and Impact Analysis. |  |
| TAC Decision |  | On 7/27/17, TAC voted to table NPRR768 for one month and refer the issue to WMS. There were two opposing votes from the Independent Power Marketer (Shell and EDF Energy Services) Market Segments. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 7/27/17, participants discussed concerns as noted in the 6/1/17 TIEC comments; whether or not NPRR768 addresses an out of market action; and if it would have an economic impact that could potentially keep prices above the cap for longer than necessary. TAC requested that WMS review the issue of potential price impacts of large Direct Current Ties (DC Ties), and provide comments back to TAC. |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR768. |  |

| Sponsor |  |
| --- | --- |
| Name | Pamela Shaw |
| E-mail Address | Pamela.shaw@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4584 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.Butterfield@ercot.com |
| Phone Number | 512-246-6521 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| TCPA 051116 | Proposed edits to expand the scope of NPRR768 to include firm Load shed and to broaden Direct Current Tie (DC Tie) references to include both imports and curtailment of exports. |
| ERCOT 062016 | Requested PRS continue to table NPRR768. |
| Koch 041217 | Expressed support for NPRR768 as amended by the 5/11/16 TCPA comments. |
| ERCOT 041817 | Clarified the impacts of BLTs ordered by ERCOT during a declared emergency would be considered in the Real-Time On-Line Reliability Deployment Price Adder. |
| STEC 042517 | Proposed edits to identify when the Real -Time On-Line Reliability Deployment Price Adder should be calculated for DC Tie imports or BLTs. |
| WMS 050417 | Requested PRS continue to table NPRR768. |
| TIEC 060117 | Stated opposition to NPRR768 and provided language to ensure price adjustments for “emergency” conditions are symmetrical and fair to customers if this proposal moves forward. |
| WMS 060817 | Endorsed NPRR768 as amended by the 6/1/17 TIEC comments. |

Market Rules Notes

Please note that the baseline Protocol language in the following sections has been updated to reflect the incorporation of the following NPRR(s) into the Protocols:

- NPRR712, Clarification of RUC Process (unboxed 10/1/16)

  - Section 6.3.2

- NPRR744, RUC Trigger for the Reliability Deployment Price Adder and Alignment with RUC Settlement (incorporated 5/1/16 and unboxed 6/1/17)

  - Section 6.5.7.3.1

- NPRR754, Revise Load Distribution Factors Report Posting Frequency (incorporated 7/1/16 and unboxed 12/9/16)

  - Section 6.3.2

- NPRR789, Publish All Mid-Term Load Forecast Results (incorporated 11/1/16 and unboxed 6/29/17)

  - Section 6.3.2

- NPRR797, Actual Load by Forecast Zone Report and Display (incorporated 1/1/17 and unboxed 6/29/17)

  - Section 6.3.2

- NPRR831, Inclusion of Private Use Networks in Load Zone Price Calculations (incorporated 7/1/17)

  - Section 6.3.2

Please note the following NPRRs propose revisions to Section 6.3.2:

- NPRR829, Incorporate Real-Time Non-Modeled Telemetered Net Generation by Load Zone into the Estimate of RTL

- NPRR842, Study Area Load Information

Proposed Protocol Language Revision

6.3.2	Activities for Real-Time Operations

(1)	Activities for Real-Time operations begin at the end of the Adjustment Period and conclude at the close of the Operating Hour.

(2)	The following table summarizes the timeline for the Operating Period and the activities of QSEs and ERCOT during Real-Time operations where “T” represents any instant within the Operating Hour.  The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section:

| Operating Period | QSE Activities | ERCOT Activities |
| --- | --- | --- |
| During the first hour of the Operating Period |  | Execute the Hour-Ahead Sequence, including HRUC, beginning with the second hour of the Operating Period Review the list of Off-Line Available Resources with a start-up time of one hour or less Review and communicate HRUC commitments and Direct Current Tie (DC Tie) Schedule curtailments Snapshot the Scheduled Power Consumption for Controllable Load Resources |
| Before the start of each SCED run | Update Output Schedules for DSRs | Validate Output Schedules for DSRs Execute Real-Time Sequence |
| SCED run |  | Execute SCED and pricing run to determine impact of reliability deployments on energy prices |
| During the Operating Hour | Telemeter the Ancillary Service Resource Responsibility for each Resource Acknowledge receipt of Dispatch Instructions Comply with Dispatch Instruction Review Resource Status to assure current state of the Resources is properly telemetered Update COP with actual Resource Status and limits and Ancillary Service Schedules Communicate Resource Forced Outages to ERCOT Communicate to ERCOT Resource changes to Ancillary Service Resource Responsibility via telemetry in the time window beginning 30 seconds prior to the five-minute clock interval and ending ten seconds prior to that five-minute clock interval | Communicate all binding Base Points, Dispatch Instructions, and the sum of each type of available reserves, including total Real-Time reserve amount for On-Line reserves, total Real-Time reserve amount for Off-Line reserves, Real-Time Reserve Price Adders for On-Line Reserves, and Real-Time Reserve Price Adders for Off-Line Reserves and LMPs for energy and Ancillary Services, and for the pricing run as described in Section 6.5.7.3.1, Determination of Real-Time On-Line Reliability Deployment Price Adder, the Reliability Unit Commitment (RUC)/Reliability Must-Run (RMR) MW relaxed, total Load Resource MW deployed that is added to the Demand, total Emergency Response Service (ERS) MW deployed that is added to the Demand, total emergency DC Tie MW that is added to or subtracted from the Demand, total Block Load Transfer (BLT) MW that is added to or subtracted from the Demand, total Low Ancillary Service Limit (LASL), total High Ancillary Service Limit (HASL), Real-Time On-Line Reliability Deployment Price Adder using Inter-Control Center Communications Protocol (ICCP) or Verbal Dispatch Instructions (VDIs) Monitor Resource Status and identify discrepancies between COP and telemetered Resource Status Restart Real-Time Sequence on major change of Resource or Transmission Element Status Monitor ERCOT total system capacity providing Ancillary Services Validate COP information Monitor ERCOT control performance Distribute by ICCP, and post on the MIS Public Area, System Lambda and the LMPs for each Resource Node, Load Zone and Hub, and the sum of each type of available reserves, including total Real-Time reserve amount for On-Line reserves, total Real-Time reserve amount for Off-Line reserves, Real-Time Reserve Price Adders for On-Line Reserves and Real-Time Reserve Price Adders for Off-Line Reserves, and for the pricing run as described in Section 6.5.7.3.1 the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to the Demand, total ERS MW deployed that is added to the Demand, total emergency DC Tie MW that is added to or subtracted from the Demand, total BLT MW that is added to or subtracted from the Demand, total On-Line LASL, total On-Line HASL, Real-Time On-Line Reliability Deployment Price Adder created for each SCED process. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post LMPs for each Electrical Bus on the MIS Public Area. These prices shall be posted immediately subsequent to deployment of Base Points from each binding SCED with the time stamp the prices are effective Post on the MIS Public Area the projected non-binding LMPs created by each SCED process for each Resource Node, the projected total Real-Time reserve amount for On-Line reserves and Off-Line reserves, the projected Real-Time On-Line Reserve Price Adders and Real-Time Off-Line Reserve Price Adders, and for the projected non-binding pricing runs as described in Section 6.5.7.3.1 the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to Demand, total emergency DC Tie MW that is added to or subtracted from the Demand, total BLT MW that is added to or subtracted from the Demand, total ERS MW deployed that are deployed that is added to the Demand, total LASL, total HASL, Real-Time On-Line Reliability Deployment Price Adder and the projected Hub LMPs and Load Zone LMPs. These projected prices shall be posted at a frequency of every five minutes from SCED for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections Post on the MIS Certified Area the projected non-binding Base Points for each Resource created by each SCED process. These projected non-binding Base Points shall be posted at a frequency of every five minutes from SCED for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections Post each hour on the MIS Public Area binding SCED Shadow Prices and active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs) Post the Settlement Point Prices for each Settlement Point immediately following the end of each Settlement Interval Post the Real-Time On-Line Reliability Deployment Price, Real-Time Reserve Price for On-Line Reserves and the Real-Time Reserve Price for Off-Line Reserves immediately following the end of each Settlement Interval Post parameters as required by Section 6.4.9, Ancillary Services Capacity During the Adjustment Period and in Real-Time, on the MIS Public Area |

(3)	At the beginning of each hour, ERCOT shall post on the MIS Public Area the following information:

(a)	Changes in ERCOT System conditions that could affect the security and dynamic transmission limits of the ERCOT System, including:

(i)	Changes or expected changes, in the status of Transmission Facilities as recorded in the Outage Scheduler for the remaining hours of the current Operating Day and all hours of the next Operating Day; and

(ii)	Any conditions such as adverse weather conditions as determined from the ERCOT-designated weather service;

(b)	Updated system-wide Mid-Term Load Forecasts (MTLFs) for all forecast models available to ERCOT Operations, as well as an indicator for which forecast was in use by ERCOT at the time of publication;

(c)	The quantities of RMR Services deployed by ERCOT for each previous hour of the current Operating Day; and

(d)	Total ERCOT System Demand, from Real-Time operations, integrated over each Settlement Interval.

(4)	No later than 0600, ERCOT shall post on the MIS Public Area the actual system Load by Weather Zone and actual system Load by Forecast Zone for each hour of the previous Operating Day.

(5)	ERCOT shall provide notification to the market and post on the MIS Public Area Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads.  This report will be published when updates to the Load distribution factors are made.

[NPRR831: Replace paragraph (5) above with the following upon system implementation:] (5) ERCOT shall provide notification to the market and post on the MIS Public Area Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads. This report will be published when updates to the Load distribution factors are made. Private Use Network net Load will be redacted from this posting.

6.5.7.3.1	Determination of Real-Time On-Line Reliability Deployment Price Adder

(1)	The following categories of reliability deployments are considered in the determination of the Real-Time On-Line Reliability Deployment Price Adder:

(a)	RUC-committed Resources, except for those whose QSEs have opted out of RUC Settlement in accordance with paragraph (11) of Section 5.5.2, Reliability Unit Commitment (RUC) Process;

(b)	RMR Resources that are On-Line, including capacity secured to prevent an Emergency Condition pursuant to paragraph (2) of Section 6.5.1.1, ERCOT Control Area Authority;

(c)	Deployed Load Resources other than Controllable Load Resources;

(d)	Deployed Emergency Response Service (ERS);

(e)	Real-Time DC Tie imports during an EEA; and

(f)	Energy delivered to ERCOT through registered Block Load Transfers (BLTs)  during an EEA;

(g)	Real-Time DC Tie exports to address emergency conditions in the receiving electric grid;

(h)	Energy delivered from ERCOT to another power pool through registered Block Load Transfers (BLTs) during emergency conditions in the receiving electric grid.

(2)	The Real-Time On-Line Reliability Deployment Price Adder is an estimation of the impact to energy prices due to the above categories of reliability deployments.  For intervals where there are reliability deployments as described in paragraph (1) above, after the two-step SCED process and also after the Real-Time On-Line Reserve Price Adder and Real-Time Off-Line Reserve Price Adder have been determined, the Real-Time On-Line Reliability Deployment Price adder is determined as follows:

(a)	For RUC-committed Resources with a telemetered Resource Status of ONRUC and for RMR Resources that are On-Line, set the LSL, LASL, and LDL to zero.

(b) 	For all other Generation Resources excluding ones with a telemetered status of ONRUC, ONTEST, STARTUP, SHUTDOWN,  and also excluding RMR Resources that are On-Line and excluding Generation Resources with a telemetered output less than 95% of LSL:

(i)  	Set LDL to the greater of Aggregated Resource Output - (60 minutes * SCED Down Ramp Rate), or LASL; and

(ii)       Set HDL to the lesser of Aggregated Resource Output + (60 minutes*SCED Up Ramp Rate), or HASL.

(c) 	For all Controllable Load Resources excluding ones with a telemetered status of OUTL:

(i)  	Set LDL to the greater of Aggregated Resource Output - (60 minutes * SCED Up Ramp Rate), or LASL; and

(ii)       Set HDL to the lesser of Aggregated Resource Output + (60 minutes*SCED Down Ramp Rate), or HASL.

(d)	Add the deployed MW from Load Resources other than Controllable Load Resources to GTBD linearly ramped over the 10-minute ramp period.  The amount of deployed MW is calculated from the Resource telemetry and from applicable deployment instructions in Extensible Markup Language (XML) messages. ERCOT shall generate a linear bid curve defined by a price/quantity pair of $300/MWh for the first MW of Load Resources deployed and a price/quantity pair of $700/MWh for the last MW of Load Resources deployed in each SCED execution.  After recall instruction, the amount of MW added to GTBD during the restoration period will be determined by validated telemetry.  The TAC shall review the validity of the prices for the bid curve at least annually.

(e) 	Add the deployed MW from ERS to GTBD. The amount of deployed MW is determined from the XML messages and ERS contracts. After recall, an approximation of the amount of un-restored ERS shall be used. After ERCOT recalls each group, GTBD shall be adjusted to reflect the restoration of load using a linear curve over the ten hour restoration period. The restoration period shall be reviewed by TAC at least annually, and ERCOT may recommend a new restoration period to reflect observed historical restoration patterns.

(f)	Add the MW from Real-Time emergency DC Tie imports to GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the ERCOT Operator.

(g)	Subtracted the MW from Real-Time emergency DC Tie exports from GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the relevant grid operator.

(h)	Add the MW from energy delivered to ERCOT through registered BLTs to GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the ERCOT Operator.

(i)	Subtracted the MW from energy delivered from ERCOT to another region through registered BLTs from GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the relevant grid operator.

(j)	Perform a SCED with changes to the inputs in items (a) through (i), above, considering only Competitive Constraints and the non-mitigated Energy Offer Curves.

(k)	Perform mitigation on the submitted Energy Offer Curves using the LMPs from the previous step as the reference LMP.

(l)	Perform a SCED with the changes to the inputs in items (a) through (i), above, considering both Competitive and Non-Competitive Constraints and the mitigated Energy offer Curves.

(m)	Determine the positive difference between the System Lambda from item (l) above and the System Lambda of the second step in the two-step SCED process described in paragraph (10)(b) of Section 6.5.7.3, Security Constrained Economic Dispatch.

(n)	Determine the amount given by the Value of Lost Load (VOLL) minus the sum of the System Lambda of the second step in the two step SCED process described in paragraph (10)(b) of Section 6.5.7.3 and the Real-Time On-Line Reserve Price Adder.

(o)	The Real-Time On-Line Reliability Deployment Price Adder is the minimum of items (m) and (n) above.