| NPRR Number | 829 | NPRR Title | Incorporate Real-Time Non-Modeled Telemetered Net Generation by Load Zone into the Estimate of RTL |
| --- | --- | --- | --- |
| Date of Decision |  | August 24, 2017 |  |
| Action |  | Tabled |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | Upon system implementation |  |
| Priority and Rank Assigned |  | Priority – 2019; Rank – 2510 |  |
| Nodal Protocol Sections Requiring Revision |  | - 1.3.1.1, Items Considered Protected Information - 6.3.2, Activities for Real-Time Operations 6.5.5.2, Operational Data Requirements 16.11.4.3.2, Real-Time Liability Estimate |  |
| Related Documents Requiring Revision/ Related Revision Requests |  | ERCOT Nodal ICCP Communications Handbook |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) provides a mechanism for a Qualified Scheduling Entity (QSE) with non-modeled generation to inform ERCOT in a timely manner the net generation to the ERCOT grid from their Non-Modeled Generators so that the output can be considered in the estimate of Real-Time Liability (RTL). Absent this NPRR, output from Non-Modeled Generators is not included in estimating the QSE’s RTL, and the QSE is required to post a superfluous amount of collateral. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Currently it is difficult for non-modeled generation to hedge in the Day-Ahead Market (DAM) due to potential superfluous collateral calls during high priced Real-Time Market (RTM) events. Using telemetered data from non-modeled generation will allow ERCOT to more accurately calculate QSE collateral requirements. This NPPR will increase DAM liquidity due to increased participation of non-modeled generation in the DAM. Implementing this NPRR has the potential to allow ERCOT to gain near Real-Time transparency into non-modeled generation which could increase their ability in analyzing Real-Time system conditions. Furthermore, ERCOT will better understand the true position of the QSE by Operating Day and will also better understand their cash flow. |  |
| Credit Work Group Review |  | See the 5/17/17 Credit Work Group (Credit WG) comments. |  |
| PRS Decision |  | On 5/11/17, PRS unanimously voted to table NPRR829 and refer the issue to the Credit Work Group (Credit WG) and WMS. All Market Segments were present for the vote. On 6/15/17, PRS unanimously voted to table NPRR829 for one month. All Market Segments were present for the vote. On 7/20/17, PRS voted to recommend approval of NPRR829 as amended by the 7/14/17 ERCOT comments and as revised by PRS. There was one abstention from the Consumer (Occidental) Market Segment. All Market Segments were present for the vote. On 8/11/17, PRS voted to endorse and forward to TAC the 7/20/17 PRS Report as amended by the 8/9/17 ERCOT comments and Impact Analysis for NPRR829 with a recommended priority of 2019 and rank of 2510. There was one opposing vote from the Municipal (CPS) Market Segment and one abstention from the Municipal (DME) Market Segment. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 5/11/17, participants noted that the term Non-Modeled Generator imprecisely encompasses the type of Resource contemplated by this NPRR and that additional, but separate, work should be done to resolve ambiguity in these related concepts. Participants acknowledged the 5/10/17 Shell Energy comments and that the Credit WG has already begun review of NPRR829. Participants requested the Credit WG continue its review of this NPRR, and for WMS to also review it. On 6/15/17, there was no discussion. On 7/20/17, participants reviewed the 7/14/17 ERCOT comments and provided recommendations for the values of the minimum number of QSEs and megawatts necessary before the aggregate amount of net injection from Non-Modeled Generators that provide Real-Time telemetry to ERCOT can be posted to the Market Information System (MIS). On 8/11/17, participants reviewed the Impact Analysis and Business Case for NPRR829. Some participants expressed concern regarding the budgetary impact and the cost benefit of NPRR829. |  |
| TAC Decision |  | On 8/24/17, TAC unanimously voted to table NPRR829. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 8/24/17, ERCOT staff noted that the Impact Analysis for NPRR829 was being further reviewed and that a revised Impact Analysis would be posted once the review was complete. |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR829 as it improves the calculation of collateral requirements and transparency into non-modeled generation. |  |

| Sponsor |  |
| --- | --- |
| Name | Corey Amthor |
| E-mail Address | camthor@erockhold.com |
| Company | Enchanted Rock / Electranet QSE I, LLC |
| Phone Number | 713-429-4091 |
| Cell Number | 816-225-0136 |
| Market Segment | Independent Retail Electric Provider (IREP) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Kelly Landry |
| E-Mail Address | klandry@ercot.com |
| Phone Number | 512-248-4630 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Shell Energy 051017 | Proposed revisions that would require the posting of aggregate Distributed Generation (DG) participation to the MIS Public Area. |
| Shell Energy 051617 | Proposed additional revisions to correct inconsistencies in the use of the terms “Non-Modeled Generator” and “Distributed Generation (DG),” and other administrative corrections. |
| Credit WG 051717 | Endorsed NPRR829 as submitted, without prejudice to the 5/16/17 Shell Energy Comments. |
| WMS 060817 | Endorsed NPRR829 as amended by the 5/16/17 Shell Energy comments. |
| ERCOT 071417 | Expressed support for NPRR829 and provided revisions to clarify that submission of this telemetry is voluntary. |
| ERCOT 080917 | Proposed that Real-Time telemetry of net generation for Non-Modeled Generators be used in RTL and proposed to more clearly delineate what Non-Modeled Generator information will be considered Protected Information. |

Market Rules Notes

Please note that the baseline language in the following section(s) has been updated to reflect the incorporation of the following NPRRs into the Protocols:

- NPRR831, Inclusion of Private Use Networks in Load Zone Price Calculations (incorporated 7/1/17)

  - Section 6.3.2

Please also note that the following NPRRs also propose revisions to the following Sections:

- NPRR768, Revisions to Real-Time On-Line Reliability Deployment Price Adder Categories

  - Section 6.3.2

- NPRR842, Study Area Load Information

  - Section 6.3.2

Proposed Protocol Language Revision

- 1.3.1.1	Items Considered Protected Information

(1)	Subject to the exclusions set out in Section 1.3.1.2, Items Not Considered Protected Information, and in Section 3.2.5, Publication of Resource and Load Information, “Protected Information” is information containing or revealing any of the following:

(a)	Base Points, as calculated by ERCOT.  The Protected Information status of this information shall expire 60 days after the applicable Operating Day;

(b)	Bids, offers, or pricing information identifiable to a specific Qualified Scheduling Entity (QSE) or Resource.  The Protected Information status of part of this information shall expire 60 days after the applicable Operating Day, as follows:

(i)	Ancillary Service Offers by Operating Hour for each Resource for all Ancillary Services submitted for the Day-Ahead Market (DAM) or any Supplemental Ancillary Services Market (SASM);

(ii)	The quantity of Ancillary Service offered by Operating Hour for each Resource for all Ancillary Service submitted for the DAM or any SASM; and

(iii)	Energy Offer Curve prices and quantities for each Settlement Interval by Resource.  The Protected Information status of this information shall expire within seven days after the applicable Operating Day if required to be posted as part of paragraph (5) of Section 3.2.5 and within two days after the applicable Operating Day if required to be posted as part of paragraph (6) of Section 3.2.5;

(c)	Status of Resources, including Outages, limitations, or scheduled or metered Resource data.  The Protected Information status of this information shall expire 60 days after the applicable Operating Day;

(d)	Current Operating Plans (COPs).  The Protected Information status of this information shall expire 60 days after the applicable Operating Day;

(e)	Ancillary Service Trades, Energy Trades, and Capacity Trades identifiable to a specific QSE or Resource.  The Protected Information status of this information shall expire 180 days after the applicable Operating Day;

(f)	Ancillary Service Schedules identifiable to a specific QSE or Resource.  The Protected Information status of this information shall expire 60 days after the applicable Operating Day;

(g)	Dispatch Instructions identifiable to a specific QSE or Resource, except for Reliability Unit Commitment (RUC) commitments and decommitments as provided in Section 5.5.3, Communication of RUC Commitments and Decommitments.  The Protected Information status of this information shall expire 180 days after the applicable Operating Day;

(h)	Raw and Adjusted Metered Load (AML) data (demand and energy) identifiable to:

(i)	A specific QSE or Load Serving Entity (LSE).  The Protected Information status of this information shall expire 180 days after the applicable Operating Day; or

(ii)	A specific Customer or Electric Service Identifier (ESI ID);

(i)	Wholesale Storage Load (WSL) data identifiable to a specific QSE.  The Protected Information status of this information shall expire 180 days after the applicable Operating Day;

(j)	Settlement Statements and Invoices identifiable to a specific QSE.  The Protected Information status of this information shall expire 180 days after the applicable Operating Day;

(k)	Number of ESI IDs identifiable to a specific LSE.  The Protected Information status of this information shall expire 365 days after the applicable Operating Day;

(l)	Information related to generation interconnection requests, to the extent such information is not otherwise publicly available.  The Protected Information status of certain generation interconnection request information expires as provided in Section 1.3.3, Expiration of Confidentiality;

(m)	Resource-specific costs, design and engineering data, including such data submitted in connection with a verifiable cost appeal;

(n)	Congestion Revenue Right (CRR) credit limits, the identity of bidders in a CRR Auction, or other bidding information identifiable to a specific CRR Account Holder.  The Protected Information status of this information shall expire as follows:

(i)	The Protected Information status of the identities of CRR bidders that become CRR Owners and the number and type of CRRs that they each own shall expire at the end of the CRR Auction in which the CRRs were first sold; and

(ii)	The Protected Information status of all other CRR information identified above in item (n) shall expire six months after the end of the year in which the CRR was effective.

(o)	Renewable Energy Credit (REC) account balances.  The Protected Information status of this information shall expire three years after the REC Settlement period ends;

(p)	Credit limits identifiable to a specific QSE;

(q)	Any information that is designated as Protected Information in writing by Disclosing Party at the time the information is provided to Receiving Party except for information that is expressly designated not to be Protected Information by Section 1.3.1.2 or that, pursuant to Section 1.3.3, Expiration of Confidentiality, is no longer confidential;

(r)	Any information compiled by a Market Participant on a Customer that in the normal course of a Market Participant’s business that makes possible the identification of any individual Customer by matching such information with the Customer’s name, address, account number, type of classification service, historical electricity usage, expected patterns of use, types of facilities used in providing service, individual contract terms and conditions, price, current charges, billing record, or any other information that a Customer has expressly requested not be disclosed (“Proprietary Customer Information”) unless the Customer has authorized the release for public disclosure of that information in a manner approved by the Public Utility Commission of Texas (PUCT).  Information that is redacted or organized in such a way as to make it impossible to identify the Customer to whom the information relates does not constitute Proprietary Customer Information;

(s)	Any software, products of software, or other vendor information that ERCOT is required to keep confidential under its agreements;

(t)	QSE, Transmission Service Provider (TSP), and Distribution Service Provider (DSP) backup plans collected by ERCOT under the Protocols or Other Binding Documents;

(u)	Direct Current Tie (DC Tie) information provided to a TSP or DSP under Section 9.17.2, Direct Current Tie Schedule Information;

(v)	Any Texas Standard Electronic Transaction (TX SET) transaction submitted by an LSE to ERCOT or received by an LSE from ERCOT.  This paragraph does not apply to ERCOT’s compliance with:

(i)	PUCT Substantive Rules on performance measure reporting;

(ii)	These Protocols or Other Binding Documents; or

(iii)	Any Technical Advisory Committee (TAC)-approved reporting requirements;

(w)	Information concerning a Mothballed Generation Resource’s probability of return to service and expected lead time for returning to service submitted pursuant to Section 3.14.1.9, Generation Resource Status Updates;

(x)	Information provided by Entities under Section 10.3.2.4, Reporting of Net Generation Capacity;

(y)	Alternative fuel reserve capability and firm gas availability information submitted pursuant to Section 6.5.9.3.1, Operating Condition Notice, Section 6.5.9.3.2, Advisory, and Section 6.5.9.3.3, Watch, and as defined by the Operating Guides;

(z)	Non-public financial information provided by a Counter-Party to ERCOT pursuant to meeting its credit qualification requirements as well as the QSE’s form of credit support;

(aa)	ESI ID, identity of Retail Electric Provider (REP), and MWh consumption associated with transmission-level Customers that wish to have their Load excluded from the Renewable Portfolio Standard (RPS) calculation consistent with Section 14.5.3, End-Use Customers, and subsection (j) of P.U.C. Subst. R. 25.173, Goal for Renewable Energy;

(bb)	Generation Resource emergency operations plans and weatherization plans;

(cc)     Information provided by a Counter-Party under Section 16.16.3, Verification of Risk Management Framework;

(dd)	Any data related to Load response capabilities that are self-arranged by the LSE or pursuant to a bilateral agreement between a specific LSE and its Customers, other than data either related to any service procured by ERCOT or non-LSE-specific aggregated data.  Such data includes pricing, dispatch instructions, and other proprietary information of the Load response product;

(ee)	Status of Non-Modeled Generators, including Outages, limitations, schedules, metered output data, or data telemetered for use in the calculation of Real-Time Liability (RTL) as described in Section 16.11.4.3.2, Real-Time Liability Estimate, except that ERCOT may disclose metered output data from a Non-Modeled Generator as part of an extract or forwarded TX SET transaction provided to the LSE associated with the ESI ID of the Premise where the Non-Modeled Generator is located.  The Protected Information status of this information shall expire 60 days after the applicable Operating Day;

(ff)	Any documents or data submitted to ERCOT in connection with an Alternative Dispute Resolution (ADR) proceeding.  The Protected Information status of this information shall expire upon ERCOT’s issuance of a Market Notice indicating the disposition of the ADR proceeding pursuant to paragraph (1) of Section 20.8, Resolution of an Alternative Dispute Resolution Proceeding and Notification to Market Participants, except to the extent the information continues to qualify as Protected Information pursuant to another paragraph of this Section 1.3.1.1; and

(gg)	Reasons for and future expectations of overrides to a specific Resource’s High Dispatch Limit (HDL) or Low Dispatch Limit (LDL).  The Protected Information status of this information shall expire 60 days after the applicable Operating Day.

- 6.3.2	Activities for Real-Time Operations

(1)	Activities for Real-Time operations begin at the end of the Adjustment Period and conclude at the close of the Operating Hour.

(2)	The following table summarizes the timeline for the Operating Period and the activities of QSEs and ERCOT during Real-Time operations where “T” represents any instant within the Operating Hour.  The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section:

| Operating Period | QSE Activities | ERCOT Activities |
| --- | --- | --- |
| During the first hour of the Operating Period |  | Execute the Hour-Ahead Sequence, including HRUC, beginning with the second hour of the Operating Period Review the list of Off-Line Available Resources with a start-up time of one hour or less Review and communicate HRUC commitments and Direct Current Tie (DC Tie) Schedule curtailments Snapshot the Scheduled Power Consumption for Controllable Load Resources |
| Before the start of each SCED run | Update Output Schedules for DSRs | Validate Output Schedules for DSRs Execute Real-Time Sequence |
| SCED run |  | Execute SCED and pricing run to determine impact of reliability deployments on energy prices |
| During the Operating Hour | Telemeter the Ancillary Service Resource Responsibility for each Resource Acknowledge receipt of Dispatch Instructions Comply with Dispatch Instruction Review Resource Status to assure current state of the Resources is properly telemetered Update COP with actual Resource Status and limits and Ancillary Service Schedules Communicate Resource Forced Outages to ERCOT Communicate to ERCOT Resource changes to Ancillary Service Resource Responsibility via telemetry in the time window beginning 30 seconds prior to the five-minute clock interval and ending ten seconds prior to that five-minute clock interval | Communicate all binding Base Points, Dispatch Instructions, and the sum of each type of available reserves, including total Real-Time reserve amount for On-Line reserves, total Real-Time reserve amount for Off-Line reserves, Real-Time Reserve Price Adders for On-Line Reserves, and Real-Time Reserve Price Adders for Off-Line Reserves and LMPs for energy and Ancillary Services, and for the pricing run as described in Section 6.5.7.3.1, Determination of Real-Time On-Line Reliability Deployment Price Adder, the total Reliability Unit Commitment (RUC)/Reliability Must-Run (RMR) MW relaxed, total Load Resource MW deployed that is added to the Demand, total Emergency Response Service (ERS) MW deployed that is added to the Demand, total Low Ancillary Service Limit (LASL), total High Ancillary Service Limit (HASL), Real-Time On-Line Reliability Deployment Price Adder using Inter-Control Center Communications Protocol (ICCP) or Verbal Dispatch Instructions (VDIs) Monitor Resource Status and identify discrepancies between COP and telemetered Resource Status Restart Real-Time Sequence on major change of Resource or Transmission Element Status Monitor ERCOT total system capacity providing Ancillary Services Validate COP information Monitor ERCOT control performance Distribute by ICCP, and post on the MIS Public Area, System Lambda and the LMPs for each Resource Node, Load Zone and Hub, and the sum of each type of available reserves, including total Real-Time reserve amount for On-Line reserves, total Real-Time reserve amount for Off-Line reserves, Real-Time Reserve Price Adders for On-Line Reserves and Real-Time Reserve Price Adders for Off-Line Reserves, and for the pricing run as described in Section 6.5.7.3.1 the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to the Demand, total ERS MW deployed that is added to the Demand, total On-Line LASL, total On-Line HASL, Real-Time On-Line Reliability Deployment Price Adder created for each SCED process. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post LMPs for each Electrical Bus on the MIS Public Area. These prices shall be posted immediately subsequent to deployment of Base Points from each binding SCED with the time stamp the prices are effective Post every 15 minutes on the MIS Public Area the aggregate net injection from Non-Modeled Generators that provide Real-Time telemetry to ERCOT, consistent with paragraph (12) of Section 6.5.5.2, Operational Data Requirements. This data shall not be displayed if less than five QSEs or less than 750 megawatts of net injection utilize the option to telemeter Real-Time output for use in the calculation of Real-Time Liability (RTL) as described in Section 16.11.4.3.2, Real-Time Liability Estimate. Post on the MIS Public Area the projected non-binding LMPs created by each SCED process for each Resource Node, the projected total Real-Time reserve amount for On-Line reserves and Off-Line reserves, the projected Real-Time On-Line Reserve Price Adders and Real-Time Off-Line Reserve Price Adders, and for the projected non-binding pricing runs as described in Section 6.5.7.3.1 the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to Demand, total ERS MW deployed that are deployed that is added to the Demand, total LASL, total HASL, Real-Time On-Line Reliability Deployment Price Adder and the projected Hub LMPs and Load Zone LMPs. These projected prices shall be posted at a frequency of every five minutes from SCED for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections Post on the MIS Certified Area the projected non-binding Base Points for each Resource created by each SCED process. These projected non-binding Base Points shall be posted at a frequency of every five minutes from SCED for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections Post each hour on the MIS Public Area binding SCED Shadow Prices and active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs) Post the Settlement Point Prices for each Settlement Point immediately following the end of each Settlement Interval Post the Real-Time On-Line Reliability Deployment Price, Real-Time Reserve Price for On-Line Reserves and the Real-Time Reserve Price for Off-Line Reserves immediately following the end of each Settlement Interval Post parameters as required by Section 6.4.9, Ancillary Services Capacity During the Adjustment Period and in Real-Time, on the MIS Public Area |

(3)	At the beginning of each hour, ERCOT shall post on the MIS Public Area the following information:

(a)	Changes in ERCOT System conditions that could affect the security and dynamic transmission limits of the ERCOT System, including:

(i)	Changes or expected changes, in the status of Transmission Facilities as recorded in the Outage Scheduler for the remaining hours of the current Operating Day and all hours of the next Operating Day; and

(ii)	Any conditions such as adverse weather conditions as determined from the ERCOT-designated weather service;

(b)	Updated system-wide Mid-Term Load Forecasts (MTLFs) for all forecast models available to ERCOT Operations, as well as an indicator for which forecast was in use by ERCOT at the time of publication;

(c)	The quantities of RMR Services deployed by ERCOT for each previous hour of the current Operating Day; and

(d)	Total ERCOT System Demand, from Real-Time operations, integrated over each Settlement Interval.

(4)	No later than 0600, ERCOT shall post on the MIS Public Area the actual system Load by Weather Zone and actual system Load by Forecast Zone for each hour of the previous Operating Day.

(5)	ERCOT shall provide notification to the market and post on the MIS Public Area Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads.  This report will be published when updates to the Load distribution factors are made.

[NPRR831: Replace paragraph (5) above with the following upon system implementation:] (5) ERCOT shall provide notification to the market and post on the MIS Public Area Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads. This report will be published when updates to the Load distribution factors are made. Private Use Network net Load will be redacted from this posting.

6.5.5.2	Operational Data Requirements

(1)	ERCOT shall use Operating Period data to monitor and control the reliability of the ERCOT Transmission Grid and shall use it in network analysis software to predict the short-term reliability of the ERCOT Transmission Grid.  Each TSP, at its own expense, may obtain that Operating Period data from ERCOT or directly from QSEs.

(2)	A QSE representing a Generation Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time telemetry data to ERCOT for each Generation Resource.  ERCOT shall make that data available, in accordance with ERCOT Protocols, NERC Reliability Standards, and Governmental Authority requirements, to requesting TSPs and DSPs operating within ERCOT.  Such data must be provided to the requesting TSP or DSP at the requesting TSP’s or DSP’s expense, including:

(a)	Net real power (in MW) as measured by installed power metering or as calculated in accordance with the Operating Guides based on metered gross real power and conversion constants determined by the Resource Entity and provided to ERCOT through the Resource Registration process.  Net real power represents the actual generation of a Resource for all real power dispatch purposes, including use in Security-Constrained Economic Dispatch (SCED), determination of the High Ancillary Service Limit (HASL), High Dispatch Limit (HDL), Low Dispatch Limit (LDL) and Low Ancillary Service Limit (LASL), and is consistent with telemetered HSL, LSL and Non-Frequency Responsive Capacity (NFRC);

(b)	Gross real power (in MW) as measured by installed power metering or as calculated in accordance with the Operating Guides based on metered real power, which may include Supervisory Control and Data Acquisition (SCADA) metering, and conversions constants determined by the Resource Entity and provided to ERCOT through the Resource Registration process;

(c)	Gross Reactive Power (in Megavolt-Amperes reactive (MVAr));

(d)	Net Reactive Power (in MVAr);

(e)	Power to standby transformers serving plant auxiliary Load;

(f)	Status of switching devices in the plant switchyard not monitored by the TSP or DSP affecting flows on the ERCOT Transmission Grid;

(g)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(h)	Generation Resource breaker and switch status;

(i)	HSL (Combined Cycle Generation Resources) shall:

(i)	Submit the HSL of the current operating configuration; and

(ii)	When providing RRS, update the HSL as needed, to be consistent with Resource performance limitations of RRS provision;

(j)	NFRC currently available (unloaded) and included in the HSL of the Combined Cycle Generation Resource’s current configuration;

(k)	High Emergency Limit (HEL), under Section 6.5.9.2, Failure of the SCED Process;

(l)	Low Emergency Limit (LEL), under Section 6.5.9.2;

(m)	LSL;

(n)	Configuration identification for Combined Cycle Generation Resources;

(o)	Ancillary Service Schedule for each quantity of RRS and Non-Spin which is equal to the Ancillary Service Resource Responsibility minus the amount of Ancillary Service deployment;

(i)	For On-line Non-Spin, Ancillary Service Schedule shall be set to zero;

(ii)	For Off-Line Non-Spin and for On-Line Non-Spin using Off-Line power augmentation technology the Ancillary Service Schedule shall equal the Non-Spin obligation and then shall be set to zero within 20 minutes following Non-Spin deployment;

(p)	Ancillary Service Resource Responsibility for each quantity of Regulation Up (Reg-Up), Regulation Down (Reg-Down), RRS and Non-Spin.  The sum of Ancillary Service Resource Responsibility for all Resources in a QSE is equal to the Ancillary Service Supply Responsibility for that QSE;

(q)	Reg-Up and Reg-Down Services participation factors represent how a QSE is planning to deploy the Ancillary Service energy on a percentage basis to specific qualified Resource(s).  The Reg-Up and Reg-Down Services participation factors for a Resource providing Fast Responding Regulation Up Service (FRRS-Up) or Fast Responding Regulation Down Service (FRRS-Down) shall be zero; and

(r)	The designated Master QSE of a Generation Resource that has been split to function as two or more Split Generation Resources shall provide Real-Time telemetry for items (a), (b), (c), (d), (e), (g), and (h) above, PSS and AVR status for the total Generation Resource in addition to the Split Generation Resource the Master QSE represents.

(3)	For each Intermittent Renewable Resource (IRR), the QSE shall set the HSL equal to the current net output capability of the facility.  The net output capability should consider the net real power of the IRR generation equipment, IRR generation equipment availability, weather conditions, and whether the IRR net output is being affected by compliance with a SCED Dispatch Instruction.

(4)	For each Aggregate Generation Resource (AGR), the QSE shall telemeter the number of its generators online.

(5)	A QSE representing a Load Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time data to ERCOT for each Load Resource and ERCOT shall make the data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to the Load Resource’s host TSP or DSP at the TSP’s or DSP’s expense.  The Load Resource’s net real power consumption, Low Power Consumption (LPC) and Maximum Power Consumption (MPC) shall be telemetered to ERCOT using a positive (+) sign convention:

(a)	Load Resource net real power consumption (in MW);

(b)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(c)	Load Resource breaker status;

(d)	LPC (in MW);

(e)	MPC (in MW);

(f)	Ancillary Service Schedule (in MW) for each quantity of RRS and Non-Spin, which is equal to the Ancillary Service Resource Responsibility minus the amount of Ancillary Service deployment;

(g)	Ancillary Service Resource Responsibility (in MW) for each quantity of Reg-Up and Reg-Down for Controllable Load Resources, and RRS and Non-Spin for all Load Resources;

(h)	The status of the high-set under-frequency relay, if required for qualification;

(i)	For a Controllable Load Resource providing Non-Spin, the Scheduled Power Consumption that represents zero Ancillary Service deployments;

(j)	For a single-site Controllable Load Resource with registered maximum Demand response capacity of ten MW or greater, net Reactive Power (in MVAr);

(k)	Resource Status (Resource Status shall be ONRL if high-set under-frequency relay is active);

(l)	Reg-Up and Reg-Down services participation factor, which represents how a QSE is planning to deploy the Ancillary Service energy on a percentage basis to specific qualified Resource(s).  The Reg-Up and Reg-Down services participation factors for a Resource providing FRRS-Up or FRRS-Down shall be zero; and

(m)	For a Controllable Load Resource providing Non-Spin, the “Scheduled Power Consumption Plus Two Hours,” representing the QSE’s forecast of the Controllable Load Resource’s instantaneous power consumption for a point two hours in the future.

(6)	A QSE with Resources used in SCED shall provide communications equipment to receive ERCOT-telemetered control deployments.

(7)	A QSE providing any Regulation Service shall provide telemetry indicating the appropriate status of Resources providing Reg-Up or Reg-Down, including status indicating whether the Resource is temporarily blocked from receiving Reg-Up and/or Reg-Down deployments from the QSE.  This temporary blocking will be indicated by the enabling of the Raise Block Status and/or Lower Block Status telemetry points.

(a)	Raise Block Status and Lower Block Status are telemetry points used in transient unit conditions to communicate to ERCOT that a Resource’s ability to adjust its output has been unexpectedly impaired.

(b)	When one or both of the telemetry points are enabled for a Resource, ERCOT will cease using the regulation capacity assigned to that Resource for Ancillary Service deployment.

(c)	This hiatus of deployment will not excuse the Resource’s obligation to provide the Ancillary Services for which it has been committed.

(d)	These telemetry points shall only be utilized during unforeseen transient unit conditions such as plant equipment failures.  Raise Block Status and Lower Block Status shall only be enabled until the Resource operator has time to update the Resource limits and Ancillary Service telemetry to reflect the problem.

(e)	The Resource limits and Ancillary Service telemetry shall be updated as soon as practicable.  Raise Block Status and Lower Block Status will then be disabled.

(8)	Real-Time data for reliability purposes must be accurate to within three percent.  This telemetry may be provided from relaying accuracy instrumentation transformers.

(9)	Each QSE shall report the current configuration of combined-cycle Resources that it represents to ERCOT.  The telemetered Resource Status for a Combined Cycle Generation Resource may only be assigned a Resource Status of OFFNS if no generation units within that Combined Cycle Generation Resource are On-Line.

(10)	A QSE representing Combined Cycle Generation Resources shall provide ERCOT with the possible operating configurations for each power block with accompanying limits.  Combined Cycle Train power augmentation methods may be included as part of one or more of the registered Combined Cycle Generation Resource configurations.  Power augmentation methods may include:

(a)	Combustion turbine inlet air cooling methods;

(b)	Duct firing;

(c)	Other ways of temporarily increasing the output of Combined Cycle Generation Resources; and

(d)	For Qualifying Facilities (QFs), an LSL that represents the minimum energy available for Dispatch by SCED, in MW, from the Combined Cycle Generation Resource based on the minimum stable steam delivery to the thermal host plus a justifiable reliability margin that accounts for changes in ambient conditions.

(11)	A QSE representing Generation Resources other than Combined Cycle Generation Resources may telemeter an NFRC value for their Generation Resource only if the QSE or Resource Entity associated with that Generation Resource has first requested and obtained ERCOT’s approval of the Generation Resource’s NFRC quantity.

(12)	A QSE representing a Non-Modeled Generator that elects to include the net generation of the Non-Modeled Generator in the estimate of Real-Time Liability (RTL) shall provide ERCOT Real-Time telemetry of the net generation of the Non-Modeled Generator.

16.11.4.3.2	Real-Time Liability Estimate

(1)	ERCOT shall estimate RTL for an Operating Day as the sum of estimates for the following RTM Settlement charges and payments:

(a)	Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node, using Real-Time Metered Generation (RTMG) as generation estimate;

(b)	Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone, using 14 day or seven day old LRS for Load estimate and Real-Time telemetry of net generation as the generation estimate;

(c)	Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charge at a Hub;

(d)	Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import;

(e)	Section 6.6.3.6, Real-Time Energy Charge for DC Tie Export Represented by the QSE Under the Oklaunion Exemption;

(f)	Section 6.6.4, Real-Time Congestion Payment or Charge for Self-Schedules; and

(g)	Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time.