| SCR Number | 795 | SCR Title | Addition of Intra-Hour Wind Forecast to GTBD Calculation |
| --- | --- | --- | --- |
| Date Posted |  | November 28, 2017 |  |
| Requested Resolution |  | Normal |  |
| Supporting Protocol or Guide Section(s) |  | 6.5.7.3, Security Constrained Economic Dispatch |  |
| Other Document Reference/Source |  | None |  |
| System Change Description |  | ERCOT is in the process of adding an intra-hour wind forecast. This System Change Request (SCR) updates the formula used by the Resource Limit Calculator to calculate the Generation To Be Dispatched (GTBD) value to include the addition of a predicted 5-minute wind ramp (PWRR). This PWRR will be calculated from the intra-hour wind forecast times five and a configurable factor to capture the forecasted 5-minute wind ramp. This addition would help Security-Constrained Economic Dispatch (SCED) take into account the potential 5-minute wind ramp in its 5-minute Base-Points and would take the burden off of Regulation to cover the 5-minute gain or loss of generation from variations in wind and instead dispatch this energy economically. This change will also aid reducing frequency recovery duration following events that occur during times with significant wind up and down ramps. Although there will be inherent forecast error in the intra-hour forecast, this change will give a better indication to SCED via GTBD of the level of wind generation that will be available in the next 5 minutes, as compared to the current assumption that the wind generation will persist at the same level as at the beginning of the 5-minute interval. Once ERCOT begins to receive the intra-hour wind forecast from its vendor(s), ERCOT will analyze the 5-minute out forecast performance and accuracy of projected wind generation ramp direction before the intra hour wind forecast component is activated in the GTBD formula. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This SCR will improve the dispatch of Base-Points to Resources to account for the ramping of uncurtailed wind generation, will reduce the amount of Regulation Service deployed, and will aid in reducing frequency recovery duration following events that occur times with significant wind ramps. |  |

| Sponsor |  |
| --- | --- |
| Name | Sandip Sharma / Nitika Mago |
| E-mail Address | sandip.sharma@ercot.com / Nitika.Mago@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4298 / 512-248-6601 |
| Cell Number |  |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.butterfield@ercot.com |
| Phone Number | 512-248-6521 |

Business Case for Proposed System Change

Issue:

ERCOT’s current Generation To Be Dispatched (GTBD) calculation does not currently have a way of accounting for the ramping of wind generation in Real-Time.  This 5 minute forecast in GTBD would help Security-Constrained Economic Dispatch (SCED) better dispatch Base-Points to Generation Resources to account for the ramping of wind generation in the next 5 minutes.  It will also help reduce the amount of Regulation used to cover the 5 minute gain or loss of this wind generation.

Resolution:

Following modifications are needed:

- Add the K6 configurable factor together with the PWRR calculated in the Energy Management System (EMS).

- The GTBD equation should be as follows:

Generation To Be Dispatched = Total Gen + K1*10*System Load Frequency Bias + K2*[(net non-conforming Load) – (net filtered non-conforming Load)] + K3*5*PLDRR + K4*Regulation Deployed + K5*ACE Integral – K6*5*PWRR