| NPRR Number | 877 | NPRR Title | Use of Actual Interval Data for IDR ESI IDs for Initial Settlement |
| --- | --- | --- | --- |
| Date Posted |  | May 17, 2018 |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Sections Requiring Revision |  | 18.6.1, Interval Data Recorder Meter Mandatory Installation Requirements |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) provides a process to allow for the use of actual metered interval data for initial settlement of an Operating Day for Electric Service Identifiers (ESI IDs) that currently require BUSIDRRQ Load Profiles. Currently, meter data is sent monthly on the 867_03 for ESI IDs with BUSIDRRQ Load Profiles, so most of the data used for initial settlement for these ESI IDs is estimated using proxy day data. The ESI IDs are resettled with actual data during final Settlement on the 55th day following the Operating Day. The result is inaccurate initial Settlement. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | By receiving interval data on a daily basis rather than on a monthly basis for ESI IDs currently assigned as BUSIDRRQ Load Profiles, ERCOT will be able to more accurately determine initial Settlement by using actual data rather than proxy day data. |  |

| Sponsor |  |
| --- | --- |
| Name | Chris Rowley |
| E-mail Address | chris.rowley@oncor.com |
| Company | Oncor Electric Delivery Company LLC |
| Phone Number | 214-486-3432 |
| Cell Number | 972-849-6327 |
| Market Segment | Investor Owned Utility (IOU) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.Butterfield@ercot.com |
| Phone Number | 512-248-6521 |

Proposed Protocol Language Revision

- 18.6.1	Interval Data Recorder Meter Mandatory Installation Requirements

(1)	Unless otherwise provided in this Section, Interval Data Recorder (IDR) Meters are required and shall be installed and utilized for Settlement of Premises having either:

(a)	A peak Demand greater than 700 kW (or 700 kVA in CenterPoint Energy’s service territory); or

(b)	Service provided at transmission voltage (above 60 kV).

(2)	For the IDR Meter installation process, refer to the Retail Market Guide Section 7.13.2.2, Mandatory Interval Data Recorder Installation Process.

(3)	A Competitive Retailer (CR), upon a Customer’s request or with a Customer’s authorization, may have an IDR Meter installed and used for Settlement purposes at any associated Premise, in accordance with a Transmission and/or Distribution Service Provider’s (TDSP’s) tariff.  Except as stated in paragraph (5) below, IDR Meters in place or installed after September 1, 1999 shall be used for Settlement.  Once an IDR Meter is installed at a Premise and used for Settlement purposes, the given Premise shall continue to be settled with its interval data, except as stated in Section 18.6.2, Interval Data Recorder Meter Optional Removal.  If a Customer or CR requests installation of an IDR Meter, the same Customer may not request removal of the IDR Meter for a period of 12 consecutive months following such installation.

(4)	All non-metered Loads such as street lighting, regardless of the aggregation level, shall not be required to install IDR Meters under the IDR Meter Mandatory Installation Requirements.  These Loads shall be settled using Load Profiles.

(5)	IDR Meters previously used specifically for separating Non-Opt-In Entity (NOIE) Load from competitive Load shall be exempt from the requirement to use an IDR Meter for Settlement purposes, provided that the IDR Meter has been removed within 120 consecutive days after the NOIE has fully implemented Customer Choice.  IDR Meters used for NOIE separation that do not meet the IDR Meter Mandatory Installation Requirements shall not be used for Settlement purposes.

(6)	For Premises that meet the requirements identified in this Section, if both the CR and TDSP agree, upon a Customer’s request or with a Customer’s authorization, the TDSP shall:

(a)	Utilize a provisioned Advanced Meter or similarly functional meter for Customer’s Premise;

(b)	Assign the appropriate Load Profile, other than one with a BUSIDRRQ Profile Type Code, to Premise’s Electric Service Identifier (ESI ID);

(c)	Submit Settlement Quality Meter Data, which will be used for Settlement, using the ERCOT specified file format for the interval data only in accordance with Retail Market Guide Section 7.15, Advanced Meter Interval Data File Format and Submission;

(d)	Submit a MarkeTrak issue to notify ERCOT that the ESI ID is no longer subject to the requirements identified in this Section and Retail Market Guide Section 7.13.2, Interval Data Recorder Meter Installation Process; and

(e)	At least 45 days prior to the effective date that the Load Profile will change, submit a MarkeTrak issue to notify the CR of the date the Load Profile will change.

[NPRR877: Replace paragraph (6) above on January 1, 2020:] (6) For Premises that meet the requirements identified in this Section at a TDSP’s discretion, or upon a Customer’s request and TDSP’s approval, a TDSP shall: (a) Utilize a provisioned Advanced Meter or similarly functional meter for Customer’s Premise; (b) Assign the appropriate Load Profile, other than one with a BUSIDRRQ Profile Type Code, to Premise’s Electric Service Identifier (ESI ID); (c) Submit Settlement Quality Meter Data, which will be used for Settlement, using the ERCOT specified file format for the interval data only in accordance with Retail Market Guide Section 7.15, Advanced Meter Interval Data File Format and Submission; (d) Submit a MarkeTrak issue to notify ERCOT that the ESI ID is no longer subject to the requirements identified in this Section and Retail Market Guide Section 7.13.2, Interval Data Recorder Meter Installation Process; and (e) At least 45 days prior to the effective date that the Load Profile will change, submit a MarkeTrak issue to notify the CR of the date the Load Profile will change.

(7)	Transmission Service Providers (TSPs) and/or Distribution Service Providers (DSPs) responsible for any Load transfer schemes between the ERCOT Region and non-ERCOT Regions shall install an IDR Meter capable of measuring the Load served during the period the Load transfer is implemented.