| PGRR Number | 063 | PGRR Title | Transmission Interconnection Study |
| --- | --- | --- | --- |
| Date of Decision |  | July 12, 2018 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Planning Guide Sections Requiring Revision |  | 3.1.9, Transmission Interconnection Study (new) |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Planning Guide Revision Request (PGRR) outlines the process to evaluate the reliability impact of any transmission projects 100 kV or above that are expected to be in-service before the completion of the next Regional Transmission Plan and were not included in the current Regional Transmission Plan, a Regional Planning Group (RPG) project submission, or a Generation Interconnection or Change Request (GINR) study. The intent of the added language is to satisfy Requirement R1 of the North America Electric Reliability Corporation (NERC) Reliability Standard FAC-002-2, Facility Interconnection Studies. By definition, these projects would include Tier 4 transmission projects that are connected at or above 100 kV. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | As a NERC Planning Coordinator, ERCOT evaluates the reliability impact of any transmission interconnections through the RPG independent review process, the GINR process, and the Regional Transmission Plan. These planning assessments sufficiently capture any new projects that have a lead time of two years or longer. However, there may be certain Tier 4 projects that may have a short lead times (such as the ones built to serve end-use facilities). Per the FAC-002-2 R1, ERCOT as a NERC Planning Coordinator is required to study the reliability impact of such interconnections. This PGRR provides guidelines for such a study. |  |
| ROS Decision |  | On 5/3/18, ROS voted unanimously to table PGRR063 and refer the issue to the Planning Working Group (PLWG). All Market Segments were present for the vote. On 7/12/18, ROS voted unanimously to recommend approval of PGRR063 as revised by ROS. All Market Segments were present for the vote. |  |
| Summary of ROS Discussion |  | On 5/3/18, there was no discussion. On 7/12/18, participants discussed revisions proposed during PLWG discussion of PGRR063. |  |

| Sponsor |  |
| --- | --- |
| Name | Sandeep Borkar |
| E-mail Address | sborkar@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6642 |
| Cell Number | 501-246-0147 |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Brittney Albracht |
| E-Mail Address | Brittney.Albracht@ercot.com |
| Phone Number | 512-225-7027 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None |  |

Market Rules Notes

None

Proposed Guide Language Revision

- 3.1.9	Transmission Interconnection Study

(1)	ERCOT shall perform an annual transmission interconnection study to analyze the reliability impact of any transmission projects 100 kV or above that are expected to be in-service before the completion of the next Regional Transmission Plan and were not included in the current Regional Transmission Plan, an RPG project submission, or a Generation Interconnection or Change Request (GINR) study pursuant to Section 5, Generation Resource Interconnection or Change Request.

(a)	ERCOT shall identify a list of transmission projects 100 kV or above that need to be included in the annual transmission interconnection study and shall send the list to the TSPs that own the projects.

(b)	Within 20 Business Days of receipt of the list, each TSP that owns an identified transmission project shall send to ERCOT a PSS/E or PowerWorld formatted incremental change file to model the project in the current Regional Transmission Plan study cases.

(c)	ERCOT shall post a study report detailing its findings on the MIS Secure Area within 20 Business Days of completion.

(2)	After each Transmission Project Information Tracking (TPIT) update ERCOT shall identify a list of transmission projects 100 kV or above that are expected to be in-service before the completion of the next annual transmission interconnection study and were not included in the previous transmission interconnection study, Regional Transmission Plan, an RPG project submission, or a GINR study pursuant to Section 5.  ERCOT shall send the list to the TSPs that own the projects.

(a)	Within 20 Business Days of receipt of the list, each TSP that owns an identified transmission project shall send to ERCOT a study report detailing the reliability impact analysis it conducted for the project.  At a minimum the report shall identify the study base case(s), contingencies, and results.

(b)	ERCOT shall review the TSP reports and provide comments to the TSP within 20 Business Days of receipt.