| NPRR Number | 890 | NPRR Title | Correction to Calculation of Real-Time LMPs at Logical Resource Node for On-Line Combined Cycle Generation Resources |
| --- | --- | --- | --- |
| Date of Decision |  | October 9, 2018 |  |
| Action |  | Approved |  |
| Timeline |  | Urgent |  |
| Effective Date |  | October 10, 2018 |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Nodal Protocol Sections Requiring Revision |  | 6.6.1.1, Real-Time Settlement Point Price for a Resource Node |  |
| Related Documents Requiring Revision/Related Revision Requests |  | Other Binding Document Revision Request (OBDRR) 008, Correction to Calculation of Real-Time LMPs at Logical Resource Node for On-Line Combined Cycle Generation Resources |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) aligns Protocol price calculation formulas with ERCOT systems calculation of the Real-Time Locational Marginal Price (LMP) at a logical Resource Node for an On-Line Combined Cycle Generation Resource (“CCGR”). This alignment includes distinguishing between scenarios in which the CCGR is On-Line and when it is Off-Line. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Paragraph (2)(a) of Section 6.6.1.1 requires that Real-Time LMPs at the logical Resource Node for On-Line CCGRs be calculated using a weighted average of LMPs at the Resource Node of each of the generation units in the On-Line CCGRs, using the Real-Time telemetered outputs of the generation units to calculate the weight factor. ERCOT’s systems, however, do not have the capability to perform this precise calculation. Rather, from July 2, 2015 until August 8, 2018, this calculation was performed using a weighted average of the Resource Node LMPs of each unit in the On-Line CCGR, but it improperly used each unit’s High Reasonability Limit (HRL) to calculate the applied weight factor. As of August 8, 2018, this calculation is performed by calculating the CCGR’s LMP implicitly using a weighted average of the Shift Factors of each unit in the On-Line CCGR, using the Real-Time telemetered outputs of the generation units to calculate the weight factor. This latter methodology was also used to calculate this LMP from nodal go-live (December 1, 2010) until July 2, 2015. Revising Section 6.6.1.1 to align with ERCOT’s existing systems will result in no cost impact, whereas implementing system changes to align with existing Protocol language could result in potentially significant costs and take some time to implement. Accordingly, this urgent NPRR is proposed as a short-term resolution to the misalignment between Section 6.6.1.1 and ERCOT’s systems. However, further stakeholder discussions are expected to occur regarding whether changing ERCOT’s systems is the preferred long-term solution to this pricing issue. The proposed language also clarifies the formula for Off-Line CCGRs, as the Protocols are currently silent on that calculation. |  |
| Credit Work Group Review |  | See the 9/19/18 Credit Work Group (Credit WG) comments |  |
| PRS Decision |  | On 8/16/18, PRS unanimously voted to waive notice for NPRR890. PRS then unanimously voted to grant NPRR890 Urgent status. Finally, PRS unanimously voted to recommend approval of NPRR890 as submitted and to forward NPRR890 and the Impact Analysis to TAC. All Market Segments were present for all three votes. |  |
| Summary of PRS Discussion |  | On 8/16/18, ERCOT Staff reviewed the purpose of NPRR890 and reiterated a commitment to working with stakeholders on a long-term solution. |  |
| TAC Decision |  | On 9/26/18, TAC unanimously voted to recommend approval of NPRR890 as recommended by PRS in the 8/16/18 PRS Report with a proposed effective date of upon ERCOT Board approval. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 9/26/18, there was no discussion. |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR890. |  |
| Board Decision |  | On 10/9/18, the ERCOT Board approved NPRR890 as recommended by TAC in the 9/26/18 TAC Report. |  |

| Sponsor |  |
| --- | --- |
| Name | Dave Maggio |
| E-mail Address | David.Maggio@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6998 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | Cory.Phillips@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Credit WG 091918 | Noted NPRR890 has positive credit implications as it more accurately matches credit exposure to activity |

Market Rules Notes

None

Proposed Protocol Language Revision

- 6.6.1.1	Real-Time Settlement Point Price for a Resource Node

(1)	The Real-Time Settlement Point Price for a Resource Node Settlement Point is the time-weighted average of the sum of the Real-Time LMPs, Real-Time On-Line Reliability Deployment Price Adders, and the Real-Time On-Line Reserve Price Adders.  The Real-Time Settlement Point Price for a 15-minute Settlement Interval is calculated as follows:

RTSPP	=	Max (-$251, ((RNWFy * (RTLMPy + RTORPA y + RTORDPA y))))

Where the Resource Node weighting factor is:

RNWFy	=	TLMPy / TLMPy

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point PriceThe Real-Time Settlement Point Price at the Settlement Point for the 15-minute Settlement Interval. |
| RTORPA y | $/MWh | Real-Time On-Line Reserve Price Adder per intervalThe Real-Time On-Line Reserve Price Adder for the SCED interval y. |
| RTLMP y | $/MWh | Real-Time Locational Marginal Price per intervalThe Real-Time LMP at the Settlement Point for the SCED interval y. |
| RTORDPA y | $/MWh | Real-Time On-Line Reliability Deployment Price AdderThe Real-Time Price Adder that captures the impact of reliability deployments on energy prices for the SCED interval y. |
| RNWFy | none | Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |

(2)	The Real-Time Settlement Point Price at the logical Resource Node for a Combined Cycle Generation Resource shall be determined in accordance with paragraph (1) above using a Real-Time LMP calculated for the logical Resource Node in each SCED Interval as follows:

(a)	The Real-Time LMP for the logical Resource Node of a Combined Cycle Generation Train for each SCED interval is calculated as follows:

For an On-line Combined Cycle Generation Resource in the SCED interval:

RTLMP y = RTSL y –(RTAGSF OnlineCCGR, c, y * RTSPc, y)

Where:

RTAGSF OnlineCCGR, c, y = ∑CCGR_PhyR RTSF CCGR_PhyR, c, y * RTONCCGRWF CCGR_PhyR, y

For a Combined Cycle Train that is Off-Line in the SCED interval:

RTLMP y = ∑CCT_PhyR RTLMPCCT_PhyR, y * RTOFFCCGRWFCCT_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTLMP y | $/MWh | Real-Time Locational Marginal Price at a logical Resource Node for a Combined Cycle TrainThe Real-Time LMP at the Combined Cycle Generation Resource logical Resource Node for a SCED interval y. |
| RTSL y | $/MWh | Real-Time System LambdaThe Real-Time Shadow Price for the system power balance constraint for the SCED interval y. |
| RTAGSFOnlineCCGR, c, y | none | Real-Time Shift Factor for an On-line Combined Cycle Generation Resource The Real-Time aggregated Shift Factor at the logical Resource Node for an On-line Combined Cycle Generation Resource for the constraint c for the SCED interval y. |
| RTSFCCGR_PhyR, c, y | none | Real-Time Shift Factor at a generation unit Connectivity Node registered to the Combined Cycle Generation ResourceThe Real-Time Shift Factor at the Connectivity Node of an On-Line generation unit designated in a Combined Cycle Train registration for the On-Line Combine Cycle Generation Resource for the constraint c for the SCED interval y. |
| RTSP c, y | $/MWh | Real-Time Shadow Price for a binding transmission constraintThe Real-Time Shadow Price for the constraint c for the SCED interval y. |
| RTLMPCCT_PhyR, y | $/MWh | Real-Time Locational Marginal Price at a generation unit Resource Node registered to the Combined Cycle TrainThe Real-Time LMP at the Resource Node of a generation unit designated in a Combined Cycle Train registration for the SCED interval y. |
| RTONCCGRWFCCGR_PhyR, y | none | Real-Time On-Line Combined Cycle Generation Resource Weighting FactorThe Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource for the SCED interval y. |
| RTOFFCCGRWFCCT_PhyR, y | none | Real-Time Off-Line Combined Cycle Generation Resource Weighting FactorThe Real-Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration when the whole Combined Cycle Train is Off-Line for the SCED interval y. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for a Combined Cycle Generation Resource. |
| CCT_PhyR | none | A generation unit designated in a Combine Cycle Train registration |
| c | none | A binding transmission constraint for the SCED interval y. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

(b)	The weight factor for each generation unit registered in an On-Line Combined Cycle Generation Resource shall be the Real-Time power output telemetry in each SCED interval for each generation unit registered in the Combined Cycle Generation Resource divided by the total Real-Time power output telemetry for all of the generation units registered in the Combined Cycle Generation Resource.  For a Off-Line Combined Cycle Train, the weight factor for each generation unit designated in a Combined Cycle Train registration shall be its High Reasonability Limit (HRL) divided by the total sum of the HRL for all generation units registered in the Combined Cycle Train.

Where:

RTONCCGRWFCCGR_PhyR, y = TGCCGR_PhyR / ∑CCGR_PhyR TGCCGR_PhyR

RTOFFCCGRWF CCT_PhyR, y = HRL CCT_PhyR / ∑CCT_PhyR HRL CCT_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTONCCGRWFCCGR_PhyR, y | none | Real-Time On-Line Combined Cycle Generation Resource Weighting FactorThe Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource for the SCED interval y. |
| TGCCGR_PhyR, y | MW | Telemetered Generation for a Combined Cycle Generation Resource generation unitThe telemetered Real-Time power generation for a generation unit designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource at the time of State Estimator execution for the SCED interval y. |
| RTOFFCCGRWFCCT_PhyR,y | none | Real-Time Off-Line Combined Cycle Generation Resource Weighting FactorThe Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration when the whole Combined Cycle Train is Off-Line for the SCED interval y. |
| HRL CCT_PhyR | MW | High Reasonability Limit—The HRL as specified in the ERCOT-approved Resource Registration data for a generation unit designated in a Combined Cycle Train registration. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for a Combined Cycle Generation Resource. |
| CCT_PhyR | none | A generation unit designated in a Combine Cycle Train registration |
| y | none | A SCED interval in the 15-minute Settlement Interval |