| NPRR Number | 905 | NPRR Title | CRR Balancing Account Resettlement |
| --- | --- | --- | --- |
| Date Posted |  | December 13, 2018 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | Upon system implementation |  |
| Priority and Rank Assigned |  | Priority – 2019; Rank - 2630 |  |
| Nodal Protocol Sections Requiring Revision |  | 7.5.7, Method for Distributing CRR Auction Revenues 7.6, CRR Balancing Account 9.10, CRR Auction Revenue Distribution Invoices 9.12, CRR Balancing Account Invoices |  |
| Related Documents Requiring Revision/ Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) provides resettlement to reflect proper distribution of the Congestion Revenue Right (CRR) Balancing Account (CRRBA). |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Currently the CRRBA is distributed to Qualified Scheduling Entities (QSEs) with Loads using only the Initial Settlement values. Profiled values in Initial Settlement do not accurately represent a QSE’s Load Ratio Share (LRS) in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month. Aligning the CRRBA distribution with actual Load and not profiled data used for preliminary Settlements represents a correct distribution of the CRRBA funds. |  |
| Credit Work Group Review |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR905 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| PRS Decision |  | On 11/15/18, PRS unanimously voted to recommend approval of NPRR905 as submitted. All Market Segments were present for the vote. On 12/13/18, PRS unanimously voted to endorse and forward to TAC the 11/15/18 PRS Report and Impact Analysis for NPRR905 with a recommended priority of 2019 and rank of 2630. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 11/15/18, there was no discussion. On 12/13/18, there was no discussion. |  |

| Sponsor |  |
| --- | --- |
| Name | Heddie Lookadoo on Behalf of the Market Settlements Working Group (MSWG) |
| E-mail Address | Heddie.Lookadoo@NRG.com |
| Company | NRG |
| Phone Number | 713-537-2040 |
| Cell Number |  |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Phillip Bracy |
| E-Mail Address | Phillip.Bracy@ercot.com |
| Phone Number | 512-248-6917 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None |  |

Market Rules Notes

None

Proposed Protocol Language Revision

7.5.7	Method for Distributing CRR Auction Revenues

(1)	ERCOT shall determine, for each month, the CRR Monthly Revenues (CMR). The CMR is the sum of:

(a)	Monthly CRR revenue for that month; and

(b)	PCRR revenues.

(2)	ERCOT shall credit the net CRR Auction revenue (including PCRR revenue) produced from CRRs cleared in each CRR Auction that source from a Settlement Point located within a 2003 ERCOT CMZ and sink at a Settlement Point located within the same 2003 ERCOT CMZ to Qualified Scheduling Entities (QSEs) in the 2003 ERCOT CMZ on a zonal Load Ratio Share (LRS) basis.  All other net CRR Auction revenues must be allocated to QSEs on an ERCOT-wide LRS basis.  For these allocation purposes, any Non-Opt-In Entity (NOIE) Load Zone is considered to be located entirely within the 2003 ERCOT CMZ that represented the largest Load for that NOIE or group of NOIEs in 2003.

(3)	For initial distribution of CMR, revenues shall be paid to each QSE based on that QSE’s LRS in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month.

(4)	ERCOT shall true up the distribution of CMRs based on that QSE’s LRS in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month in accordance with paragraph (2) of Section 9.10, CRR Auction Revenue Distribution Invoices.

(5)	The net CRR Auction revenue produced from CRRs cleared and paid for in each CRR Auction that source from a Settlement Point within a 2003 ERCOT CMZ and sink at a Settlement Point located within the same 2003 ERCOT CMZ shall be distributed on a zonal LRS basis.  The portion of the net monthly CRR Auction revenue to be distributed to each QSE with Load in that zone for a given month is calculated as follows:

LACMRZAMT z, q=	(-1) * Σa(CRRZREV z, a + PCRRZREV z, a) * MLRSZ z, q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LACMRZAMT z, q | $ | Load-Allocated CRR Monthly Revenue Zonal Amount per zone per QSE—The payment to QSE q of the revenues resulted from the CRRs that source and sink in CMZ z, for the month. |
| CRRZREV z, a | $ | CRR Zonal Revenue per zone per CRR Auction—The revenue resulted from the CRRs that source and sink in CMZ z, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a, for the month. |
| PCRRZREV z, a | $ | PCRR Zonal Revenue per zone per CRR Auction—The revenue resulted from the PCRRs that source and sink in CMZ z, pertaining to CRR Auction a, for the month. |
| MLRSZ q, z | none | Monthly Load Ratio Share Zonal per QSE per zone—The LRS of QSE q for its Load in CMZ z, for the peak-Load 15-minute Settlement Interval in the month. |
| q | none | A QSE. |
| z | none | A 2003 ERCOT CMZ. |
| a | none | A CRR Auction. |

(6)	The net CRR Auction revenue produced from CRRs cleared and paid for in each CRR Auction that do not source from a Settlement Point within a 2003 ERCOT CMZ and sink at a Settlement Point located within the same 2003 ERCOT CMZ shall be distributed on an ERCOT-wide LRS basis.  The portion of the net monthly CRR Auction Revenue Amount (from CRRs with paths that cross the 2003 ERCOT CMZ boundaries) to be distributed for a given month is calculated as follows:

LACMRNZAMT q	=	(-1) * Σa(CRRNZREV a + PCRRNZREV a) * MLRS q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LACMRNZAMT q | $ | Load-Allocated CRR Monthly Revenue Non-Zonal Amount per QSE—The payment to QSE q of the revenues resulted from the CRRs that source and sink in different CMZs, for the month. |
| CRRNZREV a | $ | CRR Zonal Revenue per CRR Auction—The revenue resulted from the CRRs that source and sink in different CMZs, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a, for the month. |
| PCRRNZREV a | $ | PCRR Zonal Revenue per CRR Auction—The revenue resulted from the PCRRs that source and sink in different CMZs, pertaining to CRR Auction a, for the month. |
| MLRS q | none | Monthly Load Ratio Share per QSE—The LRS calculated for QSE q for the peak-Load 15-minute Settlement Interval in the month. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| a | none | A CRR Auction. |

7.6	CRR Balancing Account

(1)	In the Day-Ahead Market (DAM), if the congestion rent is equal to or greater than the net amounts due to all Congestion Revenue Right (CRR) Owners for any Settlement Interval, then ERCOT shall pay the net amounts due to the CRR Owners and put any excess amount into the CRR Balancing Account (CRRBA).

(2)	In the DAM, if the congestion rent is less than the net amounts due to all CRR Owners for any Settlement Interval, then ERCOT shall short-pay each CRR Owner on a prorated basis and shall keep track of how much each CRR Owner has been short-paid.  The proration must be calculated using only the amounts due to the CRR Owner for CRRs settled in both the DAM and Real-Time and not using amounts due to ERCOT for Point-to-Point (PTP) Obligations owned by the CRR Owner.

(3)	ERCOT shall pay any positive balance in the CRRBA to each short-paid CRR Owner, with the amount paid to each CRR Owner being the lesser of (a) a prorated amount based on the short-paid amount for that CRR Owner compared to the total short-paid amount, and (b) the short-paid amount for that CRR Owner.  Any remaining positive balance in the CRRBA will first be used to fund the CRRBA fund up to the fund cap, as described in Section 7.9.3.5, CRR Balancing Account Closure, and any surplus must be allocated to all Qualified Scheduling Entities (QSEs) on the QSE’s Load Ratio Share (LRS) in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month.

(4)	For initial distribution of the CRRBA, revenues shall be paid to each QSE based on that QSE’s LRS in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month.

(5)	ERCOT shall true up the distribution of CRRBA based on that QSE’s LRS in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month in accordance with paragraph (2) of Section 9.12, CRR Balancing Account Invoices.

9.10	CRR Auction Revenue Distribution Invoices

(1)	ERCOT shall prepare  Invoices for Congestion Revenue Right (CRR) Auction Revenue Distribution (CARD)  on a monthly basis on the first Business Day following the Real-Time Market (RTM) Initial Settlement posting of the last day of the month on the date specified in the Settlement Calendar.

(2)	ERCOT shall true up the distribution of monthly CRR Auction revenues by posting additional Settlement Invoices on the first Business Day following the RTM Final Settlement posting of the last day of the month on the date specified in the Settlement Calendar.  A trued up CARD Invoice will reflect differences to financial records generated on the previous CARD Invoice for a given month.

(3)	For each cycle, the Market Participant to whom the CARD Invoice is addressed (“Invoice Recipient”) is either a payee or payor.  The Invoice Recipient is responsible for accessing the CARD Invoice on the Market Information System (MIS) Certified Area once posted by ERCOT.

(4)	Each Invoice Recipient shall pay any debit and be entitled to receive any credit shown on the CARD Invoice on the payment due date.  Payments for CARD Invoices are due on the applicable payment due date whether or not there is any Settlement and Billing dispute regarding the amount of the payment.

(5)	ERCOT shall post on the MIS Certified Area for each Invoice Recipient a CARD Invoice based the calculations located:

(a)	Section 7.5.6.4, CRR Auction Revenues; and

(b)	Section 7.5.7, Method for Distributing CRR Auction Revenues.

(6)	CARD Invoices must contain the following information:

(a)	The Invoice Recipient’s name;

(b)	The ERCOT identifier (Settlement identification number issued by ERCOT);

(c)	Net Amount Due/Payable – the aggregate summary of all charges owed to or due from the Invoice Recipient summarized by CRR Auction Revenue month;

(d)	Time Period – the CRR Auction revenue month for which the Invoice is generated, including Initial or Final distribution;

(e)	Run Date – the date on which ERCOT created and published the Invoice;

(f)	Invoice Reference Number – a unique number generated by ERCOT for payment tracking purposes;

(g)	Payment Date – the date and time that Invoice amounts are to be paid or received; and

(h)	Remittance Information Details – details including the account number, bank name and electronic transfer instructions of the ERCOT account to which any amounts owed by the Invoice Recipient are to be paid or of the Invoice Recipient’s account from which ERCOT may draw payments due.

- 9.12	CRR Balancing Account Invoices

(1)	ERCOT shall prepare  Invoices for the Congestion Revenue Right (CRR) Balancing Account (CRRBA) on a monthly basis on the first Business Day following the Real-Time Market (RTM) Initial Settlement posting of the last day of the month on the date specified in the Settlement Calendar.

(2)	ERCOT shall true up the distribution of monthly the CRRBA by posting additional Settlement Invoices on the first Business Day following the RTM Final Settlement posting of the last day of the month on the date specified in the Settlement Calendar.  A trued up CRRBA Invoice will reflect differences to financial records generated on the previous CRRBA Invoice for a given month.

(3)	ERCOT shall prepare resettlement Invoices in the event that the balance in the CRRBA for the month changes due to a Day-Ahead Market (DAM) resettlement after the initial balancing account Invoices for that month have been posted as specified in the Settlement Calendar.  The Monthly Load Ratio Share (MLRS) as described in Section 7.9.3.5, CRR Balancing Account Closure, used for the resettlement CRRBA Invoice will be the same one used for the most recently posted balancing account Invoices. A resettlement CRRBA Invoice will reflect differences to financial records generated on the previous CRRBA Invoice for a given month.

(4)	For each Invoice cycle, the Market Participant to whom the CRRBA Invoice is addressed (“Invoice Recipient”) is a payee.  The Invoice Recipient is responsible for accessing the CRRBA Invoice on the Market Information System (MIS) Certified Area once posted by ERCOT.

(5)	ERCOT shall post on the MIS Certified Area for each Invoice Recipient a CRRBA Invoice based on the calculations located in Sections 7.9.3.4, Monthly Refunds to Short-Paid CRR Owners, and 7.9.3.5.

(6)	CRRBA Invoices must contain the following information:

(a)	The Invoice Recipient’s name;

(b)	The ERCOT identifier (Settlement identification number issued by ERCOT);

(c)	Net Amount Payable – the aggregate summary of all amounts owed to the Invoice Recipient summarized by month;

(d)	Time Period – the time period covered for each line item;

(e)	Run Date – the date on which the ERCOT created and published Invoice;

(f)	Invoice Reference Number – a unique number generated by ERCOT for payment tracking purposes; and

(g)	Payment Date – the date and time that Invoice amounts are to be received.

(7)	Each Invoice Recipient shall receive any credit shown on the CRRBA Invoice on the payment due date.  Credit shown on the CRRBA Invoice will be paid on due date whether or not there is any Settlement and billing dispute regarding the amount of the payment.