## ERCOT Energy Emergency Alert Communications

| Emergency Lev els* | Trigger | Grid Operators' Actions | Automated Emergency Notifications | Follow -up Communications from External Affairs | Media/Public Notifications |
| --- | --- | --- | --- | --- | --- |
| Normal Conditions | Reserves >3,000 MW | Normal operations |  |  |  |
| Control Room Operating Condition Notice (OCN) | Projected need f or additional resources | Issue "OCN" to utilities — informational only | Public Utility Commission of Texas (PUC) and NERC regional entity (Texas Reliability Entity (Texas RE)) notified via daily emails |  |  |
| Conservation Alert | Used rarely , as needed, to encourage conservation when tight operating reserv es are expected to be a reliability concern | Monitor need f or additional generation and v oluntary demand response resources | None – conservation messages entered and deploy ed manually in non-emergency situations | Coordinate public notification with PUC staf f; notify media and market participants’ media contacts/PIOs of change in conditions | Spotlight on home page of ercot.com and mobile app; social media posts; app messages and/or news release |
| Control Room Advisory | Reserves <3,000 MW | Issue "Adv isory" to utilities — informational only | PUC and Texas RE notified via daily emails; operations notices at control room discretion |  |  |
| Control Room Watch | Reserves <2,500 MW | Use quick-start capacity and release non-spinning reserves (available within 30 minutes) | Automated Emergency Notification Sy stem phone call and email to PUC staff, the Independent Market Monitor (IMM), Texas RE and FERC; operations notice at control room discretion | If potential emergency situation, additional inf ormation sent to SOC, PUC, OPUC, RRC, TCEQ, ERCOT Board, Gov ernment/Legislature, IMM, Texas RE, FERC and market participants' media contacts/PIOs | Consider conservation alert to support grid reliability |
| Energy Em ergency Level 1 POWER WATCH - Conservation Needed | Reserves <2,300 MW and not expected to be recovered within 30 minutes | Use additional capacity available from other grids (v ia DC Ties; 500 MW on average) and commit all available units; implement weather-sensitive and 30-minute Emergency Response Service (ERS)** resources and TDSP Load Management programs if needed. Deploy Responsive Reserves if PRC <2000. | Abov e plus State Operations Center (SOC) (notif ies city, county officials & law enf orcement), Office of Public Utility Counsel (OPUC), Government/Legislature staf f and ERCOT Board; media contacts f or utilities | If needed, notify grid emergency lists with additional inf ormation | Emergency Alerts list***, Twitter and Facebook, mobile app alerts; spotlight on ercot.com and mobile app; news release if appropriate |
| Energy Em ergency Level 2 POWER WARNING A- Conservation Critical | Frequency cannot be maintained above 59.91 Hz or Physical Responsive Capability (PRC) <1,750 MW and not expected to be recovered within 30 minutes | Deploy demand response resources: Load Resources under contract (typically ~1,150 MW) and/or remaining ERS** (currently 1,123 MW on peak), in either order. Begin block load transfers of load to other grids if appropriate. | Abov e, plus major news services | Same as abov e | Same as abov e |
| Energy Em ergency Level 3 POWER WARNING B - High Risk of Rotating Outages | Frequency cannot be maintained above 59.91 Hz or PRC cannot be maintained >1,375 MW | Ensure all resources allowed under EEA 2 are deploy ed | Same as abov e | Same as abov e | Same as abov e |
| Energy Em ergency Level 3 POWER EMERGENC Y - Rotating Outages in Progress | Frequency cannot be maintained above 59.91 Hz or PRC <1,000 MW and not expected to be recovered within 30 minutes | Instruct transmission operators to implement rotating outages; areas affected are at the discretion of the utilities | Same as abov e | Same as above | Emergency Alerts list, Twitter and Facebook, mobile app alerts; spotlight on ercot.com and mobile app; issue news release; activ ate call center if needed |

*Depending on system conditions overall, ERCOT Operations has some discretion regarding emergency levels and specific actions, based on these guidelines.

**Emergency Response Service (ERS) may include 10-minute, 30-minute and Weather-Sensitive Demand Response resources.

***Sign up for Emergency Alerts and News Bulletins email lists at  http://lists.ercot.com

PUBLIC – Electric Reliability Council of Texas	May 2018

#### ERCOT Summer Communications Overview

##### May 2018

ERCOT strives to provide timely, accurate and consistent communications in the event of an unexpected, abnormal or critical situation that threatens the reliability of the electric grid or operations of the ERCOT market.

The crisis communications plan provides the framework for the coordination of communications between ERCOT and appropriate contacts in keeping with protocols and Public Utility Commission of Texas (PUC) rules.

The ERCOT region covers most of Texas, including Houston, Dallas, Fort

Worth,  San Antonio,

Austin, Corpus Christi, Abilene, Midland and the Rio Grande Valley. It does not include consumers in the El Paso area, the Panhandle, Northeast Texas (including Longview, Marshall

#### Definitions

Crisis

A crisis is any unexpected, abnormal or critical situation that threatens the reliability of the regional electric grid, market operations or the safety or security of ERCOT’s employees or property.

Emergency

ERCOT Protocols define an emergency condition as an “operating  condition  where the safety or reliability of the ERCOT System is compromised or threatened, as determined by ERCOT.” All emergencies are crises, but not all crises are emergencies.

Energy Emergency Alert (EEA)

A designation  in the ERCOT Protocols and  Operating  Guides  that provides orderly, predetermined procedures for addressing a shortfall in electric supply. This includes calling upon  all available resources and, if necessary, shedding load during  electric  system emergencies.

Operating Conditions  Notice (OCN)

The first level of communication issued by the control room in anticipation of tight grid conditions and a possible emergency.

Triggering Event

A triggering event is an event that causes ERCOT to declare  a crisis.

ERCOT will generally only communicate about crises directly affecting ERCOT, the ERCOT market or the regional electric grid.

and Texarkana) and Southeast Texas (including	 	                                                                                             Beaumont, Port Arthur and The  Woodlands).

In the event of a system issue, public-facing ERCOT communication methods may include: ERCOT app (available in the Apple App Store or Google Play), social media, ERCOT Emergency Alerts ListServ and information  posted on ercot.com.

News releases may be distributed depending on the situation and when/if time permits, the Corporate Communications team will send updates directly to market participant   communicators.

###### Energy Emergency Alerts C ommunications Content and Triggering Events

Throughout each day, ERCOT system operators continuously assess the ability of available electric generation capacity to meet electricity consumption for current and future hours, with particular attention paid to available capacity during the projected peak demand hours of the day, from 3 to 7 p.m. in the summer.

If ERCOT determines there may be insufficient generation available, it can issue a series of notices, advisories or watches to generation resource owners to inform them of the situation. In many cases, this may result in generation owners revising their plans to include additional capacity if available. If the situation is not resolved through these voluntary actions, and the operating reserves (as measured by Physical Responsive Capability (PRC)) fall below 3,000 megawatts (MW), ERCOT Operations may

issue a Control Room Advisory. This serves as an informational notice to the generation owners that additional  capacity may be needed.

Control Room Watch

If operating reserves drop below 2,500 MW and are not expected to recover within 30 minutes, Operations will issue a Control Room Watch, which allows ERCOT to purchase  additional  reserve capacity from the market and take other actions to bring  on extra capacity.

Commencing with the Control Room Watch and subsequent EEA levels, the automated Emergency Notification  System (ENS)  is  initiated,  which includes automated emails, texts and phone calls to predetermined lists, social media posts and mobile app notifications.

Conservation Alert

ERCOT will only request conservation in advance if it is absolutely necessary for grid reliability. ERCOT will notify market participants and the media if conservation is officially requested.

EEA Level 1

When PRC levels drop below 2,300 MW and are not expected to recover within  30 minutes,  Level 1 of the EEA provides  for ERCOT to increase the supply of electricity  generation capacity  by  utilizing available DC Tie capability. ERCOT also can initiate demand response through its Emergency Response Service (ERS) resources (30-minute ERS and weather-sensitive loads) at this time.

EEA Level 2

When frequency cannot be maintained above 59.91 Hz or PRC levels are less than 1,750 MW and are not expected to recover within 30 minutes, Level 2 of the EEA is reached. EEA Level 2 seeks to reduce demand on the electricity grid by interrupting primarily large industrial consumers who have contractually agreed to have power interrupted in an emergency — known as Load Resources — and/or ERS resources, if available.

EEA Level 3

The EEA process culminates with Level 3, which begins if PRC cannot be maintained above 1,375 MW or frequency cannot be maintained above

  - Hz. If conditions  do not improve,  continue to

deteriorate or PRC drops below  1,000 MW and is not expected to recover within 30 minutes, ERCOT orders local utilities to institute mandatory rotating outages to reduce demand on the system.

Following the automated ENS notification to announce an EEA Level 3, ERCOT Corporate Communications staff will communicate outage information with the appropriate contacts .

Conclusion of an event

Once ERCOT concludes an EEA event, a notice will be provided indicating emergency conditions are  over and grid operations have returned to normal.

###### Communications Related to Localized Reliability Issues

In addition to the possibility of a regional generation shortfall, ERCOT also monitors the grid for the possibility of localized transmission outages or other reliability problems that could cause interruptions of service on a local level. In such cases, Corporate Communications, in consultation with the PUC, may also issue an advisory for a specific geographic area within the region if there is a reasonable probability that localized outages may be needed.

Government Contact: Rob Orr, rob.orr@ercot.com,  512-275-7011

Media Contact: Leslie Sopko, leslie.sopko@ercot.com, 512-275-7432

In a localized situation, ERCOT does not wish to create a statewide issue or concern where none exists, so ERCOT staff will work closely with local utilities in the affected area(s) to deliver a targeted conservation message when those actions are needed.

POWER WATCH – Conservation Needed; Chance of Rotating Outages

##### _

Power Watch – Conservation Needed

Chance of Rotating Outages

Consumers and businesses are encouraged to reduce their electricity use as much as possible today during peak hours from 3 to 7 p.m., to help prevent electricity emergencies or the need for rotating outages to protect the electric grid.

Explain  the situation

Steps to help reduce electricity use include:

    - Turn thermostat up 2 to    3 degrees during the peak hours of 3 to 7 p.m.

    - Set programmable thermostats to higher temperatures when no one is    home.

    - If home, use fans to feel 4 to 6 degrees   cooler.

    - Schedule pool pumps to run in the early morning or overnight hours; shut off between 4  and 6 p.m.

    - Limit use of large appliances (dishwasher, washer, dryer, etc.) to morning or after 7 p.m.

    - If you cook indoors from 3 to 7 p.m., use a microwave or   slow cooker.

    - Close blinds and drapes during late  afternoon.

More conservation tips: www.ercot.com/about/conservation http://powertosavetexas.org/

Power Watches are issued by the regional electric grid operator, the Electric Reliability Council of Texas (ERCOT), during periods of high demand when supplies of reserve power are low. At this stage, ERCOT has emergency measures to bring on additional generation, so rotating outages are not likely.

###### How to track electricity demand

    - View daily peak demand forecast, current load and available generation at http://www.ercot.com.

    - Get real-time notices of Energy Emergency Alerts by following @ERCOT_ISO on Twitter, liking the Electric Reliability Council of Texas page on Facebook, by signing up for the Apple or Android ERCOT mobile app (available for download at the Apple App Store or Google Play), or by signing up for the EmergencyAlerts list on http://lists.ercot.com.

###### Consumer assistance

Public Utility  Commission of Texas Hotline – 1-888-782-8477

###### ERCOT Region

The ERCOT Region includes Houston, Dallas, Fort Worth, San Antonio, Austin, Corpus Christi, Abilene and the Rio Grande Valley. It does not include the El Paso area, the Texas Panhandle (except in Competitive Renewable Energy Zones), Northeast Texas (Longview, Marshall and Texarkana), and Southeast Texas (Beaumont, Port Arthur, and The Woodlands). An ERCOT region map can be viewed at: http://www.ercot.com/news/mediakit/maps/index.html.

The Electric Reliability Council of Texas, Inc., (ERCOT) manages the flowof electric power to approximately

24 million Texas customers – representing 90 percent of the state's electric load and 75 percent of the Texas land

area.  As the Independent System Operator for the region, ERCOT schedules power on an electric grid that connects more than 46,500 miles of transmission lines and more than 600 generation units. ERCOT also manages financial settlement for the competitive wholesale bulk-power market and administers customer switching for more than 7     million Texans in competitive choice areas. ERCOT is a membership-based 501(c)(4) nonprofit corporation, governed   by a board of directors and subject to oversight by the Public Utility Commission of Texas and the Texas Legislature.

Media Contact: media@ercot.com

### Power Warning – Conservation CRITICAL; HIGH Risk of Rotating Outages

Power Warning – Conservation CRITICAL

HIGH Risk of Rotating Outages

Conservation is critical to avoid electricity emergencies or the need for rotating outages during the peak hours from 3 to 7 p.m. Consumers should reduce their consumption of electricity as much as possible to help prevent an emergency.

- Turn thermostat up 2 to 3 degrees during the peak hours of 3 to 7   p.m.

- Set programmable thermostats to higher temperatures when no one    is home.

- If home, use fans to feel 4 to 6 degrees   cooler.

- Schedule pool pumps to run in the early morning or overnight hours; shut off between 4 and 6  p.m.

- Limit use of large appliances (dishwasher, washer, dryer, etc.) to morning or after 7 p.m.

- If you cook indoors from 3 to 7 p.m., use a microwave or   slow cooker.

- Close blinds and drapes during late  afternoon.

More conservation tips: www.ercot.com/about/conservation http://powertosavetexas.org/

Power Warnings are issued by the regional electric grid operator, the Electric Reliability Council of Texas (ERCOT), when there is a likelihood that rotating outages will be needed to reduce load.

Rotating outages are controlled, temporary interruptions of electrical service initiated by each utility when supplies of reserve power are exhausted. Without this safety valve, generators would overload and begin shutting down to avoid damage, risking a domino effect of a widespread, uncontrolled outage.

Rotating outages primarily affect residential neighborhoods and small businesses.

The outages are typically limited to 10 to 45 minutes before being rotated to a different neighborhood. Some customers may experience longer outages if power surges cause equipment failure during the restoration process. Customers can minimize power surges by turning off appliances, lights and other equipment, except for one light to determine when power has been restored.

###### How to track electricity demand

- View daily peak demand forecast, current load and available generation at  http://www.ercot.com.

- Get real-time notices of Energy Emergency Alerts by following @ERCOT_ISO on Twitter, liking the Electric Reliability Council of Texas page on Facebook, by signing up for the Apple or Android ERCOT mobile app (available for download at the Apple App Store or Google Play), or by signing up for the EmergencyAlerts list on http://lists.ercot.com.

###### Consumer assistance

Public Utility  Commission of Texas Hotline –  1-888-782-8477

###### Call your electric utility for information about local outages

ERCOT manages the state’s high-voltage bulk electricity grid. For questions about local outages at your home or business, or questions about rotating outage procedures for your neighborhood, check with the utility company or transmission provider listed on your electric bill.

###### ERCOT Region

The ERCOT Region includes Houston, Dallas, Fort Worth, San Antonio, Austin, Corpus Christi, Abilene and the Rio Grande Valley. It does not include the El Paso area, the Texas Panhandle, Northeast Texas (Longview, Marshall and Texarkana), and Southeast Texas (Beaumont, Port Arthur, a nd The Woodlands). An ERCOT region map can be viewed at: http://www.ercot.com/news/mediakit/maps/index.html.

The Electric Reliability Council of Texas, Inc., (ERCOT) manages the flowof electric power to approximately

24 million Texas customers – representing 90 percent of the state's electric load and 75 percent of the Texas land  area.  As the Independent System Operator for the region, ERCOT schedules power on an electric grid that connects more than 46,500 miles of transmission lines and more than 600 generation units. ERCOT also manages financial settlement for the competitive wholesale bulk-power market and administers customer switching for more than 7     million Texans in competitive choice areas. ERCOT is a membership-based 501(c)(4) nonprofit corporation, governed   by a board of directors and subject to oversight by the Public Utility Commission of Texas  and the Texas   Legislature.

Media Contact: media@ercot.com

### Power Emergency – Conservation CRITICAL; Rotating Outages in Progress

Power Emergency – Conservation CRITICAL Rotating Outages Have Begun

The Electric Reliability Council of Texas (ERCOT) has instructed utilities to begin rotating outages because there is not enough electric generation available to keep up with consumer demand .

Rotating outages are controlled, temporary interruptions of electric service, typically lasting 10 to 45 minutes in each affected area. The locations and durations are determined by the local utilities.

The need for rotating outages likely will last [identify affected time period].

Consumers and businesses are urged to reduce their electricity use to the lowest level possible, including these steps:

- Turn thermostat up 2 to 3 degrees during the peak hours    from 3 to 7 p.m.

- Set programmable thermostats to higher temperatures when no one    is home.

- If home, use fans to feel 4 to 6 degrees   cooler.

- Schedule pool pumps to run in the early morning or overnight hours; shut off between 4  and 6 p.m.

- Limit use of large appliances (dishwasher, washer, dryer, etc.) to morning or after 7 p.m.

- If you cook indoors from 3 to 7 p.m., use a microwave or    slow cooker.

- Close blinds and drapes during late  afternoon.

More conservation tips: www.ercot.com/about/conservation http://powertosavetexas.org/

###### Background

A Power Emergency indicates that ERCOT has instructed utilities to implement rotating outages to reduce load.

Rotating outages are controlled, temporary interruptions of electrical service initiated by each utility when supplies of reserve power are exhausted. Without this safety valve, generators would overload and begin shutting down to avoid damage, risking a domino effect of a widespread, uncontrolled outage.

The outages are typically limited to 10 to 45 minutes before being rotated to a different neighborhood. Some customers may experience longer outages if power surges cause equipment failure during the restoration process. Customers can minimize power surges by turning off appliances, lights and other equipment, except for one light to determine when power has been restored.

###### How to track electricity demand

- View daily peak demand forecast, current load and available generation at  http://www.ercot.com.

- Get real-time notices of Energy Emergency Alerts by following @ERCOT_ISO on Twitter, liking the Electric Reliability Council of Texas page on Facebook, by signing up for the Apple or Android ERCOT mobile app (available for download at the Apple App Store or Google Play), or by signing up for the EmergencyAlerts list on http://lists.ercot.com.

###### Consumer assistance

Public Utility  Commission of Texas Hotline –  1-888-782-8477

###### Call your electric utility for information about local outages

ERCOT manages the state’s high-voltage bulk electricity grid. For questions about local outages at your home or business, or questions about rotating outage procedures for your neighborhood, check with the utility company or transmission provider listed on your electric bill.

###### ERCOT Region

The ERCOT Region includes Houston, Dallas, Fort Worth, San Antonio, Austin, Corpus Christi, Abilene and the Rio Grande Valley. It does not include the El Paso area, the Texas Panhandle (except in Competitive Renewable Energy Zones), Northeast Texas (Longview, Marshall and Texarkana), and Southeast Texas (Beaumont, Port Arthur, and The Woodlands). An ERCOT region map can be viewed at: http://www.ercot.com/news/mediakit/maps/index.html.

The Electric Reliability Council of Texas, Inc., (ERCOT) manages the flowof electric power to approximately

24 million Texas customers – representing 90 percent of the state's electric load and 75 percent of the Texas land area. As the Independent System Operator for the region, ERCOT schedules power on an electric grid that connects more than 46,500 miles of transmission lines and more than 600 generation units. ERCOT also manages financial settlement for the competitive wholesale bulk-power market and administers customer switching for more than 7     million Texans in competitive choice areas. ERCOT is a membership-based 501(c)(4) nonprofit corporation, governed   by a board of directors and subject to oversight by the Public Utility Commission of Texas  and the   Texas Legislature.

Media Contact: media@ercot.com

## Grid Conditions Communication Methods

###### Twitter

@ERCOT_ISO

###### Facebook

Electric Reliability Council of Texas

###### ERCOT mobile app

Emergency condition information sent via push notifications Available through the Apple App Store and Google Play

ERCOT ListServs EmergencyAlerts – Receives email notifications about Energy Emergency Alerts (EEAs)

News_Bulletins – Receives all ERCOT news releases via email

News_Media_Only (restricted list)

– Available for members of the press only

Self-subscribe at lists.ercot.com

Additional emergency lists are available for market participant media contacts/PIOs, as well as government and legislative contacts. Please contact media@ercot.com for more information.

May 2018