| Principle Number | 1.3 | Principle Title | Offering and Awarding Ancillary Services in Real-Time |
| --- | --- | --- | --- |
| Date Posted |  | July 1, 2019 |  |
| Executive Summary |  | Real-Time Co-Optimization (RTC) will allow Qualified Scheduling Entities (QSEs) to offer and be awarded Ancillary Services (AS) in Real-Time. |  |
| Principle Description |  | The purpose of Key Principle 1.3, Offering and Awarding Ancillary Services in Real-Time is to outline the key mechanisms and timelines for submitted AS offers, as well as those considered and awarded under RTC. Specifically, this principle addresses the following concepts: (a) AS offer structure and timing; (b) QSE telemetry for updating available capacity from AS offers; (c) Under-frequency relay (UFR) Load Resource scheduling for Responsive Reserve (RRS) UFR and ERCOT Contingency Reserve Service (ECRS); and (d) Constraints to be used for the RTC clearing process. |  |
| RTCTF Discussion |  |  |  |
| TAC Action Requested |  |  |  |
| TAC Action Summary |  |  |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for Voting

- QSEs will have ability in Real-Time to indicate whether a Resource is temporarily unable to provide AS due to operational constraints.

- UFR Load Resources will be able to self-provide RRS UFR and ECRS; the amount of which will based on Day-Ahead Market (DAM) and AS trades.

- RTC will account for frequency responsive capacity of a Combined Cycle Generation Resource (CCGR) when awarding Regulation and RRS Primary Frequency Response (PFR).

# Previously Approved Principle Concepts

None

# Future Decision Points and Issues for Developing Principle Concepts

Functionality and Process Concepts

- QSEs will have ability in Real-Time to indicate whether a Resource is temporarily unable to provide AS due to operational constraints.

Details under development:

- For scenarios where the reduced AS capability is less than the QSE-submitted AS Offer (MW), QSEs can, via Resource-specific telemetry, inform ERCOT of the reduced AS capability (MW) by specific AS type from a Resource.

- Under RTC, ERCOT systems will use Resource-specific telemetered values to limit AS Awards.

- ERCOT will be proposing is to add new Resource-specific telemetry points (analog – MW) for each AS type/subtype:

  - Regulation Up (Reg-Up) Capability

  - Regulation Down (Reg-Down) Capability

  - Fast Responding Regulation Up Service (FFRS-Up) Capability

  - Fast Responding Regulation Down Service (FFRS-Down) Capability

  - RRS PFR Capability

  - RRS UFR Capability

  - RRS FFR Capability

  - ECRS Capability

  - Non-Spinning Reserve (Non-Spin) Capability

- Future discussion will also address whether a QSE with a submitted telemetered Ramp Rate up, as telemetered by a QSE for a Resource, with a High Dispatch Limit (HDL) that is greater than the Resource’s current telemetered MW, should be able to indicate that the Resource is unable to provide Reg-Up, ECRS and/or Non-Spin.

- UFR Load Resources will be able to self-provide RRS UFR and ECRS; the amount of which will based on DAM and AS trades.

Details under development:

- Under RTC, UFR Load Resources will not capable of arming and disarming UFR relay based on non-zero/zero AS Awards if treated like Generation Resources.

  - New Resource-specific telemetry points (analog – MW) will be added for RRS UFR, RRS FFR and ECRS, allowing ERCOT visibility into MW service quantities self-provided by a UFR Load Resource.

    - System-wide RRS and ECRS AS Demand Curves (ASDCs) will be reduced by the validated amount of UFR Load Resource self-provided service quantities.

    - ERCOT is in the process of developing validation processes and rules regarding these new telemetry values.

      - The 60% limitation rule (per NPRR 863) for system-wide RRS from UFR Load Resources and FFR and 420 MW from FFR will be maintained.

      - Telemetered UFR Load Resource self-provided RRS, ECRS MW amounts with respect to DAM awards and AS Trades under normal conditions and scarcity conditions will be reconciled.

- RTC will account for will account for frequency responsive capacity of a CCGR when awarding Regulation and RRS PFR.

Details under development:

- An On-Line CCGR can have portions of its capacity that are not frequency responsive (e.g., steam generator, duct burner, etc.).  Therefore, under RTC, Real-Time results should not be such that awarded Regulation and/or RR PFR (or portions thereof) could be coming from CCGR capacity that is Non-Frequency Responsive Capacity (NFRC).

- ERCOT will be proposing an enhanced mathematical model of CCGR, and new telemetry to account for frequency responsive capacity of a CCGR for awarding Regulation and RRS PFR.

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |