| Principle Number | 1.4 | Principle Title | Systems/Applications that Provide Input into the Real-Time Optimization Engine |
| --- | --- | --- | --- |
| Date Posted |  | July 1, 2019 |  |
| Executive Summary |  | The current ERCOT systems and applications that provide input for the Real-Time Market (RTM) optimization engine are not designed to accommodate Real-Time Co-optimization (RTC)–i.e., the awarding of Ancillary Services (AS) in Real-Time. This RTC principle addresses the necessary modifications to those systems and applications. |  |
| Principle Description |  | ERCOT’s systems and applications that provide input for the RTM optimization engine will be modified to account for RTC (e.g., the Resource Limit Calculator (RLC)). Real-Time information provided by Qualified Scheduling Entities (QSEs) to ERCOT, and ERCOT to QSEs, will need to be reviewed and modified to accommodate such changes. |  |
| RTCTF Discussion |  | On 6/7/19, the RTCTF reached consensus on the first part of KP1.4. |  |
| TAC Action Requested |  |  |  |
| TAC Action Summary |  |  |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for Voting

- Under RTC, for Resources that are being economically dispatched by ERCOT, the current practice of pre-reserving portions of their capacity to provide Ancillary Services (AS) through telemetry from the Resource’s QSE will be discontinued.  AS awards will now be an output of RTC.

  - Because of this, RLC calculated Resource Limits of High Ancillary Service Limits (HASLs) and Low Ancillary Service Limits (LASLs) will be discontinued.

  - With the HASL and LASL calculations being discontinued, the method for calculating dispatch limits needs to be modified.  High Sustained Limits (HSLs), Low Sustained Limits (LSLs), and telemetered ramp rates will be used directly to calculate High Dispatch Limits (HDLs) and Low Dispatch Limits (LDLs), per the formulas:

HDL=Min(HSL,TelemMW+NormalRampRateUp*5)

LDL=Max(LSL,TelemMW-NormalRampRateDn*5)

- Under RTC, the following will occur:

        - HASL and LASL calculations will be discontinued;

        - AS will be a Resource specific award (and an output of RTC);

        - AS will be awarded only to Resources that are qualified to provide the service; and

        - Regulation instructions will be Generation Resource specific.

Furthermore, the following modifications to Real-Time telemetered information provided from/to QSEs via RLC will be necessary:

- For Generation Resources, AS-related (i) responsibility and schedule telemetry, and (ii) Regulation Up/Down participation factor telemetry from QSEs will be discontinued;

- New telemetry for QSEs to track (i) Resource-specific AS awards, and  (ii) Generation Resource-specific Regulation Up/Down instructions from Load Frequency Control (LFC); and

- New telemetry for QSEs to send AS-specific desired maximum capability for any AS qualified Resource.

# Previously Approved Principle Concepts

None

# Future Decision Points and Issues for Developing Principle Concepts

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |