| NPRR Number | 919 | NPRR Title | Exemption from Governor Primary Frequency Response Control for Certain Resources in Private Use Networks |
| --- | --- | --- | --- |
| Date Posted |  | January 18, 2019 |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Sections Requiring Revision |  | 8.5.1.1, Governor in Service |  |
| Related Documents Requiring Revision/Related Revision Requests |  | NOGRR184, Related to NPRR919, Exemption from Governor Primary Freqency Response Control for Certain Resources in Private Use Networks |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) allows a Resource in a Private Use Network whose primary function is providing steam to a host, and is not participating in the ERCOT energy market, to be exempted from Primary Frequency Response service requirements. The Governor will still be in service, but may be allowed to be in pressure control instead of Primary Frequency Response control. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | The intent of this exclusion is to reduce or eliminate unnecessary Primary Frequency Response model validation testing per North American Electric Reliability Corporation (NERC) Standard MOD-027-1, Verification of Models and Data for Turbine/Governor and Load Control or Active Power/Frequency Controls Functions; and Primary Frequency Response testing per Section 8.5.1.1 and paragraph (2) of Nodal Operating Guide Section 2.2.7. |  |

| Sponsor |  |
| --- | --- |
| Name | Eric Woelfel |
| E-mail Address | ewoelfel@fpcusa.com |
| Company | Formosa Utiltiy Venture, Ltd. |
| Phone Number | 973-716-7389 |
| Cell Number | 973-634-6107 |
| Market Segment | Independent Generator |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Phillip Bracy |
| E-Mail Address | Phillip.Bracy@ercot.com |
| Phone Number | 512-248-6917 |

Proposed Protocol Language Revision

8.5.1	Generation Resource and QSE Participation

8.5.1.1	Governor in Service

(1)	At all times an All-Inclusive Generation Resource is On-Line, its Governor must remain in service and be allowed to respond to all changes in system frequency except during startup, shutdown, or testing.  A Generation Entity may not reduce Primary Frequency Response on an individual All-Inclusive Generation Resource even during abnormal conditions without ERCOT’s consent (conveyed by way of the Resource Entity’s Qualified Scheduling Entity (QSE)) unless equipment damage is imminent.  All All-Inclusive Generation Resources that have capacity available to either increase output or decrease output in Real-Time must provide Primary Frequency Response, which may make use of that available capacity.  Only Generation Resources providing Regulation Up (Reg-Up), Regulation Down (Reg-Down), Responsive Reserve (RRS), or Non-Spinning Reserve (Non-Spin) from On-Line Resources, as specified in Section 8.1.1, QSE Ancillary Service Performance Standards, shall be required to reserve capacity that may also be used to provide Primary Frequency Response.

(2)	At ERCOT’s sole discretion, any Generation Resource in a Private Use Network that is solely used for providing steam and has a zero High Sustained Limit (HSL) for at least 24 months may be granted an exemption from having the Governor in Primary Frequency Response service.  The Governor may instead have the option of being in pressure control service.  This specific Generation Resource would also not be permitted to participate in any ancillary service market.  The requesting Generation Resource should submit to ERCOT an attestation and supporting evidence showing the HSL for the requesting Generation Resource was zero for at least 24 months.  ERCOT shall make a determination on the exemption within 60 days of receipt of the attestation.