| Principle Number | 1.4 | Principle Title | Systems/Applications that Provide Input into the Real-Time Optimization Engine |
| --- | --- | --- | --- |
| Date Posted |  | August 13, 2019 |  |
| Executive Summary |  | The current ERCOT systems and applications that provide input for the current Real-Time Market (RTM) optimization engine are not designed to accommodate Real-Time Co-optimization (RTC)–i.e., the awarding of Ancillary Services (AS) in Real-Time. Key Principle (KP) 1.4 addresses modifications to those systems and applications that will be necessary to accommodate RTC. |  |
| Principle Description |  | ERCOT’s systems and applications that provide input for the current RTM optimization engine will be modified to account for RTC (e.g., the Resource Limit Calculator (RLC)). Real-Time information exchanged between Qualified Scheduling Entities (QSEs) and ERCOT will need to be reviewed and modified to accommodate such changes. |  |
| RTCTF Discussion |  | On 6/7/19, the RTCTF reached consensus on the subsection (1) of KP1.4. On 7/12/19, the RTCTF added “Telemetered” into the equations in KP 1.4, subsection (1) for clarification with respect to Ramp Rates. On 8/9/19, the RTCTF reached consensus on KP 1.4, subsection (2), and ERCOT introduced KP 1.4, subsection (4). |  |
| TAC Action Requested |  | On 7/24/19, TAC vote to endorse KP 1.4, subsection (1) for purposes of informing the Board. |  |
| TAC Action Summary |  | On 7/24/19, TAC endorsed KP 1.4, subsection (1). |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC ENDORSEMENT

- Under RTC, the following will occur:

        - HASL and LASL calculations will be discontinued;

        - AS will be a Resource specific award (and an output of RTC);

        - AS will be awarded only to Resources that are qualified to provide the service; and

        - Regulation instructions will be Generation Resource specific.

Furthermore, the following modifications to Real-Time telemetered information provided from/to QSEs via RLC will be necessary:

- For Generation Resources, AS-related (i) responsibility and schedule telemetry, and (ii) Regulation Up/Down participation factor telemetry from QSEs will be discontinued;

- New telemetry for QSEs to track (i) Resource-specific AS awards, and  (ii) Generation Resource-specific Regulation Up/Down instructions from Load Frequency Control (LFC);

# PRINCIPLE COncepts Previously endorsed by tac

- Under RTC, for Resources that are being economically dispatched by ERCOT, the current practice of pre-reserving portions of their capacity to provide Ancillary Services (AS) through telemetry from the Resource’s QSE will be discontinued.  AS awards will now be an output of RTC.

  - Because of this, RLC calculated Resource Limits of High Ancillary Service Limits (HASLs) and Low Ancillary Service Limits (LASLs) will be discontinued.

  - With the HASL and LASL calculations being discontinued, the method for calculating dispatch limits needs to be modified.  High Sustained Limits (HSLs), Low Sustained Limits (LSLs), and telemetered ramp rates will be used directly to calculate High Dispatch Limits (HDLs) and Low Dispatch Limits (LDLs), per the formulas:

HDL=Min(HSL,TelemMW+NormalTelemeterdRampRateUp*5)

LDL=Max(LSL,TelemMW-NormalTelemeteredRampRateDn*5)

# Principle Concepts IN DISCUSSION AT RTCTF

- Under RTC, there will be new telemetry types to be submitted by QSEs with respect to AS-specific maximum capability for any AS-qualified Resource.

- To better understand ramp capability for determining ECRS and Non-Spin awards, the following new telemetry points will be needed for Resources providing those services:

  - A blended 10-minute Emergency Ramp Rate Up for the Resources that will be used to check the feasibility of ECRS awards.  This value will be the ten-minute output change capability of the Resource divided by ten (positive change for Resources injecting into the grid and negative change for Resources withdrawing from the grid).

  - A blended 30-minute Normal Ramp Rate Up for the Resources that will be used to check the feasibility of Non-Spin awards.  This value will be the 30-minute output change capability of the Resource divided by 30 (positive change for Resources injecting into the grid and negative change for Resources withdrawing from the grid).

# Future Decision Points and Issues for Developing Principle Concepts

Additional detail on new telemetry points between ERCOT and QSEs.

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |