| NPRR Number | 930 | NPRR Title | Weekly RUC Modifications for Forecasted Emergency Conditions |
| --- | --- | --- | --- |
| Date |  | August 1, 2019 |  |
| Submitter’s Information |  |  |  |
| Name |  | Dan Woodfin / Austin Rosel |  |
| E-mail Address |  | dwoodfin@ercot.com / arosel@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-3115 / 512-248-6686 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

Since the July PRS meeting, ERCOT has worked with stakeholders and held a WebEx to discuss remaining procedural issues not resolved by the July 15, 2019 STEC comments.  ERCOT believes that the comments below resolve all known stakeholder and ERCOT issues in Section 2, Definitions and Acronyms, and Section 3, Management Activities for the ERCOT System, related to this topic.  Specifically, these comments:

- Agree with STEC’s proposal to provide eight Business Hours’ notice before performing an Outage Adjustment Evaluation (OAE) with the addition of a provision to allow less than eight Business Hours’ notice if the Outages are scheduled to start before that time has passed;

- Allow a Qualified Scheduling Entity (QSE) to notify ERCOT that a specific Resource cannot be considered in an OAE for reasons of Resource reliability, compliance with contractual warranty obligations, or other reasons beyond the QSE’s control so that ERCOT will not include these Resources in the OAE;

- Clarify that ERCOT will issue an Advance Action Notice (AAN) and, if appropriate, an OAE when one or more Resources could address a violation of reliability standards other than transmission-related standards;

- Modify the description of certain information that STEC had proposed would be included and posted in the assessment of the need for an OAE; and

- Include several other less substantive changes and clarifications.

In addition, ERCOT recommends the following additional changes to the Settlement section of Nodal Protocol Revision Request (NPRR) 930 in order to provide further clarification while preserving the objectives described in the July 15, 2019 STEC comments.  Specific changes include the following:

- Clarification to Section 5.6.5.1, Make-Whole Payment for Canceled or Delayed Outages for OSAs, to clarify that QSEs representing Resources that receive an OSA have 60 days from the issuance of an Real-Time Market (RTM) Initial Statement for an Operating Day to submit certain documentation to recover costs due to a cancellation or delay of an Outage;

- Removal of the cost of selling a hedge or repurchasing a hedge;

- A new paragraph to exclude costs associated with financial transactions, including selling or repurchasing a hedge; and

- Changes to Section 5.6.5.3, Timeline for Calculating RUC Clawback Charges for Resources Receiving OSAs, to add back language that was deleted in the July 15, 2019 STEC comments.

Revised Cover Page Language

| NPRR Number | 930 | NPRR Title | Process, Pricing, and Cost Recovery for Delayed Resource Outages |
| --- | --- | --- | --- |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms and Abbreviations 3.1.4.6, Outage Coordination of Forecasted Emergency Conditions 3.1.6.9, Withdrawal of Approval or Acceptance and Rescheduling of Approved or Accepted Planned Outages of Resource Facilities 5.1, Introduction 5.5.2, Reliability Unit Commitment (RUC) Process 5.6.5, Settlement for Canceled or Delayed Outages for Outage Schedule Adjustments (OSAs) 5.6.5.1, Make-Whole Payment for Canceled or Delayed Outages for OSAs (new) 5.6.5.2, RUC Make-Whole Payment and RUC Clawback Charge for Resources Receiving OSAs (new) 5.6.5.3, Timeline for Calculating Clawback Chargesfor Resources Receiving OSAs (new) 5.7.2, RUC Clawback Charge 5.7.4, RUC Make-Whole Charges 5.8, Annual RUC Reporting Requirement 6.5.7.3, Security Constrained Economic Dispatch 6.5.9.3.1.1, Advance Action Notice (new) 9.5.6, RTM Resettlement Statement |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) requires ERCOT to use an Outage Adjustment Evaluation (OAE) process to delay Rccepted or approved Outages after issuing an Advance Action Notice to describe the need and providing time for Qualified Scheduling Entities (QSEs) to adjust their outage plans. AITResource that receives an Outage Schedule Adjustment as a result of an OAE is committed, it will be made whole to its actual costs incurred due to delaying or canceling and rescheduling the Outage and committing the Resource, as appropriate.. In addition, it sets an offer floor for the Resource at $72500/MWht Together, these changes will encourage Resources to self-commit to avoid the opportunity cost of offering at a very high price, while making them whole if they do follow ERCOT’s instructions. A QSE with a Resource that is unable to return to service due to reliability reasons may update its Outage to a Forced Outage so that it may continue its Outage plans to support the Resource’s longevity and availability during critical Load periods. |  |
| Business Case |  | The ERCOT market design relies on very high prices during a limited number of hours that reflect the Value of Lost Load (VOLL) or the shortage of reserves. When ERCOT takes actions for short-term reliability needs that have an impact on market confidence in this mechanism, those actions can inadvertently change future investment decisions made by Market Participants. Paragraph (f)(12) of Public Utility Commission of Texas (PUCT) Substantive Rule 25.503, Oversight of Wholesale Market Participants, creates a duty for ERCOT Market Participants who identify a provision in ERCOT procedures that produces an outcome inconsistent with the efficient and reliable operation of the ERCOT-administered markets to call the provision to the attention of ERCOT Staff and the appropriate ERCOT subcommittee. This NPRR calls such attention to this matter and provides a structured, predictable way for ERCOT to operate reliably in the future – for both short-term and long-term needs of the ERCOT market. NPRR365, Change in Resource Outage Approvals from Eight to 45 Days (formerly "Change in Resource Outage Approvals from Eight to 90 Days"), created a new Section 3.1.6.9, Withdrawal of Approval or Acceptance and Rescheduling of Approved or Accepted Planned Outages of Resource Facilities, that seems to conflict with Section 3.1.4.6, Outage Coordination of Forecasted Emergency Conditions, that has been in place since the first draft of the Nodal Protocols. This NPRR harmonizes these sections to create more predictability and establishes a mechanism for ERCOT to use in these circumstances. |  |

Market Rules Notes

Please note the baseline Protocol language in the following section(s) has been updated to reflect the incorporation of the following NPRR(s) into the Protocols:

- NPRR910, Clarify Treatment of RUC Resource that has a Day-Ahead Market Three-Part Supply Award (unboxed 5/31/19)

  - Section 5.5.2

Revised Proposed Protocol Language

2.1 	DEFINITIONS

Advance Action Notice (AAN)

A type of Operating Condition Notice (OCN) that identifies a possible future Emergency Condition and describes future action ERCOT expects to take to address that condition unless the need for ERCOT action is alleviated by Qualified Scheduling Entity (QSE) and/or Transmission Service Provider (TSP) actions or by other system developments.

Outage Adjustment Evaluation (OAE)

A study performed by ERCOT when it forecasts an inability to meet applicable reliability standards and has exercised all other reasonable options and needs to delay or to cancel and reschedule one or more Resource Outages, unless the issue is due to transmission reliability and is limited to Resources at a single site.

Outage Schedule Adjustment (OSA)

An adjustment to delay or to cancel and reschedule a Resource’s Planned Outage that has already been accepted or approved by ERCOT.  The OSA is issued by ERCOT to the Qualified Scheduling Entity (QSE) representing the Resource.

Outage Schedule Adjustment (OSA) Period

The portion of a Resource’s Planned Outage schedule for which ERCOT issues an Outage Schedule Adjustment (OSA).  The OSA Period will commence at the planned start time for the Resource Outage, based on the Resource’s Planned Outage existing in the Outage Scheduler at the time the OAE is performed, and will end at the time stated in the OSA.

- 2.2	ACRONYMS AND ABBREVIATIONS

AAN			Advance Action Notice

OAE			Outage Adjustment Evaluation

OSA			Outage Schedule Adjustment

3.1.4.6	Outage Coordination of Potential Transmission Emergency Conditions

(1)	If ERCOT forecasts an inability to meet applicable transmission reliability standards, has exercised all other reasonable options, and there is only one QSE with approved or accepted Resource Outages which could resolve the situation if the start of one or more of the Resource Outages at a single Resource site were delayed or one or more ongoing Resource Outages at a single Resource site were restored early, then ERCOT may contact that QSE and attempt to reach a mutually acceptable solution to delay or reschedule one or more of those Outages.  In this case, ERCOT is not obligated to follow the process described in Section 3.1.6.9, Withdrawal of Approval or Acceptance of Planned Outages of Resource Facilities.  ERCOT shall not provide information to the QSE during these contacts which is not directly related to the QSE’s Planned Resource Outage(s) and is not otherwise available to all other Market Participants.

WC

or one Busines

(2)	If ERCOT and the QSE are unable to reach a mutually agreeable solution to change the Resource Outage, ERCOT may issue an Outage Schedule Adjustment (OSA) to the QSE for the Resource.

(3)	If there are Resources at multiple sites with approved or accepted Resource Outages, whose approval or acceptance could be withdrawn to meet the applicable transmission reliability standards, ERCOT shall utilize the process described in Section 3.1.6.9, Withdrawal of Approval or Acceptance of Planned Outages of Resource Facilities.

(4)	This Section is not intended to restrict ongoing Outage Coordination activities occurring more than seven days in advance of Real-Time.

- 3.1.6.9	Withdrawal of Approval or Acceptance and Rescheduling of Approved or Accepted Planned Outages of Resource Facilities

(1)	If ERCOT believes it cannot meet applicable reliability standards and has exercised all other reasonable options, and the delayed initiation of, or early termination of, one or more approved or accepted Resource Outages not addressed by Section 3.1.4.6, Outage Coordination of Potential Transmission Emergency Conditions, could resolve the situation, then ERCOT shall issue an Advance Action Notice (AAN) pursuant to Section 6.5.9.3.1.1, Advance Action Notice.

(a)	The AAN shall describe the reliability problem, the date and time that the possible Emergency Condition would begin, the date and time that the possible Emergency Condition would end, and a summary of the actions ERCOT believes it might take, including, if applicable, the amount of capacity it would seek from an Outage Adjustment Evaluation (OAE) and Outage Schedule Adjustments (OSAs).  The AAN must state the time at which ERCOT will execute an OAE, if an OAE is deemed necessary.

(b)	ERCOT shall issue the AAN a minimum of 24 hours prior to performing an OAE.  Additionally, unless impracticable pursuant to paragraph (3)(f) below, the OAE should not be performed until eight Business Hours have elapsed following issuance of the AAN.  ERCOT shall not issue an OSA under this Section unless it has first completed an OAE.

(c)	Following the AAN, ERCOT may communicate with Market Participants about the reliability problem, however, ERCOT may not provide information about market conditions to a subset of Market Participants that is not generally available to all Market Participants.

(d)	As conditions change, ERCOT shall, to the extent practicable, update the AAN in order to provide simultaneous notice to Market Participants.

(e)	This section does not limit Transmission and/or Distribution Service Provider (TDSP) access to ERCOT data and communications.

(f)

(2)	QSEs shall update their Resource COPs and the Outage Scheduler to the best of their ability before the time stated in the AAN when ERCOT will execute the OAE, to reflect any decisions to voluntarily delay or cancel any Outage prior to the OAE so as to remove the Outage from OAE and OSA consideration.

(3)	If, after the planned OAE execution time has passed as noted in paragraph (1)(b) above, ERCOT continues to forecast an inability to meet applicable reliability standards after the updates to the Resource COPs and Outage Schedules, ERCOT may conduct an OAE and issue one or more OSAs.

(a)	ERCOT may contact QSEs representing Resources to be included in the OAE for more information prior to conducting an OAE or issuing an OSA. Otu

(b)	ERCOT may not consider nuclear-powered Generation Resources for an OSA.

(c)	Prior to the execution of an OAE, a QSE may notify ERCOT that a specific Resource cannot be considered in the OAE, for all or part of the period covered by the AAN, due to Resource reliability, compliance with contractual warranty obligations, or other reasons beyond the QSE’s control.  ERCOT will not consider this Resource in the OAE.

(d)	In order to determine which Outages to delay, ERCOT shall first consider the Outage duration, dividing the Outages in categories of zero to two days, two to four days, four to seven days, or more than seven days, then withdraw approval or acceptance on a last in, first out basis within that duration category, so that shorter Outages are delayed first, and the timing of Outage submissions is considered within that category.

(e)	ERCOT may only issue an OSA to the QSE for a Resource that has a COP Resource Status of OUT within the forecasted Emergency Condition described above in this section.

(f)	If the Resource Outage for which the OSA would be issued is scheduled to begin before eight Business Hours have elapsed following issuance of the AAN, ERCOT may issue the OSA prior to the beginning of the Resource Outage after the end of the 24-hour notice period.

(g)	Following the receipt of an OSA, during the OSA Period:

(i)	The QSE for the Resource may choose to show the Resource as OFF in the COP or may elect to leave the Resource On-Line due to equipment or reliability concerns or if the Resource Category is coal or lignite.  If the Resource remains On-Line, it must utilize a status of ONRUC.

(ii)	If the Resource remains On-Line pursuant to paragraph (i) above, it must remain at LSL unless deployed above LSL by SCED.  In addition, the QSE must update the Resource’s Energy Offer Curve to $7,500 for all MWs above LSL.

[NPRR930: Replace paragraph (ii) above with the following upon system implementation:] (ii) If the Resource remains On-Line pursuant to paragraph (i) above, it must remain at LSL unless deployed above LSL by SCED.

(iii)  	If the Resource chooses to show the Resource as OFF in the COP, the Resource may not be self-committed during the OSA Period and shall only be available for commitment by Reliability Unit Commitment.

(4)	ERCOT shall work in good faith with the QSEs to reschedule any delayed or canceled Outages resulting from an AAN under paragraph (1) above, regardless of whether the Resource took voluntary actions or received an OSA.

(a)	If ERCOT issues an OSA, the QSE may submit a new request for approval of the Planned Outage schedule, however the new Outage may not begin prior to the end time of the OSA Period.

(b)	If a transmission Outage was scheduled in coordination with a Resource Outage that is delayed, ERCOT shall also delay that transmission Outage when necessary.

(5)	If insufficient capacity to meet the need described in the AAN is made available through the processes described in paragraphs (2) and (3) above, ERCOT may contact QSEs having Resources with a Resource Status of OUT in the most recently submitted COP to determine if it is feasible for the Outage of those Resources to be ended by the time of the possible Emergency Condition described in the AAN.  ERCOT may issue an OSA to the QSE for any Resource that the QSE agrees can feasibly be returned to service during the period of the possible Emergency Condition described in the AAN.

(6)	If system conditions change such that the need described in the AAN increases, ERCOT shall update the AAN and may repeat the process described in this section.  For any subsequent iterations of this process, ERCOT shall issue the updated AAN with as much lead time as is practical prior to starting any subsequent OAE, but with a minimum of two hours’ notice.

(7)	ERCOT must perform a planning assessment to determine whether to issue an AAN or OSA.  The planning assessment may not assume total renewable production lower than the sum of the selected Wind-powered Generation Resource Production Potential (WGRPP) and PhotoVoltaic Generation Resource Production Potential (PVGRPP) forecasts for each hour less any reasonably expected severe weather impacts.  The available capacity in ERCOT’s planning assessment must include targeted reserve levels and include forecasted capacity available through DC Tie imports or curtailment of DC Tie exports.  ERCOT must post the following inputs of the planning assessment to the MIS Public Area within an hour of issuing an AAN, including but not limited to:

(a)	The Load forecast;

(b)	Load forecast vendor selection;

(c)	Wind forecast;

(d)	Wind forecast vendor selection;

(e)	Solar forecast;

(f)	Solar forecast vendor selection;

(g)	Expected severe weather impacts forecast;

(h)	Targeted reserve levels;

(i)	DC Tie import forecast;

(j)	DC Tie export curtailment forecast;

(k)	Any aggregate derating of Resource(s) and/or Forced Outage assumptions in total MWs; and

(l)	Any aggregated fuel derating assumptions in total MWs.

[NPRR930: Replace paragraph (7) above with the following upon system implementation:] (7) ERCOT must perform a planning assessment to determine whether to issue an AAN or OSA. The planning assessment may not assume total renewable production lower than the sum of the selected Wind-powered Generation Resource Production Potential (WGRPP) and PhotoVoltaic Generation Resource Production Potential (PVGRPP) forecasts for each hour less any reasonably expected severe weather impacts. The available capacity in ERCOT’s planning assessment must include targeted reserve levels and include forecasted capacity available through DC Tie imports or curtailment of DC Tie exports, forecasted capacity provided from Settlement Only Distributed Generators (SODGs) and Settlement Only Transmission Generators (SOTGs), and forecasted capacity from price-responsive Demand based on information reported to ERCOT in accordance with Section 3.10.7.2.1, Reporting of Demand Response. ERCOT must post the following inputs of the planning assessment to the MIS Public Area within an hour of issuing an AAN, including but not limited to: (a) The Load forecast; (b) Load forecast vendor selection; (c) Wind forecast; (d) Wind forecast vendor selection; (e) Solar forecast; (f) Solar forecast vendor selection; (g) Expected severe weather impacts forecast; (h) Targeted reserve levels; (i) DC Tie import forecast; (j) DC Tie export curtailment forecast; (k) SODG and SOTG forecasts; (l) The forecast of capacity provided by price-responsive Demand; (m) Any aggregate derating of Resource(s) and/or Forced Outage assumptions in total MWs; and (n) Any aggregated fuel derating assumptions in total MWs.

(8)	Notwithstanding anything in this Section, ERCOT need not comply with any other requirement in this Section if the occurrence of an unforeseen Real-Time condition requires that ERCOT withdraw approval of one or more Resource Outages in order to meet applicable reliability standards.  The unforeseen Real-Time condition cannot be the result of changes that Ancillary Services are procured to address.  In exercising its discretion under this paragraph, ERCOT is not required to issue an AAN or OAE before issuing an OSA, but shall:

(a)	Issue the OSA to the QSE of the Resource for the purpose of make whole compensation; and

(b)	Present the justification for the out of market action to the Technical Advisory Committee (TAC) at its next meeting that is at least 14 Business Days after the OSA.

5.1	Introduction

(1)	Transmission security analysis and Reliability Unit Commitment (RUC) are used to ensure ERCOT System reliability and to ensure that enough Resource capacity, in addition to Ancillary Service capacity, is committed in the right locations to reliably serve the forecasted Load on the ERCOT System including Direct Current Tie (DC Tie) Load that has not been curtailed.

(2)	ERCOT shall conduct at least one Day-Ahead Reliability Unit Commitment (DRUC) and at least one Hourly Reliability Unit Commitment (HRUC) before each hour of the Operating Day.  ERCOT, in its sole discretion, may conduct a RUC at any time to evaluate and resolve reliability issues.

(3)	The DRUC must be run after the close of the Day-Ahead Market (DAM).

(4)	The DRUC uses Three-Part Supply Offers, capped at the maximum of generic or verifiable minimum energy and Startup Costs, submitted before the DAM by Qualified Scheduling Entities (QSEs) that were considered in the DAM but not awarded in the DAM.  A QSE may not submit a Three-Part Supply Offer to be considered in the DRUC unless the offer was also submitted for consideration in the DAM.

(5)	ERCOT must initiate the HRUC process at least one hour before the Operating Hour to fine-tune the Resource commitments using updated Load forecasts and updated Outage information.

(6)	The RUC Study Period for DRUC is the next Operating Day.  The RUC Study Period for HRUC is the balance of the current Operating Day plus the next Operating Day if the DRUC for the Operating Day has been solved.

(7)	HRUC may decommit Resources only to maintain the reliability of the ERCOT System.

(8)	For each RUC Study Period, the RUC considers capacity requirements for each hour of the RUC Study Period with the objective of minimizing costs based on logic described in Section 5.5.2, Reliability Unit Commitment (RUC) Process.

(9)	The calculated Resource commitments arising from each RUC process, and a list of Off-Line Available Resources having a start-up time of one hour or less, must be reviewed by ERCOT before issuing Dispatch Instructions to QSEs to commit, extend, or decommit Resources.

(10)	The Security Sequence is a set of prerequisite processes for RUC that describes the key system components and inputs that are required to support the RUC process, the RUC process itself, and the ERCOT review of the Resource commitment recommendations made by the RUC process.

(11)	The RUC process may not be used to buy Ancillary Service unless the Ancillary Service Offers submitted in the DAM are insufficient to meet the requirements of the Ancillary Service Plan.

(12)	After the use of market processes to the fullest extent practicable without jeopardizing the reliability of the ERCOT System, any ERCOT Dispatch Instructions for additional capacity that order a QSE to commit a specific Generation Resource to be On-Line shall be considered a RUC Dispatch for the purpose of the Settlement of payments and charges related to the committed Generation Resource.  An Operating Condition Notice (OCN), Advisory, Watch, or Emergency Notice requesting the available capacity of any currently available Generation Resources but not naming specific Generation Resources is not considered a RUC Dispatch for purposes of Settlement.

(13)	ERCOT shall post on the Market Information System (MIS) Certified Area, for each Off-Line Generation Resource that may be selected by an HRUC process, the current time since the Generation Resource last went Off-Line (in hours) and the corresponding start-up times ERCOT is using for each such Off-Line Generation Resource.  The time since the Generation Resource last went Off-Line and start-up times shall be updated at least hourly.

(14)	Prior to 1330 in the Day-Ahead, ERCOT may issue a Weekly Reliability Unit Commitment (WRUC) Verbal Dispatch Instruction (VDI) to inform a QSE that a Resource is required to be On-Line for all or part of a future Operating Day.  Following the receipt of a WRUC:

(a)	The QSE may self-commit the Resource for the WRUC-instructed hours by updating the Resource’s Current Operating Plan (COP) to reflect the appropriate On-Line Resource Status for the WRUC-instructed hours prior to the DRUC process execution for the associated Operating Day.  Resources that have been self-committed by a QSE in accordance with a WRUC:

(i)	May have a Three-Part Supply Offer submitted into the DAM, and any of the WRUC-instructed hours in which the Three-Part Supply Offer is awarded in the DAM become DAM-Committed Intervals for the Resource and are settled accordingly; and

(ii)	Will not be issued a RUC commitment for the WRUC-instructed hours that were self-committed or DAM-committed.

(b)	ERCOT will commit the Resource as part of the DRUC process for the relevant Operating Day for all WRUC-instructed hours not DAM-committed or QSE self-committed.  For all purposes, including RUC Settlement, the Resource will be considered as committed by the DRUC for these hours.

(15)	If ERCOT issues an Outage Schedule Adjustment (OSA) pursuant to Section 3.1.4.6, Outage Coordination of Potential Transmission Emergency Conditions, or Section 3.1.6.9, Withdrawal of Approval or Acceptance and Rescheduling of Approved or Accepted Planned Outages of Resource Facilities, commitmendditi

QSEs with Resources that received an OSA shall be made whole to their actual costs incurred due to delaying or canceling and rescheduling the Outage as described in Section 5.6.5.1, Make-Whole Payment for Canceled or Delayed Outages for Outage Schedule Adjustments (OSAs).

5.5.2	Reliability Unit Commitment (RUC) Process

(1)	The RUC process recommends commitment of Generation Resources, to match ERCOT’s forecasted Load including Direct Current Tie (DC Tie) Schedules, subject to all transmission constraints and Resource performance characteristics.  The RUC process takes into account Resources already committed in the Current Operating Plans (COPs), Resources already committed in previous RUCs, Off-Line Available Resources having a start-up time of one hour or less, and Resource capacity already committed to provide Ancillary Service.  The formulation of the RUC objective function must employ penalty factors on violations of security constraints.  The objective of the RUC process is to minimize costs based on the Resource costs described in paragraphs (4) through (7) below. For all hours of the RUC Study Period within the RUC process, Quick Start Generation Resources (QSGRs) with a COP Resource Status of OFFQS shall be considered as On-Line with Low Sustained Limit (LSL) at zero MW.  QSGRs with a Resource Status of OFFQS shall only be committed by ERCOT through a RUC instruction in instances when a reliability issue would not otherwise be managed through Dispatch Instructions from Security-Constrained Economic Dispatch (SCED).

(2)	The RUC process can recommend Resource decommitment.  ERCOT may only decommit a Resource to resolve transmission constraints that are otherwise unresolvable. Qualifying Facilities (QFs) may be decommitted only after all other types of Resources have been assessed for decommitment.  In addition, the HRUC process provides decision support to ERCOT regarding a Resource decommitment requested by a Qualified Scheduling Entity (QSE).

(3)	ERCOT shall review the RUC-recommended Resource commitments and the list of Off-Line Available Resources having a start-up time of one hour or less to assess feasibility and shall make any changes that it considers necessary, in its sole discretion.  ERCOT may deselect Resources recommended in DRUC and in all HRUC processes if in ERCOT’s sole discretion there is enough time to commit those Resources in the future HRUC processes, taking into account the Resources’ start-up times, to meet ERCOT System reliability.  After each RUC run, ERCOT shall post the amount of capacity deselected per hour in the RUC Study Period to the MIS Secure Area.  A Generation Resource shown as On-Line and available for SCED dispatch for an hour in its COP prior to a DRUC or HRUC process execution, according to Section 5.3, ERCOT Security Sequence Responsibilities, will be considered self-committed for that hour.  For purpose of Settlement, snapshot data will be used as specified in paragraph (2) of Section 5.3.  ERCOT shall issue RUC instructions to each QSE specifying its Resources that have been committed as a result of the RUC process.  ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the MIS Secure Area any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes.

[NPRR884: Replace paragraph (3) above with the following upon system implementation:] (3) ERCOT shall review the RUC-recommended Resource commitments and the list of Off-Line Available Resources having a start-up time of one hour or less to assess feasibility and shall make any changes that it considers necessary, in its sole discretion. During the RUC process, ERCOT may also review and commit, through a RUC instruction, Combined Cycle Generation Resources that are currently planned to be On-Line but are capable of transitioning to a configuration with additional capacity. ERCOT may deselect Resources recommended in DRUC and in all HRUC processes if in ERCOT’s sole discretion there is enough time to commit those Resources in the future HRUC processes, taking into account the Resources’ start-up times, to meet ERCOT System reliability. After each RUC run, ERCOT shall post the amount of capacity deselected per hour in the RUC Study Period to the MIS Secure Area. A Generation Resource shown as On-Line and available for SCED dispatch for an hour in its COP prior to a DRUC or HRUC process execution, according to Section 5.3, ERCOT Security Sequence Responsibilities, will be considered self-committed for that hour. For purpose of Settlement, snapshot data will be used as specified in paragraph (2) of Section 5.3. ERCOT shall issue RUC instructions to each QSE specifying its Resources that have been committed as a result of the RUC process. ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the MIS Secure Area any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes.

(4)	To determine the projected energy output level of each Resource and to project potential congestion patterns for each hour of the RUC, ERCOT shall calculate proxy Energy Offer Curves based on the Mitigated Offer Caps (MOCs) for the type of Resource as specified in Section 4.4.9.4, Mitigated Offer Cap and Mitigated Offer Floor, for use in the RUC.  Proxy Energy Offer Curves are calculated by multiplying the MOC by a constant selected by ERCOT from time to time that is no more than 0.10% and applying the cost for all Generation Resource output between High Sustained Limit (HSL) and LSL.  The intent of this process is to minimize the effect of the proxy Energy Offer Curves on optimization.

(5)	ERCOT shall use the RUC process to evaluate the need to commit Resources for which a QSE has submitted Three-Part Supply Offers and other available Off-Line Resources in addition to Resources that are planned to be On-Line during the RUC Study Period.  All of the above commitment information must be as specified in the QSE’s COP.  For available Off-Line Resources with a cold start time of one hour or less that have not been removed from special consideration under paragraph (7) below pursuant to paragraph (4) of Section 8.1.2, Current Operating Plan (COP) Performance Requirements, the Startup Offers and Minimum-Energy Offer from a Resource’s Three-Part Supply Offer shall not be used in the RUC process.

(6)	ERCOT shall create Three-Part Supply Offers for all Resources that did not submit a Three-Part Supply Offer, but are specified as available but Off-Line, excluding Resources with a Resource Status of EMR, in a QSE’s COP.  For such Resources, excluding available Off-Line Resources with a cold start time of one hour or less that have not been removed from special consideration under paragraph (7) below pursuant to paragraph (4) of Section 8.1.2, ERCOT shall use in the RUC process 150% of any approved verifiable Startup Cost and verifiable minimum-energy cost or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and the applicable Resource Category Generic Minimum-Energy Offer Cost as described specified in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, registered with ERCOT.  However for Settlement purposes, ERCOT shall use any approved verifiable Startup Costs and verifiable minimum-energy cost for such Resources, or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and Generic Minimum-Energy Offer Cost.

(7)	For all available Off-Line Resources having a cold start time of one hour or less and not removed from special consideration pursuant to paragraph (4) of Section 8.1.2, ERCOT shall scale any approved verifiable Startup Cost and verifiable minimum-energy cost or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and the applicable Resource Category Generic Minimum-Energy Offer Cost as specified in Section 4.4.9.2.3 for use in the RUC process.

The above parameter is defined as follows:

| Parameter | Unit | Current Value* |
| --- | --- | --- |
| 1HRLESSCOSTSCALING | Percentage | Maximum value of 20% |
| * The current value for the parameter(s) referenced in this table above will be recommended by the Technical Advisory Committee (TAC) and approved by the ERCOT Board. ERCOT shall update parameter value(s) on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value. |  |  |

(8)	The RUC process must treat all Resource capacity providing Ancillary Service as unavailable for the RUC Study Period, unless that treatment leads to infeasibility (i.e., that capacity is needed to resolve some local transmission problem that cannot be resolved by any other means).  If an ERCOT Operator decides that the Ancillary Service capacity allocated to that Resource is infeasible based on ERCOT System conditions, then, ERCOT shall inform each affected QSE of the amount of its Resource capacity that does not qualify to provide Ancillary Service, and the projected hours for which this is the case.  In that event, the affected QSE may, under Section 6.4.9.1.2, Replacement of Infeasible Ancillary Service Due to Transmission Constraints, either:

(a) 	Substitute capacity from Resources represented by that QSE;

(b)	Substitute capacity from other QSEs using Ancillary Service Trades; or

(c)	Ask ERCOT to replace the capacity.

(9)	Factors included in the RUC process are:

(a)	ERCOT System-wide hourly Load forecast allocated appropriately over Load buses;

(b)	Transmission constraints – Transfer limits on energy flows through the electricity network;

(i)	Thermal constraints – protect transmission facilities against thermal overload;

(ii)	Generic constraints – protect the transmission system against transient instability, dynamic instability or voltage collapse;

(c)	Planned transmission topology;

(d)	Energy sufficiency constraints;

(e)	Inputs from the COP, as appropriate;

(f)	Inputs from Resource Parameters, including a list of Off-Line Available Resources having a start-up time of one hour or less, as appropriate;

(g)	Each Generation Resource’s Minimum-Energy Offer and Startup Offer, from its Three-Part Supply Offer;

(h)	Any Generation Resource that is Off-Line and available but does not have a Three-Part Supply Offer;

(i)	Forced Outage information; and

(j)	Inputs from the eight-day look ahead planning tool, which may potentially keep a unit On-Line (or start a unit for the next day) so that a unit minimum duration between starts does not limit the availability of the unit (for security reasons).

(10)	The HRUC process and the DRUC process are as follows:

(a)	The HRUC process uses current Resource Status for the initial condition for the first hour of the RUC Study Period.  All HRUC processes use the projected status of transmission breakers and switches starting with current status and updated for each remaining hour in the study as indicated in the COP for Resources and in the Outage Scheduler for transmission elements.

(b)	The DRUC process uses the Day-Ahead forecast of total ERCOT Load including DC Tie Schedules for each hour of the Operating Day.  The HRUC process uses the current hourly forecast of total ERCOT Load including DC Tie Schedules for each hour in the RUC Study Period.

(c)	The DRUC process uses the Day-Ahead weather forecast for each hour of the Operating Day.  The HRUC process uses the weather forecast information for each hour of the balance of the RUC Study Period.

(11)	A QSE that has one or more of its Resources RUC-committed to provide Ancillary Services must increase its Ancillary Service Supply Responsibility by the total amount of RUC-committed Ancillary Service quantities.  The QSE may only use a RUC-committed Resource to meet its Ancillary Service Supply Responsibility during that Resource’s RUC-Committed Interval if the Resource has been committed by the RUC process to provide Ancillary Service.  The QSE shall indicate the exact amount and type of Ancillary Service for which it was committed as the Resource’s Ancillary Service Resource Responsibility and Ancillary Services Schedule for the RUC-Committed Intervals for both telemetry and COP information provided to ERCOT.  Upon deployment of the Ancillary Services, the QSE shall adjust its Ancillary Services Schedule to reflect the amounts requested in the deployment.

[NPRR884: Replace paragraph (11) above with the following upon system implementation:] (11) A QSE that has one or more of its Resources RUC-committed to provide Ancillary Services must increase its Ancillary Service Supply Responsibility by the total amount of RUC-committed Ancillary Service quantities. The QSE may only use a RUC-committed Resource to meet its Ancillary Service Supply Responsibility during that Resource’s RUC-Committed Interval if the Resource has been committed by the RUC process to provide Ancillary Service, or the Resource is a Combined Cycle Generation Resource that was RUC-committed to transition from one On-Line configuration to a different configuration with additional capacity. For cases in which the commitment was to provide Ancillary Service, the QSE shall indicate the exact amount and type of Ancillary Service for which it was committed as the Resource’s Ancillary Service Resource Responsibility and Ancillary Services Schedule for the RUC-Committed Intervals for both telemetry and COP information provided to ERCOT. Upon deployment of the Ancillary Services, the QSE shall adjust its Ancillary Services Schedule to reflect the amounts requested in the deployment.

(12)	A QSE with a Resource that is not a Reliability Must-Run (RMR) Unit or has not received an Outage Schedule Adjustment (OSA) that has been committed in a RUC process or by a RUC Verbal Dispatch Instruction (VDI) may opt out of the RUC Settlement (or “buy back” the commitment) by setting the telemetered Resource Status of the RUC-committed Resource to ONOPTOUT for the first SCED run that the Resource is On-Line and available for SCED dispatch during the first hour of a contiguous block of RUC-Committed Hours.  A QSE that opts out of RUC Settlement forfeits RUC Settlement for the affected Resource for a given block of RUC Buy-Back Hours.  A QSE that opts out of RUC Settlement treatment must make the Resource available to SCED for all RUC Buy-Back Hours.  All hours in a contiguous block of RUC-Committed Hours that includes the RUC Buy-Back Hour shall be considered RUC Buy-Back Hours.  However, if a contiguous block of RUC-Committed Hours spans more than one Operating Day, each contiguous block of RUC-Committed Hours within each Operating Day shall be treated as an independent block for purposes of opting out, and a QSE that wishes to opt out of RUC Settlement for the RUC-Committed Hours in the next Operating Day must set its telemetered Resource Status to ONOPTOUT for the first SCED run the next Operating Day.

[NPRR884: Replace paragraph (12) above with the following upon system implementation:] (12) A QSE with a Resource that is not a Reliability Must-Run (RMR) Unit or has not received an Outage Schedule Adjustment (OSA) that has been committed in a RUC process or by a RUC Verbal Dispatch Instruction (VDI) may opt out of the RUC Settlement (or “buy back” the commitment) by setting the telemetered Resource Status of the RUC-committed Resource to ONOPTOUT for the first SCED run that the Resource is On-Line and available for SCED dispatch during the first hour of a contiguous block of RUC-Committed Hours. All the configurations of the same Combined Cycle Train shall be treated as the same Resource for the purpose of creating the block of RUC-Committed Hours. A RUC-committed Combined Cycle Generation Resource may opt out of the RUC Settlement by setting the telemetered Resource Status to ONOPTOUT for any On-Line configuration of the same Combined Cycle Train for the first SCED run that the Combined Cycle Train is On-Line and available for SCED Dispatch during the first hour of a contiguous block of RUC-Committed Hours. A Combined Cycle Generation Resource that is RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity may opt out of the RUC Settlement following the same rule for RUC-committed Combined Cycle Generation Resources described above. A QSE that opts out of RUC Settlement forfeits RUC Settlement for the affected Resource for a given block of RUC Buy-Back Hours. A QSE that opts out of RUC Settlement treatment must make the Resource available to SCED for all RUC Buy-Back Hours. All hours in a contiguous block of RUC-Committed Hours that includes the RUC Buy-Back Hour shall be considered RUC Buy-Back Hours. However, if a contiguous block of RUC-Committed Hours spans more than one Operating Day, each contiguous block of RUC-Committed Hours within each Operating Day shall be treated as an independent block for purposes of opting out, and a QSE that wishes to opt out of RUC Settlement for the RUC-Committed Hours in the next Operating Day must set its telemetered Resource Status to ONOPTOUT for the first SCED run the next Operating Day.

(13)	If a QSE-committed Resource experiences a Forced Outage or Startup Loading Failure in an hour for which another Resource under the control of the same QSE is committed by a RUC instruction, the QSE may opt out of RUC Settlement for the RUC-committed Resource in accordance with paragraph (12) above, or if the Forced Outage or Startup Loading Failure occurs after the beginning of the first RUC-Committed Interval, the QSE may opt out of RUC Settlement by submitting a dispute pursuant to Section 9.14, Settlement and Billing Dispute Process, requesting a correction of the RUC Settlement treatment for the RUC-committed Resource.

(14)	ERCOT shall, as soon as practicable, post to the MIS Secure Area a report identifying those hours that were considered RUC Buy-Back Hours, along with the name of each RUC-committed Resource whose QSE opted out of RUC Settlement.

(15)	A Resource that has a Three-Part Supply Offer cleared in the Day-Ahead Market (DAM) and subsequently receives a RUC commitment for the Operating Hour for which it was awarded will be treated as if the telemetered Resource Status was ONOPTOUT for purposes of Section 6.5.7.3, Security Constrained Economic Dispatch, and Section 6.5.7.3.1, Determination of Real-Time On-Line Reliability Deployment Price Adder.

5.6.5	Settlement for Canceled or Delayed Outages for Outage Schedule Adjustments (OSAs)

5.6.5.1	Make-Whole Payment for Canceled or Delayed Outages for OSAs

(1) 	If ERCOT issues an Outage Schedule Adjustment (OSA) pursuant to Section 3.1.4.6, Outage Coordination of Potential Transmission Emergency Conditions, or Section 3.1.6.9, Withdrawal of Approval or Acceptance and Rescheduling of Approved or Accepted Planned Outages of Resource Facilities, ERCOT shall pay the QSE representing the Resource a Make-Whole Payment as calculated in Section 5.6.5.2, RUC Make-Whole Payment and RUC Clawback Charge for Resources receiving OSAs, if the QSE has:

(a)	Submitted a Settlement and billing dispute consistent with the dispute process described in Section 9.14, Settlement and Billing Dispute Process; and

(b)	Submitted the following within 60 days of the issuance of a Real-Time Market (RTM) Initial Statement for an Operating Day on which one or more OSAs was in effect for the Resource:

(i)	An attestation signed by an officer or executive with authority to bind the QSE stating that the information contained in the submission is accurate;

(ii)	The dollar amount and calculation of the net financial loss, if applicable, by Settlement Interval for:

(A)	Actual and indirect costs incurred due to delaying or canceling and rescheduling the Outage.  Such costs include, but are not limited to:

(1)	Additional staff or contractor time;

(2)	Costs associated with re-planning the Outage;

(3)	Costs of equipment rental (including but not limited to cranes, manlifts, welding machines, etc.);

(4)	Costs of facility rentals and other incidental incremental costs incurred by the Resource, its QSE, or its fuel supplier (e.g. mine-related expenses) created by the cancellation or delay of the Outage; and

(5)	Indirect costs necessary for moving any additional Outages due to the Outage Schedule Adjustment (OSA).

(B)	Costs covered by paragraph (A) above do not include:

(1)	The cost of materials due to be installed during the Outage.  Such equipment will presumably be installed at a later date in a rescheduled or delayed Outage; and

(2) 	All loss amounts associated with the Outage Resource as a result of any financial transaction, including selling or repurchasing a hedge (whether the hedge is for energy, Ancillary Services, or fuel); and

(iii)	Sufficient documentation to support the QSE’s calculation of all submitted costs.

.

- 5.6.5.2	RUC Make-Whole Payment and RUC Clawback Charge for Resources Receiving OSAs

(1)	To compensate QSEs representing Resources that submitted a timely Settlement and billing dispute, ERCOT shall calculate a RUC Make-Whole Payment for an Operating Day for the OSA Period, allocated to each instructed Operating Hour as follows:

(a)	For a Resource with a RUC instruction issued, the RUC Guarantee shall include the following:

(i)	Eligible Startup costs per Section 5.6.2, RUC Startup Cost Eligibility;

(ii)	Minimum-energy costs;

(iii)	10% of both Startup costs and minimum-energy costs; and

(iv)	Approved net financial loss as defined in Section 5.6.5.1, Make-Whole Payment for Canceled or Delayed Outages for Outage Schedule Adjustments (OSAs)

(b)	For a Resource without a RUC Instruction issued, the RUC Guarantee shall include only the following:

(i)	Approved net financial loss as defined in Section 5.6.5.1.

(2) 	Notwithstanding the clawback provisions described in Section 5.7.2, RUC Clawback Charge, in the calculation of RUC Make-Whole Payments as described in paragraph (1) above, the clawback percentage shall be set at 100%.

- 5.6.5.3	Timeline for Calculating RUC Clawback Charges for Resources Receiving OSAs

(1)	Within ten Business Days after the issuance of the RTM True-Up Statement, ERCOT will issue a Miscellaneous Invoice to charge the QSE representing the Resource that has received an OSA the excess revenues not clawed back in RUC Settlements.

(2)	All clawed back revenues will be paid to QSEs based on a 15-minute Load-Ratio Share basis.

5.7.2	RUC Clawback Charge

(1)	A QSE for a Resource shall pay a RUC Clawback Charge for the Operating Day if the RUC Guarantee is less than the sum of:

(a)	RUC Minimum-Energy Revenue calculated in Section 5.7.1.2, RUC Minimum-Energy Revenue;

(b)	Revenue Less Cost Above LSL During RUC-Committed Hours calculated in Section 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours; and

(c)	Revenue Less Cost During QSE-Clawback Intervals calculated in Section 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals.

(2)	The amount of the RUC Clawback Charge is a percentage of the difference calculated in paragraph (1) above.  Whether or not the QSE submits a Three-Part Supply Offer for a Resource in the Day Ahead Market (DAM) determines if that Resource will have a clawback applied in its Settlement.  If the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM, then the clawback percentage in RUC Committed Hours is 50% and the clawback percentage in QSE Clawback Intervals is 0%.  If not, then the clawback percentage in RUC Committed Hours is 100% and the clawback percentage in QSE Clawback Intervals is 50%.

(3)	If an Energy Emergency Alert (EEA) is in effect for any period of the Operating Day, then in all RUC Committed Hours and all QSE Clawback Intervals of the Operating Day the clawback percentage is 0% if the QSE submitted a validated Three Part Supply Offer for the Resource into the DAM and 50% otherwise.

(4)	For Combined Cycle Trains, if at least one Combined Cycle Generation Resource is offered into the DAM, then the Combined Cycle Train is considered to be offered into the DAM.

(5)	The RUC Clawback Charge for a Resource, including RMR Units, for each Operating Day is allocated evenly over the RUC-Committed Hours for that Resource

(6)	For each RUC-committed Resource, the RUC Clawback Charge for each RUC-Committed Hour of the Operating Day is calculated as follows:

If (RUCMEREV q, r, d + RUCEXRR q, r, d – RUCG q, r, d) > 0,

Then,

RUCCBAMT q, r, h	=	[(RUCMEREV q, r, d + RUCEXRR q, r, d – RUCG q, r, d) * RUCCBFR q, r, d + RUCEXRQC q, r, d * RUCCBFC q, r, d] / RUCHR q, r, d

Otherwise,

RUCCBAMT q, r, h 	= [Max (0, RUCMEREV q, r, d + RUCEXRR q, r, d + RUCEXRQC q, r, d – RUCG q, r, d) * RUCCBFC q, r, d] / RUCHR q, r, d

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCCBAMT q, r, h | $ | RUC Clawback Charge––The RUC Clawback Charge to a QSE for Resource r represented by QSE q as described in this Section, for each RUC-Committed Hour h of the Operating Day for that Resource. When one or more Combined Cycle Generation Resources are committed by RUC, a charge is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCG q, r, d | $ | RUC Guarantee—The sum of eligible Startup Costs and Minimum-Energy Costs for Resource r represented by QSE q during all RUC-Committed Hours, for the Operating Day d. See Section 5.7.1.1, RUC Guarantee. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCMEREV q, r, d | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for generation of Resource r represented by QSE q up to LSL during all RUC-Committed Hours, for the Operating Day d. See Section 5.7.1.2. When one or more Combined Cycle Generation Resources are committed by RUC, RUC Minimum-Energy Revenue is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCEXRR q, r, d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for Resource r represented by QSE q above the LSL less the cost during all RUC-Committed Hours, for the Operating Day d. See Section 5.7.1.3. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost Above LSL During RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCEXRQC q, r, d | $ | Revenue Less Cost from QSE-Clawback Intervals—The sum of the total revenue for Resource r represented by QSE q less the cost during all QSE-Clawback Intervals for the Operating Day d. See Section 5.7.1.4. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost from QSE-Clawback Intervals is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources earning revenue in QSE Clawback Intervals. |
| RUCCBFR q, r, d | none | RUC Clawback Factor for RUC-Committed Hours—The Clawback Factor for Resource r represented by QSE q for RUC-Committed Hours, as specified in paragraphs (2) and (3) above, for the Operating Day d. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for RUC-Committed Hours is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCCBFC q, r, d | none | RUC Clawback Factor for QSE Clawback Intervals—The Clawback Factor for Resource r represented by QSE q for QSE Clawback Intervals, as specified in paragraphs (2) and (3) above, for the Operating Day d. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for QSE Clawback Intervals is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCHR q, r, d | none | RUC Hour—The total number of RUC-Committed Hours, for Resource r represented by QSE q for the Operating Day d. When one or more Combined Cycle Generation Resources are committed by RUC, the total number of RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| h | none | An hour in the RUC-commitment period. |

[NPRR884: Replace paragraph (6) above with the following upon system implementation:] (6) For each RUC-committed Resource, the RUC Clawback Charge for each RUC-Committed Hour of the Operating Day is calculated as follows: If (RUCMEREV q, r, d + RUCEXRR q, r, d – RUCACREV q, r, d – RUCG q, r, d) > 0, Then, RUCCBAMT q, r, h = [(RUCMEREV q, r, d + RUCEXRR q, r, d – RUCACREV q, r, d – RUCG q, r, d) * RUCCBFR q, r, d + RUCEXRQC q, r, d * RUCCBFC q, r, d] / RUCHR q, r, d Otherwise, RUCCBAMT q, r, h = [Max (0, RUCMEREV q, r, d + RUCEXRR q, r, d + RUCEXRQC q, r, d – RUCACREV q, r, d – RUCG q, r, d) * RUCCBFC q, r, d] / RUCHR q, r, d Where, The RUCAC revenue is calculated for a Combined Cycle Train as follows: RUCACREV q, r, d = Max{0, RUCMEREV96 q, r, i + Max(0, RUCEXRR96 q, r, i)} The above variables are defined as follows: Variable; Unit; Definition. RUCCBAMT q, r, h; $; RUC Clawback Charge––The RUC Clawback Charge to a QSE for Resource r represented by QSE q as described in this Section, for each RUC-Committed Hour h of the Operating Day for that Resource. When one or more Combined Cycle Generation Resources are committed by RUC, a charge is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCG q, r, d; $; RUC Guarantee—The sum of eligible Startup Costs and Minimum-Energy Costs for Resource r represented by QSE q during all RUC-Committed Hours, for the Operating Day d. See Section 5.7.1.1, RUC Guarantee. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCMEREV q, r, d; $; RUC Minimum-Energy Revenue—The sum of the energy revenues for generation of Resource r represented by QSE q up to LSL during all RUC-Committed Hours, for the Operating Day d. See Section 5.7.1.2. When one or more Combined Cycle Generation Resources are committed by RUC, RUC Minimum-Energy Revenue is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCEXRR q, r, d; $; Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for Resource r represented by QSE q above the LSL less the cost during all RUC-Committed Hours, for the Operating Day d. See Section 5.7.1.3. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost Above LSL During RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCEXRQC q, r, d; $; Revenue Less Cost from QSE-Clawback Intervals—The sum of the total revenue for Resource r represented by QSE q less the cost during all QSE-Clawback Intervals for the Operating Day d. See Section 5.7.1.4. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost from QSE-Clawback Intervals is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources earning revenue in QSE Clawback Intervals.. RUCACREV q, r, d; $; Revenue from RUCAC Hours—The net positive sum for the energy revenues for generation of Resource r represented by QSE q up to LSL and the total revenue for Resource r operating above its LSL less the cost during all RUCAC-Hours, for the Operating Day d. When one or more Combined Cycle Generation Resources are RUCAC, revenue from RUCAC Hours is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources that were RUC-committed during the RUCAC-Hours.. RUCMEREV96 q, r, i; $; RUC Minimum-Energy Revenue by Interval—The energy revenues for generation of Resource r represented by QSE q up to LSL during all RUC-Committed Hours, for the Settlement Interval i. When one or more Combined Cycle Generation Resources are committed by RUC, RUC Minimum-Energy Revenue is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. During RUCAC-Intervals for a Combined Cycle Train, the minimum energy revenue is calculated as the difference between the minimum energy revenue of the RUC-committed configuration and the QSE-committed configuration.. RUCEXRR96 q, r, i; $; Revenue Less Cost Above LSL During RUC-Committed Hours by Interval—The total revenue for Resource r represented by QSE q operating above its LSL less the cost during all RUC-Committed hours, for the Settlement Interval i. When one or more Combined Cycle Generation Resources are committed by RUC, revenue less cost above LSL is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCCBFR q, r, d; none; RUC Clawback Factor for RUC-Committed Hours—The Clawback Factor for Resource r represented by QSE q for RUC-Committed Hours, as specified in paragraphs (2) and (3) above, for the Operating Day d. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for RUC-Committed Hours is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCCBFC q, r, d; none; RUC Clawback Factor for QSE Clawback Intervals—The Clawback Factor for Resource r represented by QSE q for QSE Clawback Intervals, as specified in paragraphs (2) and (3) above, for the Operating Day d. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for QSE Clawback Intervals is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCHR q, r, d; none; RUC Hour—The total number of RUC-Committed Hours, for Resource r represented by QSE q for the Operating Day d. When one or more Combined Cycle Generation Resources are committed by RUC, the total number of RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. q; none; A QSE.. r; none; A RUC-committed Generation Resource.. d; none; An Operating Day containing the RUC-commitment.. h; none; An hour in the RUC-commitment period.. i; none; A 15-minute Settlement Interval within the hour that includes a RUCAC instruction..

5.7.4	RUC Make-Whole Charges

(1)	All QSEs that were capacity-short in each RUC will be charged for that shortage, as described in Section 5.7.4.1, RUC Capacity-Short Charge.  If the revenues from the charges under Section 5.7.4.1 are not enough to cover all RUC Make-Whole Payments for a Settlement Interval, then the difference will be uplifted to all QSEs on a Load Ratio Share (LRS) basis, as described in Section 5.7.4.2, RUC Make-Whole Uplift Charge.

(2)	On a monthly basis, within ten days after the Initial Settlement of the last day of the month has been completed, ERCOT shall post on the Market Information System (MIS) Secure Area the total RUC Make-Whole Charges and RUC Clawback Payment Amounts, by Settlement Interval, by QSE capacity-shortfall and by amount uplifted.

5.8	Annual RUC Reporting Requirement

(1)	ERCOT shall report to the Technical Advisory Committee (TAC), each January, an assessment of market impacts and Settlements for the aggregate Reliability Unit Commitment (RUC) activity, delineated by type of RUC instruction as follows:

(a) 	RUC instructions issued for Ancillary Service shortages (failure to sufficiently procure one or more Ancillary Service markets in the Day-Ahead Market (DAM) or subsequent Supplemental Ancillary Service Markets (SASMs));

(b)	RUC instructions issued for irresolvable transmission system constraints;

(c) 	RUC instructions issued in anticipation of extreme cold weather/startup failures;

(d)	RUC instructions issued for capacity;

(e)	RUC instructions issued for system inertia;

(f)	RUC instructions issued  to Resources receiving an Outage Schedule Adjustment; and

(g)	A summary of RUC Settlements;

(i)	RUC charges associated with RUC Make-Whole Amount Total per RUC, as defined in Section 5.7.4.1, RUC Capacity-Short Charge; and

(ii)	RUC Shortfall Total, as defined in Section 5.7.4.1.1, Capacity Shortfall Ratio Share.

- 6.5.7.3	Security Constrained Economic Dispatch

(1)	The SCED process is designed to simultaneously manage energy, the system power balance and network congestion through Resource Base Points and calculation of LMPs every five minutes.  The SCED process uses a two-step methodology that applies mitigation prospectively to resolve Non-Competitive Constraints for the current Operating Hour.  The SCED process evaluates Energy Offer Curves, Output Schedules and Real-Time Market (RTM) Energy Bids to determine Resource Dispatch Instructions by maximizing bid-based revenues minus offer-based costs, subject to power balance and network constraints.  The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.  An RTM Energy Bid represents the bid for energy distributed across all nodes in the Load Zone in which the Controllable Load Resource is located.

(2)	The SCED solution must monitor cumulative deployment of Regulation Services and ensure that Regulation Services deployment is minimized over time.

(3)	In the Generation To Be Dispatched (GTBD) determined by LFC, ERCOT shall subtract the sum of the telemetered net real power consumption from all Controllable Load Resources available to SCED.

(4)	For use as SCED inputs, ERCOT shall use the available capacity of all committed Generation Resources by creating proxy Energy Offer Curves for certain Resources as follows:

(a)	Non-IRRs and Dynamically Scheduled Resources (DSRs) without Energy Offer Curves

(i)	ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below for:

(A)	Each non-IRR for which its QSE has submitted an Output Schedule instead of an Energy Offer Curve; and

(B)	Each DSR that has not submitted incremental and decremental Energy Offer Curves.

| MW | Price (per MWh) |
| --- | --- |
| HSL | SWCAP |
| Output Schedule MW plus 1 MW | SWCAP minus $0.01 |
| Output Schedule MW | -$249.99 |
| LSL | -$250.00 |

(b)	DSRs with Energy Offer Curves

(i)	For each DSR that has submitted incremental and decremental Energy Offer Curves, ERCOT shall create a monotonically increasing proxy Energy Offer Curve.  That curve must consist of the incremental Energy Offer Curve that reflects the available capacity above the Resource’s Output Schedule to its HSL and the decremental Energy Offer Curve that reflects the available capacity below the Resource’s Output Schedule to the LSL.  The curve must be created as described below:

| MW | Price (per MWh) |
| --- | --- |
| Output Schedule MW plus 1 MW to HSL | Incremental Energy Offer Curve |
| LSL to Output Schedule MW | Decremental Energy Offer Curve |

(c)	Non-IRRs without full-range Energy Offer Curves

(i)	For each non-IRR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the Resource’s available capacity, ERCOT shall create a proxy Energy Offer Curve that extends the submitted Energy Offer Curve to use the entire available capacity of the Resource above the highest point on the Energy Offer Curve to the Resource’s HSL and the offer floor from the lowest point on the Energy Offer Curve to its LSL, using these points:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(d)	IRRs

(i)	For each IRR that has not submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | $1,500 |
| HSL minus 1 MW | -$249.99 |
| LSL | -$250.00 |

(ii)	For each IRR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the IRR’s available capacity, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with the highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(e)	RUC-committed Resources

(i)        For each RUC-committed Resource that has not submitted an Energy Offer Curve, ERCOT shall create a proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | $1,500 |
| Zero | $1,500 |

(ii)       For each RUC-committed Resource that has submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in Energy Offer Curve) | Greater of $1,500 or price associated with the highest MW in QSE submitted Energy Offer Curve |
| Energy Offer Curve | Greater of $1,500 or the QSE submitted Energy Offer Curve |
| Zero | Greater of $1,500 or the first price point of the QSE submitted Energy Offer Curve |

[NPRR930: Insert paragraph (iii) below upon system implementation:] (iii) For each RUC-committed Resource during the time period stated in the Advance Action Notice if any Resource received an Outage Schedule Adjustment, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL; $7,500. Zero; $7,500.

[NPRR884: Insert paragraphs (iv) and (v) below upon system implementation:] (iv) For each Combined Cycle Generation Resource that was RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity, as instructed by ERCOT, that has not submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL of RUC-committed configuration; $1,500. Zero; $1,500. (v) For each Combined Cycle Generation Resource that was RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity, as instructed by ERCOT, that has submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL of RUC-committed configuration (if more than highest MW in Energy Offer Curve); Greater of $1,500 or price associated with the highest MW in QSE submitted Energy Offer Curve. Energy Offer Curve for MW at and above HSL of QSE-committed configuration; Greater of $1,500 or the QSE submitted Energy Offer Curve. HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve); Greater of $1,500 or price associated with the highest MW in QSE submitted Energy Offer Curve. Energy Offer Curve for MW at and below HSL of QSE-committed configuration; The QSE submitted Energy Offer Curve. 1 MW below lowest MW in Energy Offer Curve (if more than LSL); -$249.99. LSL (if less than lowest MW in Energy Offer Curve); -$250.00.

(5)	The Entity with decision making authority, as more fully described in Section 3.19.1, Constraint Competitiveness Test Definitions, over how a Resource or Split Generation Resource is offered or scheduled, shall be responsible for all offers associated with each Resource, including offers represented by a proxy Energy Offer Curve.

(6)	For a Controllable Load Resource whose QSE has submitted an RTM Energy Bid that does not cover the full range of the Resource’s available Demand response capability, consistent with the Controllable Load Resource’s telemetered quantities, ERCOT shall create a proxy energy bid as described below:

| MW | Price (per MWh) |
| --- | --- |
| LPC to MPC minus maximum MW of RTM Energy Bid | Price associated with the lowest MW in submitted RTM Energy Bid curve |
| MPC minus maximum MW of RTM Energy Bid to MPC | RTM Energy Bid curve |
| MPC | Right-most point (lowest price) on RTM Energy Bid curve |

(7)	ERCOT shall ensure that any RTM Energy Bid is monotonically non-increasing.  The QSE representing the Controllable Load Resource shall be responsible for all RTM Energy Bids, including bids updated by ERCOT as described above.

(8)	A Controllable Load Resource with a telemetered status of OUTL is not considered as dispatchable capacity by SCED.  A QSE may use this function to inform ERCOT of instances when the Controllable Load Resource is unable to follow SCED Dispatch Instructions.  Under all telemetered statuses including OUTL, the remaining telemetry quantities submitted by the QSE shall represent the operating conditions of the Controllable Load Resource that can be verified by ERCOT.  A QSE representing a Controllable Load Resource with a telemetered status of OUTL is still obligated to provide any applicable Ancillary Service Resource Responsibilities previously awarded to that Controllable Load Resource.

(9)	Energy Offer Curves that were constructed in whole or in part with proxy Energy Offer Curves shall be so marked in all ERCOT postings or references to the energy offer.

(10)	The two-step SCED methodology referenced in paragraph (1) above is:

(a)	The first step is to execute the SCED process to determine Reference LMPs.  In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints.  Energy Offer Curves for all On-Line Generation Resources and RTM Energy Bids from available Controllable Load Resources, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

(b)	The second step is to execute the SCED process to produce Base Points, Shadow Prices, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints.  The second step must:

(i)	Use Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT.  Each Energy Offer Curve must be bounded at the lesser of the Reference LMP (from Step 1) or the appropriate Mitigated Offer Floor.  In addition, each Energy Offer Curve subject to mitigation under the criteria described in Section 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test, must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node plus a variable not to exceed 0.01 multiplied by the value of the Resource’s Mitigated Offer Cap (MOC) curve at the LSL or the appropriate MOC;

(ii)	Use RTM Energy Bid curves for all available Controllable Load Resources, whether submitted by QSEs or created by ERCOT.  There is no mitigation of RTM Energy Bids; and

(iii)	Observe all Competitive and Non-Competitive Constraints.

(c)	ERCOT shall archive information and provide monthly summaries of security violations and any binding transmission constraints identified in Step 2 of the SCED process.  The summary must describe the limiting element (or identified operator-entered constraint with operator’s comments describing the reason and the Resource-specific impacts for any manual overrides).  ERCOT shall provide the summary to Market Participants on the MIS Secure Area and to the Independent Market Monitor (IMM).

(11)	For each SCED process, in addition to the binding Base Points and LMPs, ERCOT shall calculate a non-binding projection of the Base Points and Resource Node LMPs, Real-Time Reliability Deployment Price Adders, Real-Time On-Line Reserve Price Adders, Real-Time Off-Line Reserve Price Adders, Hub LMPs and Load Zone LMPs at a frequency of every five minutes for at least 15 minutes into the future based on the same inputs to the SCED process as described in this Section, except that the Resource’s HDL and LDL and the total generation requirement will be as estimated at future intervals.  The Resource’s HDL and LDL will be calculated for each interval of the projection based on the ramp rate capability over the study period.  ERCOT shall estimate the projected total generation requirement by calculating a Load forecast for the study period.  In lieu of the steps described in Section 6.5.7.3.1, Determination of Real-Time On-Line Reliability Deployment Price Adder, the non-binding projection of Real-Time Reliability Deployment Price Adders shall be estimated based on GTBD, reliability deployments MWs, and aggregated offers.  The Energy Offer Curve from SCED Step 2, the virtual offers for Load Resources deployed and the power balance penalty curve will be compared against the updated GTBD to get an estimate of the System Lambda from paragraph (2)(l) of Section 6.5.7.3.1.  ERCOT shall post the projected non-binding Base Points for each Resource for each interval study period on the MIS Certified Area and the projected non-binding LMPs for Resource Nodes, Real-Time Reliability Deployment Price Adders, Real-Time On-Line Reserve Price Adders, Real-Time Off-Line Reserve Price Adders, Hub LMPs and Load Zone LMPs on the MIS Public Area pursuant to Section 6.3.2, Activities for Real-Time Operations.

(12)	For each SCED process, ERCOT shall calculate a Real-Time On-Line Reserve Price Adder and a Real-Time Off-Line Reserve Price Adder based on the On-Line and Off-Line available reserves in the ERCOT System and the Operating Reserve Demand Curve (ORDC).  The Real-Time Off-Line available reserves shall be administratively set to zero when the SCED snapshot of the Physical Responsive Capability (PRC) is equal to or below the PRC MW at which Energy Emergency Alert (EEA) Level 1 is initiated.  In addition, for each SCED process, ERCOT shall calculate a Real-Time On-Line Reliability Deployment Price Adder.  The sum of the Real-Time Reliability Deployment Price Adder and the Real-Time On-Line Reserve Price Adder shall be averaged over the 15-minute Settlement Interval and added to the Real-Time LMPs to determine the Real-Time Settlement Point Prices.  The price after the addition of the sum of the Real-Time On-Line Reliability Deployment Price Adder and the Real-Time On-Line Reserve Price Adder to LMPs approximates the pricing outcome of the impact to energy prices from reliability deployments and the Real-Time energy and Ancillary Service co-optimization since the Real-Time On-Line Reserve Price Adder captures the value of the opportunity cost of reserves based on the defined ORDC.  An Ancillary Service imbalance Settlement shall be performed pursuant to Section 6.7.5, Real-Time Ancillary Service Imbalance Payment or Charge, to make Resources indifferent to the utilization of their capacity for energy or Ancillary Service reserves.

(13)	ERCOT shall determine the methodology for implementing the ORDC to calculate the Real-Time On-Line Reserve Price Adder and Real-Time Off-Line Reserve Price Adder.  Following review by TAC, the ERCOT Board shall review the recommendation and approve a final methodology.  Within two Business Days following approval by the ERCOT Board, ERCOT shall post the methodology on the MIS Public Area.

(14)	At the end of each season, ERCOT shall determine the ORDC for the same season in the upcoming year, based on historic data using the ERCOT Board-approved methodology for implementing the ORDC.  Annually, ERCOT shall verify that the ORDC is adequately representative of the loss of Load probability for varying levels of reserves.  Twenty days after the end of the Season, ERCOT shall post the ORDC for the same season of the upcoming year on the MIS Public Area.

(15)	ERCOT may override one or more of a Controllable Load Resource’s parameters in SCED if ERCOT determines that the Controllable Load Resource’s participation is having an adverse impact on the reliability.

6.5.9.3.1.1	Advance Action Notice

(1) 	ERCOT may issue an Advance Action Notice (AAN) in anticipation of a possible Emergency Condition.  Any AAN will identify actions ERCOT expects to take to address the possible Emergency Condition unless the need for ERCOT action is alleviated by QSE and/or TSP actions taken, or by other system developments that occur, before a time stated in the AAN.

(2)	An AAN may not require action on the part of any Market Participant but may include additional information so that Market Participants can modify their plans in such a way that mitigates the need for ERCOT to take additional actions.

(3) 	An AAN will be canceled if ERCOT determines that the possible Emergency Condition has been alleviated by QSE or TSP action, by ERCOT action, or by other system developments.

- 9.5.6	RTM Resettlement Statement

(1)	ERCOT shall issue a RTM Resettlement Statement using corrected Settlement data due to resolution of Settlement and billing disputes.  Any resettlement occurring after an RTM True-Up Statement has been issued must meet the same Interval Data Recorder (IDR) Meter Data Threshold requirements defined in Section 9.5.8, RTM True-Up Statement, and is subject to the same limitations for filing a dispute.  Despite the preceding sentence, the ERCOT Board may, in its discretion, direct ERCOT to run a resettlement of any Operating Day, at any time, to address unusual circumstances.

(2)	ERCOT shall issue a RTM Resettlement Statement for a given Operating Day due to errors other than errors in prices when:

(a)	The total of all errors other than price errors results in an absolute value impact greater than 4% of the total RTM Statement amount for any single Statement Recipient for the Operating Day; and

(b)	The impact to the Statement Recipient is greater than $400.00.

(3)	Changes to meter data managed through a process other than a dispute or Alternative Dispute Resolution will not require evaluation of a resettlement defined in paragraph (2) above.

(4)	For any Settlement and billing disputes resolved prior to issuance of the RTM Final Statement, ERCOT shall effect the dispute’s resolution on the RTM Final Statement for that Operating Day.  If a dispute is submitted by 15 Business Days after the issuance of the RTM Initial Statement for an Operating Day and is not resolved on the RTM Final Statement, ERCOT will affect the dispute’s resolution on an RTM Resettlement Statement for that Operating Day.  ERCOT shall issue such an RTM Resettlement Statement within a reasonable time after resolving the Settlement and billing dispute.

(5)	ERCOT must effect the resolution of any dispute submitted more than 15 Business Days after the issuance of the RTM Initial Statement on the next available Resettlement or RTM True-Up statement for that Operating Day.  For Settlement and billing disputes resolved under Section 9.14, Settlement and Billing Dispute Process, and submitted at least 20 Business Days before the scheduled date for issuance of the RTM True-Up Statement, ERCOT will include adjustments relating to the dispute on the RTM True-Up Statement.  Resolved disputes must be included on the next available Settlement Invoice after ERCOT has issued the RTM True-Up Statement.

(6)	ERCOT may not issue an RTM Resettlement Statement less than 20 days before a scheduled RTM Final Statement or RTM True-Up Statement for the relevant Operating Day.  An RTM Resettlement Statement will reflect differences to financial records generated on the previous Settlement Statement for the given Operating Day.

(7)	ERCOT may issue an RTM Resettlement Statement after the issuance of an RTM Final Statement in order to resolve approved disputes related to Section 5.6.5.2, RUC Make-Whole Payment and RUC Clawback Charge for Resources Receiving OSAs.