| Principle Number | 1.4 | Principle Title | Systems/Applications that Provide Input into the Real-Time Optimization Engine |
| --- | --- | --- | --- |
| Date Posted |  | July 17, 2019 |  |
| Executive Summary |  | The current ERCOT systems and applications that provide input for the Real-Time Market (RTM) optimization engine are not designed to accommodate Real-Time Co-optimization (RTC)–i.e., the awarding of Ancillary Services (AS) in Real-Time. This RTC principle addresses the necessary modifications to those systems and applications. |  |
| Principle Description |  | ERCOT’s systems and applications that provide input for the RTM optimization engine will be modified to account for RTC (e.g., the Resource Limit Calculator (RLC)). Real-Time information provided by Qualified Scheduling Entities (QSEs) to ERCOT, and ERCOT to QSEs, will need to be reviewed and modified to accommodate such changes. |  |
| RTCTF Discussion |  | On 6/7/19, the RTCTF reached consensus on the first part of KP1.4. On 7/12/19, the RTCTF added “Telemetered” into the equations in (1) for clarity on Ramp Rates. |  |
| TAC Action Requested |  | TAC vote to endorse Principle 1.4, subsection (1) for purposes of informing the Board. |  |
| TAC Action Summary |  |  |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for Voting

- Under RTC, for Resources that are being economically dispatched by ERCOT, the current practice of pre-reserving portions of their capacity to provide Ancillary Services (AS) through telemetry from the Resource’s QSE will be discontinued.  AS awards will now be an output of RTC.

  - Because of this, RLC calculated Resource Limits of High Ancillary Service Limits (HASLs) and Low Ancillary Service Limits (LASLs) will be discontinued.

  - With the HASL and LASL calculations being discontinued, the method for calculating dispatch limits needs to be modified.  High Sustained Limits (HSLs), Low Sustained Limits (LSLs), and telemetered ramp rates will be used directly to calculate High Dispatch Limits (HDLs) and Low Dispatch Limits (LDLs), per the formulas:

HDL=Min(HSL,TelemMW+NormalTelemeteredRampRateUp*5)

LDL=Max(LSL,TelemMW-NormalTelemeteredRampRateDn*5)

# Previously Approved Principle Concepts

None

# Future Decision Points and Issues for Developing Principle Concepts

Telemetry changes associated with any change to the RLC logic, both from the QSEs to ERCOT and from ERCOT to the QSEs.

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |