| Principle Number | 1.5 | Principle Title | Process for Deploying Ancillary Services |
| --- | --- | --- | --- |
| Date Posted |  | July 17, 2019 |  |
| Executive Summary |  | To implement Real-Time Co-optimization (RTC), certain processes for deploying Ancillary Services (AS) will need to be modified to accommodate the awarding of AS in Real-Time. This RTC principle looks at the ERCOT systems, ERCOT to Qualified Scheduling Entity (QSE) communications, QSE to ERCOT communications, and other processes in place that play a role in the sending of Dispatch instructions and deployment of AS. |  |
| Principle Description |  | To implement RTC, certain processes for deploying Ancillary Services (AS) will need to be modified to accommodate AS awards in Real-Time. |  |
| RTCTF Discussion |  | On 6/7/19, the RTCTF reached consensus on a number of the principle concepts under KP1.5. However, alternatives were proposed for concepts 3 and 6. In both cases, the alternatives relate to the issue of whether Regulation Service instructions from ERCOT should be Resource-specific or maintain the current practice of instructing QSEs at the portfolio level. On 6/21/19 Shams presented modified alternative language for (3) and (6). On 7/12/19 Shams presented modified alternative language for (3) and removed alternative for (6). Both ERCOT and Shams presented a brief position for their alternatives, but without resolution at RTCTF. Alternatives for (3) will be going to 7/24/19 TAC, with consensus on (1)-(2), and (4)-(6). |  |
| TAC Action Requested |  | TAC vote to endorse Principle 1.5, subsections (1)-(6), with specific direction on which alternative language for (3) for purposes of informing the Board. |  |
| TAC Action Summary |  |  |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for Voting

- The AS manager application will be modified to obtain Resource-specific AS responsibility/award information as an output from RTC (i.e., it will no longer be driven by Real-Time telemetry and Current Operating Plan (COP) information provided by Qualified Scheduling Entities (QSEs)).

- Under the current process, energy for immediate dispatch and Locational Marginal Prices (LMPs) from Security-Constrained Economic Dispatch (SCED) are binding; this process will remain in place with RTC.  Thus, RTC awards for AS products, energy, dispatch, and their respective prices (e.g., LMPs, Market Clearing Prices for Capacity (MCPCs)) will be immediately binding as applicable to various AS products (e.g., Regulation Up Service (Reg-Up) and Regulation Down Service (Reg-Down)).

- Two alternative concepts have been proposed:

Alternative 1 (ERCOT): Regulation Service instructions from ERCOT will become Resource specific (i.e., no longer QSE portfolio level; participation factors will be removed).

Alternative 2 (Siddiqi): Regulation Service instructions from ERCOT will be Resource specific. However, QSEs will have the option of telemetering participation factors (PFs). After each RTC run, until the time ERCOT accepts new telemetered PFs, Resources providing Reg-Up/Reg-Down will be expected to follow ERCOT Resource specific Reg-Up/Reg-Down deployments. Once ERCOT accepts new telemetered PFs for QSEs taking advantage of this option, Resources will be expected to follow PF-adjusted Resource specific Reg-Up/Reg-Down deployments until the next RTC run.  Note: Regulation deployments from ERCOT are proportional and not economic. QSEs can optimize the deployment within their portfolios for economics and to manage storage levels between various storage resources or between CLR and GR sides of CAES projects. Maintaining the Participation Factors does not in any way change the RTC SCED award of Regulation or any of the considerations that go into making that award. Nor does it impact LFC. It only impacts real-time Resource monitoring, GREDP, and Base Point Deviation calculations. Keeping the Participation Factors actually increases the efficiency of Regulation deployment.

- Load Frequency Control (LFC) will be modified to address more frequent awards of Regulation Service to qualified Resources; upon the receipt of new Base Points and AS awards from RTC, LFC will reset Regulation Service instructions to zero.

- Updated Desired Base Points (UDBP) will be replaced by Updated Desired Set Point (UDSP)—UDSP will be a single value that is the sum of two components: Base Ramp and Resource-specific Regulation Service instruction.  Base Ramp will be a four minute ramp similar to UDBP, except that the starting point of the Base Ramp will be the expected output of the Resource using the previous Base Point and the last Resource-specific Regulation instruction from LFC before new Base Points were input to LFC (i.e., the expected output based on these two components).  For Resources that are not providing Regulation Service, the Regulation instruction component will be zero.  LFC will then determine the Resource-specific instruction and add it to the Base Ramp.

LFC will send UDSP every four seconds for all Resources receiving a Base Point from RTC and will continue to do so as new RTC results become available.  The UDSP ramp may be temporarily halted for Resources that have Base Points directionally opposite a significant frequency deviation.

- The calculated system level regulation requirement will be distributed as Regulation Service Instructions to each Resource proportionate to their Regulation Service awards. Issued Resource-specific Regulation Service instructions will respect Resource limits (i.e., HSL, LSL, and ramp rates) by considering UDSP and Resource-specific Regulation Service instructions. Remaining un-deployed system level regulation will be distributed to Resources with Regulation Service awards that have un-deployed Regulation Service award capacity.

# Previously Approved Principle Concepts

None

# Future Decision Points and Issues for Developing Principle Concepts

Functionality and Process Concepts

- LFC – Process of deploying RRS

- Telemetry changes necessary to accommodate LFC and AS manager changes

- Impact of AS Demand Curves (ASDCs)

- Removal of the existing process for QSEs to update telemetered AS schedules following manual deployment for Generation Resources and Controllable Load Resources

- Elimination of Non-Spinning Reserve (Non-Spin) offer floor

- Continued ability of RTC to be executed off-cycle, between regularly scheduled five-minute executions.

- RTC failure process

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |