| Principle Number | 1.3 | Principle Title | Offering and Awarding Ancillary Services in Real-Time |
| --- | --- | --- | --- |
| Date Posted |  | August 13, 2019 |  |
| Executive Summary |  | Real-Time Co-Optimization (RTC) will allow Qualified Scheduling Entities (QSEs) to offer and be awarded Ancillary Services (AS) in Real-Time. |  |
| Principle Description |  | The purpose of Key Principle 1.3, Offering and Awarding Ancillary Services in Real-Time is to outline the key mechanisms and timelines for submitted AS Offers, as well as those considered and awarded under RTC. Specifically, this principle addresses the following concepts: (a) AS Offer structure and timing; (b) QSE telemetry for updating available capacity from AS Offers; (c) Under-frequency relay (UFR) Load Resource scheduling for Responsive Reserve (RRS) UFR and ERCOT Contingency Reserve Service (ECRS); (d) On-Line hydro Generation Resource (hydro Generation Resources not operating in ONRR mode) scheduling for RRS, Non-Spinning Reserve (Non-Spin), and ECRS; and (e) Constraints to be used for the RTC clearing process. |  |
| RTCTF Discussion |  | On 6/21/19, ERCOT provided a presentation and whitepaper discussing constraint formulation for RTC. There was specific discussion on subsections (1), (2), and (4). On 7/12/19, RTCTF discussed comments that ERCOT received, and ERCOT provided a presentation looking at telemetered ramp rates and constraints for Combined Cycle Generation Resources (CCGRs). On 8/9/19, South Texas Electric Cooperative Inc. (STEC) presented proposed changes to Key Principle 1.3, with a focus on the treatment of hydro Resources (subsection (3)). ERCOT also presented on a number of topics related to Key Principle 1.3, including discussion on subsections (5) through (9). |  |
| TAC Action Requested |  |  |  |
| TAC Action Summary |  |  |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC Endorsement

None

# Principle concepts Previously Endorsed by TAC

None

# Principle Concepts in DisCussion at RTCTF

- QSEs will have ability in Real-Time to indicate whether a Resource is temporarily unable to provide AS due to operational constraints.

- UFR Load Resources will be able to self-provide RRS UFR and ECRS; the amount of which will based on Day-Ahead Market (DAM) and AS trades.

- On-Line hydro Generation Resources not operating in ONRR mode will be able to self-provide RRS, Non-Spin, and ECRS; the amount of which will based on DAM self-arrangement, DAM AS awards and AS trades.

- RTC will account for frequency responsive capacity of a Combined Cycle Generation Resource (CCGR) when awarding Regulation and RRS Primary Frequency Response (PFR).

- RTC will not change limitations on sub-categories of AS products (e.g., FRRS, FFR, and RRS and ECRS provided via UFR).

- Off-Line Resources providing Non-Spin that are in startup due to a manual deployment of Non-Spin by ERCOT will continue to be eligible for being awarded Non-Spin for the first 25 minutes following the deployment.  The eligible capacity will be based on the High Sustained Limit (HSL) of the Resource less its Base Point instruction.

- Resources operating in quick-start mode that are in startup due to a deployment from ERCOT will continue to be eligible for being awarded ECRS and Non-Spin.  The eligible capacity will be based on the HSL of the Resource less its base point instruction.

- For Load Resources providing AS via a UFR that are deployed for the AS through a manual instruction from ERCOT:

  - During the period of deployment, RTC will continue to award AS to the Load Resource with the awards based on the ERCOT instruction amount.

  - During recall of the deployment, AS awards will not be based on the deployment amount.  Instead, AS awards will be based on self-provision amounts and limited by the physical capability of the Resource to provide AS.

- Within RTC, ERCOT operators will have the ability to prevent RTC from awarding AS to Resources that, when deployed, may violate transmission constraints.

# Future Decision Points and Issues for Developing Principle Concepts

- Additional concepts related to constraint formulaton (e.g., treatement of Resources with an ONRR status)

- Offer structure for AS Offer in Real-Time

- Timeline for submission of AS Offers for Real-Time

- Potential changes to Verifiable Costs

- Creation of proxy AS Offers

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |

# Appendix Material with additional details

- QSEs will have the ability in Real-Time to indicate whether a Resource is temporarily unable to provide AS due to operational constraints.

Details under development:

- For scenarios where the reduced AS capability is less than the QSE-submitted AS Offer (MW), QSEs can, via Resource-specific telemetry, inform ERCOT of the reduced AS capability (MW) by specific AS type from a Resource.

- Under RTC, ERCOT systems will use Resource-specific telemetered values to limit AS awards.

- ERCOT will be proposing to add new Resource-specific telemetry points (analog – MW) for each AS type/subtype:

  - Regulation Up (Reg-Up) capability;

  - Regulation Down (Reg-Down) capability;

  - Fast Responding Regulation Up Service (FFRS-Up) capability;

  - Fast Responding Regulation Down Service (FFRS-Down) capability;

  - RRS PFR capability;

  - RRS UFR capability;

  - RRS FFR capability;

  - ECRS capability; and

  - Non-Spinning Reserve (Non-Spin) capability.

- Future discussion will also address whether a QSE with a submitted telemetered Ramp Rate up for a Resource with a High Dispatch Limit (HDL) that is greater than the Resource’s telemetered MW, should be able to indicate that the Resource is unable to provide Reg-Up, ECRS and/or Non-Spin.

- UFR Load Resources will be able to self-provide RRS UFR and ECRS; the amount of which will based on DAM and AS trades.

- On-Line hydro Generation Resources not operating in ONRR mode will be able to self-provide RRS, Non-Spin, and ECRS; the amount of which will based on DAM self-arrangement, DAM AS awards and AS trades.

Details under development:

- Under RTC, UFR Load Resources will not be capable of arming and disarming UFR relays based on non-zero/zero AS awards if treated like Generation Resources

  - New Resource-specific telemetry points (analog – MW) will be added for RRS UFR, RRS FFR and ECRS, allowing ERCOT visibility into MW service quantities self-provided by a UFR Load Resource.

    - System-wide RRS and ECRS AS Demand Curves (ASDCs) will be reduced by the validated amount of UFR Load Resource self-provided service quantities.

    - ERCOT is in the process of developing validation processes and rules regarding these new telemetry values.

      - The 60% limitation rule (per NPRR 863) for system-wide RRS from UFR Load Resources and FFR and the annually determined limit from FFR will be maintained.

      - Telemetered UFR Load Resource self-provided RRS, ECRS MW amounts with respect to DAM awards and AS Trades under normal conditions and scarcity conditions will be reconciled.

- Under RTC, On-Line hydro Generation Resources not operating in ONRR mode must adhere to constraints imposed by governmental agencies or by treaty regarding the operations of those Resources.  These Resources often have the opportunity to provide incremental energy in response to a deployment of AS and are therefore good candidates to provide AS, but cannot regularly exchange the AS capacity for an incremental energy deployment.

  - New Resource-specific telemetry points (analog – MW) will be added for RRS, Non-Spin, and ECRS, allowing ERCOT visibility into MW service quantities self-provided by a hydro Generation Resource.

  - System-wide RRS, Non-Spin, and ECRS AS Demand Curves (ASDCs) will be reduced by the validated amount of online hydro Generation Resource self-provided service quantities.

- RTC will account for will account for frequency responsive capacity of a CCGR when awarding Regulation and RRS PFR.

Details under development:

- An On-Line CCGR can have portions of its capacity that are not frequency responsive (e.g., steam generator, duct burner, etc.).  Therefore, under RTC, Real-Time results should not be such that awarded Regulation and/or RR PFR (or portions thereof) could be coming from CCGR capacity that is Non-Frequency Responsive Capacity (NFRC).

- ERCOT will be proposing an enhanced mathematical model of CCGR, and new telemetry to account for frequency responsive capacity of a CCGR for awarding Regulation and RRS PFR.