| Principle Number | 1.5 | Principle Title | Process for Deploying Ancillary Services |
| --- | --- | --- | --- |
| Date Posted |  | September 3, 2019 |  |
| Executive Summary |  | To implement Real-Time Co-optimization (RTC), certain processes for deploying Ancillary Services (AS) will need to be modified to accommodate the awarding of AS in Real-Time. This RTC principle looks at the ERCOT systems, ERCOT to Qualified Scheduling Entity (QSE) communications, QSE to ERCOT communications, and other processes in place that play a role in the sending of Dispatch instructions and deployment of AS. |  |
| Principle Description |  | To implement RTC, certain processes for deploying Ancillary Services (AS) will need to be modified to accommodate AS awards in Real-Time. |  |
| RTCTF Discussion |  | On 6/7/19, the RTCTF reached consensus on a number of the principle concepts under KP1.5. However, alternatives were proposed for concepts 3 and 6. In both cases, the alternatives relate to the issue of whether Regulation Service instructions from ERCOT should be Resource-specific or maintain the current practice of instructing QSEs at the portfolio level. On 6/21/19, Shams Siddiqi presented modified alternative language for (3) and (6). On 7/12/19, Shams Siddiqi presented modified alternative language for (3) and removed alternative for (6). Both ERCOT and Shams presented a brief position for their alternatives, but without resolution at RTCTF. Alternatives for (3) will be going to 7/24/19 TAC, with consensus on (1)-(2), and (4)-(6). On 8/9/19, RTCTF made a minor edit to subsection (8) and reached concensus on subsections (7) through (13). |  |
| TAC Action Requested |  | On 7/24/19, TAC vote to endorse KP1.5, subsections (1)-(6), with specific direction on which alternative language for (3) for purposes of informing the Board. On 8/28/19, TAC vote to endorse KP1.5, subsections (7)-(13) for purposes of informing the Board. |  |
| TAC Action Summary |  | On 7/24/19, TAC voted to endorse KP1.5, subsections (1)-(6), including alternative 1 for (3), for purposes of informing the Board. On 8/28/19, TAC voted to endorse KP1.5, subsections (7)-(13) for purposes of informing the Board. |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC Endorsement

- Operational procedures for deploying Offline Non-spin and RRS from Load Resources remain the same

- For manual deployment of Generation Resources carrying RRS with the Resource on Synchronous Condenser Fast Response Mode or carrying RRS as FFR capable Resources excluding NCLR, LFC will send energy deployment instructions.

- The existing process for QSEs to update telemetered AS schedules following manual deployment for Generation Resources and Controllable Load Resources will be removed under RTC.

- Under scarcity conditions, energy to be served is given priority and smaller amounts of each Ancillary Service will be procured. This will result in scarcity prices being set by the demand curves and reflected in energy prices and MCPCs.

- The administrative price floor for Non-Spin will be replaced by prices determined from awarded offers and the ASDC for Non-Spin.

- RTC will continue to have the ability for the Real-Time co-optimization (i.e., RTC) to be executed off-cycle, manually or automatically, between regularly scheduled 5-minute executions.

- The processes and procedures during a SCED/RTC failure will remain the same: emergency base points and held prices (SPPs, Meter prices and MCPCs) through the 15 minute recovery period

# Principle concepts Previously endorsed by TAC

- The AS manager application will be modified to obtain Resource-specific AS responsibility/award information as an output from RTC (i.e., it will no longer be driven by Real-Time telemetry and Current Operating Plan (COP) information provided by Qualified Scheduling Entities (QSEs)).

- Under the current process, energy for immediate dispatch and Locational Marginal Prices (LMPs) from Security-Constrained Economic Dispatch (SCED) are binding; this process will remain in place with RTC.  Thus, RTC awards for AS products, energy, dispatch, and their respective prices (e.g., LMPs, Market Clearing Prices for Capacity (MCPCs)) will be immediately binding as applicable to various AS products (e.g., Regulation Up Service (Reg-Up) and Regulation Down Service (Reg-Down)).

- Regulations Service instructions from ERCOT will become Resource specific (i.e., no longer QSE portfolio level; participation factors will be removed).

- Load Frequency Control (LFC) will be modified to address more frequent awards of Regulation Service to qualified Resources; upon the receipt of new Base Points and AS awards from RTC, LFC will reset Regulation Service instructions to zero.

- Updated Desired Base Points (UDBP) will be replaced by Updated Desired Set Point (UDSP)—UDSP will be a single value that is the sum of two components: Base Ramp and Resource-specific Regulation Service instruction.  Base Ramp will be a four minute ramp similar to UDBP, except that the starting point of the Base Ramp will be the expected output of the Resource using the previous Base Point and the last Resource-specific Regulation instruction from LFC before new Base Points were input to LFC (i.e., the expected output based on these two components).  For Resources that are not providing Regulation Service, the Regulation instruction component will be zero.  LFC will then determine the Resource-specific instruction and add it to the Base Ramp.

LFC will send UDSP every four seconds for all Resources receiving a Base Point from RTC and will continue to do so as new RTC results become available.  The UDSP ramp may be temporarily halted for Resources that have Base Points directionally opposite a significant frequency deviation.

- The calculated system level regulation requirement will be distributed as Regulation Service Instructions to each Resource proportionate to their Regulation Service awards. Issued Resource-specific Regulation Service instructions will respect Resource limits (i.e., HSL, LSL, and ramp rates) by considering UDSP and Resource-specific Regulation Service instructions. Remaining un-deployed system level regulation will be distributed to Resources with Regulation Service awards that have un-deployed Regulation Service award capacity.

# Principle Concepts IN DISCUSSION AT RTCTF

None

# Future Decision Points and Issues for Developing Principle Concepts

Functionality and Process Concepts

- Emergency Operations Settlement will pay the additional energy deployed during emergency base points operations, if necessary. Ancillary Service awards and prices will be held during the failure period and will not be adjusted.

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |