| Key Topic Concept (KTC) Number | 6 | KTC Title | ESR Options to Maintain Desired Level of State of Charge |
| --- | --- | --- | --- |
| Date Posted |  | November 5, 2019 |  |
| Executive Summary |  | This KTC recommends an improved method for Energy Storage Resources (ESRs) to communicate to ERCOT their willingness or unwillingness to be dispatched by SCED. This KTC also provides information about the offer structure for a single model ESR. |  |
| Recommendation Description |  | This KTC recommends that updates to Energy Offer Curves (EOCs) by Limited Duration Resources (LDRs) may be submitted immediately prior to the start of the Operating Hour. As a second phase consideration, LDRs could be allowed to submit EOCs during the Operating Hour. |  |
| BESTF Discussion |  | On 10/18/19, ERCOT staff presented a refresher on previous stakeholder discussion around the ability of Limited Duration Resources (LDRs) to have their Energy Offer Curves (EOCs) closer to Real-Time. Task force participants largely seemed in favor of a phased approach. On 11/4/19, the BESTF reached consensus on KTC 6 item 1. ERCOT staff presented information about the offer structure of a single model ESR (item 2 of this KTC document). |  |
| TAC Action Requested |  | BESTF plans to request at the 11/20/19 TAC meeting a vote to approve KTC 6 item 1. |  |
| TAC Action Summary |  |  |  |

Proposed KTC Recommendation Language

# Key Topic/Concept recommendation Language for TAC Approval

11/20/19 TAC Meeting (Consensus from 11/4/19 BESTF meeting):

- ERCOT systems should be updated to capture whether or not a Resource is a LDR.  This Resource attribute will be captured in ERCOT’s registration systems and passed to other downstream systems (e.g., the Energy Management System (EMS) and Market Management System (MMS)).

- As an initial phase, the EOC submission window for LDRs should be expanded such that the window ends just prior to the beginning of the Operating Hour.  ERCOT’s MMS should be updated to recognize that a Resource is an LDR and have validation to ensure the submission window is only expanded for LDRs.

- As a second phase consideration, LDRs could be allowed to submit EOCs during the Operating Hour.  Intra-Operating Hour EOCs would be a separate submittal type and would only allow updates with the MW quantity and price pairs.  Again, ERCOT’s MMS should be updated to recognize that a Resource is an LDR and restrict the new submittal type to only LDRs.

# Key Topic/Concept recommendation Language Previously APProved by tac

None.

# Key Topic/Concept recommendation Language IN DISCUSSION AT BESTF

1.  (Item 2) Offer Structure for the single device ESR model.

# Future Decision Points and Issues for Developing Key topic/Concept recommendation Language

None.

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |