| Principle Number | 1.2 | Principle Title | System-Wide Offer Cap and Power Balance Penalty Price |
| --- | --- | --- | --- |
| Date Posted |  | November 4, 2019 |  |
| Executive Summary |  | The values of and interaction between System-Wide Offer Cap (SWOC), Value-of-Lost-Load (VOLL), and Power Balance Penalty Price (PBPP) must be evaluated as part of implementation of Real-Time Co-optimization (RTC). This key principle identifies which values are changing and which one are remaining the same. |  |
| Principle Description |  | The purpose of Key Principle 1.2, System-Wide Offer Cap and Power Balance Penalty Curve, is to identify the parameters to be used both before and after meeting the Peaker Net Margin (PNM) threshold with the Resource Adequacy cycle: - SWOC - VOLL - PBPP, currently represented as the Power Balance Penalty Curve (PBPC) Rules are also identified for the capping of prices under scarcity conditions and this key principle considers the interaction of SWOC between the Day-Ahead Market (DAM) and Real-Time Market (RTM). |  |
| RTCTF Discussion |  | On 8/9/19, ERCOT staff presented material introducing KP1.2 subsections (1) and (2). On 8/27/19, the RTCTF reached consensus on KP1.2 subsections (1) and (2). Also, ERCOT staff presented material introducing KP1.2 subsection (3). On 9/19/19, the RTCTF reviewed subsection (3) and discussed the ERCOT, Luminant, and Siddiqi options. RTCTF narrowed the options to Luminant proposal or Siddiqi proposal for consideration at the next RTCTF. On 10/9/19, the RTCTF reviewed subsection (3) and reached consensus that there is not a need to adjust Real-Time Market Clearing Prices for Capacity (MCPCs) when there is an ex post capping of energy prices as proposed in (3) and therefore removed (3) from further consideration (relocated to KP8 at the 10/30/19 RTCTF meeting based on TAC discussions). On 10/30/19, ERCOT Staff introduced concepts for the potential need for separate SWOC in DAM. |  |
| TAC Action Requested |  | On 9/25/19, TAC vote to endorse KP1.2 subsections (1) and (2) for purposes of informing the Board. |  |
| TAC Action Summary |  | On 9/25/19, TAC voted to endorse KP1.2 subsections (1) and (2) for purposes of informing the Board. |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC Endorsement

None

# Principle concepts Previously Endorsed by TAC

1)	For the period of the annual Resource Adequacy cycle where the PNM threshold has not been met, the following parameters will be in effect for the RTM:

a.	SWOC will be equal to $2,000/MWh;

b.	VOLL and the maximum Ancillary Service Demand Curve (ASDC) value will be equal to $9,000/MWh;

c.	The energy price, exclusive of congestion costs, produced every execution of the RTM will be capped at $9,000/MWh; and

d.	PBPP will be equal to $11,000.01/MWh.

2)	After the PNM threshold has been met, a process will be used to adjust the parameters for the RTM to be:

a.	SWOC will be equal to $2,000/MWh;

b.	VOLL and the maximum ASDC value will be equal to $2,000/MWh;

c.	The energy price, exclusive of congestion costs, produced every execution of the RTM will be capped at $2,000/MWh; and

d.	PBPP will be equal to $4,000.01/MWh.

# Principle Concepts in DisCussion at RTCTF

3)   For the Day-Ahead Market (DAM), the SWOC will be equal to $9,000/MWh.

# Future Decision Points and Issues for Developing Principle Concepts

None

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |