| Principle Number | 3 | Principle Title | Reliability Unit Commitment |
| --- | --- | --- | --- |
| Date Posted |  | November 4, 2019 |  |
| Executive Summary |  | The Reliability Unit Commitment (RUC) process is designed to study forecasted conditions on the grid and determine whether there is sufficient Resource capability planned to be available to reliably meet those conditions in Real-Time. If there is not sufficient Resource capability planned to be available, RUC may recommend the commitment of additional Resources. With the implementation Real-Time Co-optimization (RTC), this objective remains the same. However, to better project Real-Time conditions on the grid and determine the potential need for Resource commitments, RUC will be modified to be consistent with how energy and Ancillary Services (AS) will be awarded in Real-Time. In the same way that there will be flexibility in Real-Time to move AS among qualified Resources, the RUC optimization engine will also look at moving AS among qualified Resources to meet the forecasted conditions on the grid to align with Real-Time and deliver more effective reliability solutions. The RUC Settlement process will be adjusted to consider Real-Time AS Revenues in the Make-Whole and clawback Settlements. The RUC Capacity-Short Charge will be adjusted to consider AS Supply Responsibility amounts not covered with qualified AS capability. |  |
| Principle Description |  | To better project Real-Time conditions on the grid and determine the potential need for Resource commitments, RUC will be modified to be consistent with how energy and AS will be awarded in the Real-Time Market (RTM). To facilitate this change under RTC, RUC will review Resources scheduled to be available to determine whether additional Resource commitments are needed to meet the Load forecast and minimum AS requirements, and resolve transmission congestion per defined penalty curves and factors. RUC Settlements will be adjusted to consider Real-Time AS award revenue and to accommodate changes in the RUC engine. |  |
| RTCTF Discussion |  | On 6/7/19, the RTCTF considered principle concepts 1 through 9. There was discussion on a number of the principles and alternatives were proposed for concepts 2 and 6. RTCTF discussed all the principle concepts coming back to RTCTF for review at the next meeting. On 6/21/19, ERCOT gave an overview on the concepts for subsections (10), (11), and (12). On 7/12/19, Shams Siddiqi removed Alternative for (6). Shams Siddiqi and ERCOT made presentations on (2). After further discussion ERCOT agreed to provide RUC AS Demand Curve capability, and (2) was updated to reflect the agreement. No additional feedback on subsections (10) through (12). On 8/9/19, RTCTF agreed that there was consencus on subsections (10) through (12). On 10/9/19, ERCOT gave an overview on the concepts for subsections (13) through (18). On 10/30/19, RTCTF reviewed the concepts in (13) through (18) as part of the KP1.1 RUC / Reliability Deployment pricing discussion. |  |
| TAC Action Requested |  | On 7/24/19, TAC vote to endorse KP3, subsections (1) through (9) for purposes of informing the Board. On 8/28/19, TAC vote to endorse KP3, subsections (10) through (12) for purposes of informing the Board. |  |
| TAC Action Summary |  | On 7/24/19, TAC voted to endorse KP3, subsections (1) through (9) for purposes of informing the Board. On 8/28/19, TAC voted to endorse KP3, subsections (10) through (12) for purposes of informing the Board. |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC Endorsement

None

# principle concepts Previously endorsed by tac

1)	RUC will continue to ensure adequate capacity for Real-Time to meet energy and AS needs, and resolve transmission constraints; since it is designed to distribute AS across all available Resources, it has the additional flexibility for resolving transmission constraints as well as AS needs and should result in fewer RUC commitments for congestion.

2)	RUC will be built with the capability to use RUC Ancillary Service Demand Curves (ASDCs). RUC will attempt to solve for a Resource commitment that meets the Load forecast and AS Plan considering Resources’ COPs and using defined penalty curves.  The values of these curves will be determined at a future date.

3)	Modifications will be made to the existing set of data elements provided by Qualified Scheduling Entities (QSEs) in their COPs to accommodate changes to RUC optimization.

4)	QSEs will have a mechanism in their COPs to indicate, for each hour, the physical ability/inability of a Resource to provide AS (i.e., the Resource Status).

5)	The amount of AS that can be provided by a Resource will be constrained by its qualifications and capabilities.

6)	Proxy AS Offers will be used in RUC in determining a co-optimized solution where AS Offers have not been submitted.

7)	In addition to online qualified Resources, the RUC engine will consider a COP Resource Status of OFFQS (Off-Line but available for SCED deployment) for a Resource that is qualified for ERCOT Contingency Reserve Service (ECRS), as being able to provide ECRS.

8)	In addition to online qualified Resources, the RUC engine will consider a COP Resource Status of OFF (Off-Line but available for commitment in the DAM and RUC) for a Resource that is qualified for Non-Spinning Reserve (Non-Spin), as being able to provide Non-Spin.

9)	The current process under which ERCOT Operators review recommendations from the RUC optimization and make commitment instruction decisions will remain in place.  This process includes:

a.	ERCOT Operators will give Market Participants ample time to respond to postings of capacity shortages for future hours; and

b.	If a generation commitment is recommended by RUC for a future hour, ERCOT Operators will delay any Dispatch Instruction until the next RUC process would not have sufficient lead time to commit the Resource for the same future hour thus giving QSEs a chance to self-commit.

10)	Revenues from Real-Time AS awards will be used to offset the RUC Guarantee for the RUC Make-Whole Payment.

11)	Revenues from Real-Time AS awards will be included as revenues in the RUC Clawback Charge.

12)	The Capacity-Short Charge will be adjusted to allocate RUC Make-Whole Costs to QSEs that are short in either energy capacity or AS capacity.

a.	QSEs with AS Supply Responsibility greater than their AS capability will be allocated a portion of RUC Make-Whole costs.

b.	QSEs with an overall shortage in energy plus AS Supply Responsibility will be allocated a portion of the RUC Make-Whole costs.

c.	AS capability for Capacity Short Charge Settlement purposes will be based on AS Offers validated against COP information.

# principle concepts in discussion at rtctf

13)	The RUC engine will use the same proxy methodology for AS as Real-Time

14)	The RUC engine will use the same scaling for AS offers as energy offers

15)	RUC will use new information contained in the COP to determine how much capability for each AS product each Resource will be capable of providing

16)	The Energy Offer Floor for RUC-instructed Resources which have not opted out of RUC settlement will be $1,500/MWh for both the dispatching and pricing run

17)	AS Offer Floors for RUC-instructed Resources which have not opted out of RUC Settlement will be the same for both the dispatch and pricing runs.

18)	RTC will not affect the ability for QSEs to opt out of RUC Settlement.

# Future Decision Points and Issues for Developing Principle Concepts

None

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |