| Principle Number | 1.6 | Principle Title | Ancillary Service Imbalance Settlement |
| --- | --- | --- | --- |
| Date Posted |  | November 25, 2019 |  |
| Executive Summary |  | The current Real-Time Ancillary Service (AS) imbalance Settlement process is driven by Operating Reserve Demand Curve (ORDC) prices and MW amounts; it does not consider that under Real-Time Co-Optimization (RTC), AS will be awarded in Real-Time. Because the ORDC process will be removed, and AS will be awarded in Real-Time, the AS imbalance Settlement process will be modified. |  |
| Principle Description |  | The Real-Time AS imbalance Settlement process for the ORDC will be replaced with a new process for RTC. |  |
| RTCTF Discussion |  | On 6/7/19, the RTCTF reached consensus on principle concepts 1 through 4 for KP1.6. Subsequent discussion at 6/21/19 and 7/12/19 but no issues identified and no changes to language. On 11/19/19, ERCOT Staff presented material to introduce KP1.6 subsection (5), and noted the Credit Work Group will review at its November 22, 2019 meeting. |  |
| TAC Action Requested |  | TAC vote to endorse Principle 1.6, subsections (1)-(4) for purposes of informing the Board. |  |
| TAC Action Summary |  | On 7/24/19, TAC endorsed subsections (1)-(4). |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC ENDORSEMENT

None

# principle concepts Previously endorsed by tac

1)	The new AS imbalance Settlement process will not create new sources of uplift or Make-Whole Charges/Payments for Load Serving Entities (LSEs).

2)	AS imbalance calculations will be created for each AS product in the Real-Time Market (RTM).

3)	15-minute Settlement of non-zero net AS imbalance amounts will be charged/paid to Qualified Scheduling Entities (QSEs) on a Load Ratio Share (LRS) basis.  Where the amount cleared in DAM plus corresponding AS trades is equal to the corresponding RTM award MW amount, there will be no LRS-based charge/payment.

4)	Under RTC, a QSE’s RTM AS Obligation shall equal the RTM LRS multiplied by sum of all RTM AS awards (i.e., there will be no portfolio self-arrangement for the RTM).

# Principle Concepts IN DISCUSSION AT RTCTF

5)  Under RTC, the following revision will be made credit exposure calculations to account for the RTC AS activity:

a.	Updates will be made to the Real-Time Liability Completed and Not Settled (RTLCNS) component of Estimated Aggregate Liability (EAL) calculation to include RTC AS activity.

b.	Updates will be made to the Real-Time Liability Estimates (RTL) component of the EAL to include RTC AS activity (these components will flow into RTLCNS).

c.	Updates will be made to the Minimum Current Exposure (MCE) component of Total Potential Exposure (TPE) calculation to include RTC AS activity.

# Future Decision Points and Issues for Developing Principle Concepts

None

| Applicable Protocol Section(s) | 6.7, Real-Time Settlement Calculations for the Ancillary Services |
| --- | --- |
| Impacted System(s) / Application(s) |  |