| Principle Number | 7 | Principle Title | Performance Monitoring |
| --- | --- | --- | --- |
| Date Posted |  | November 25, 2019 |  |
| Executive Summary |  | This Key Principle identifies changes to performance monitoring that will be necessary to reflect the implementation of Real-Time Co-optimization (RTC). |  |
| Principle Description |  | The purpose of Key Principle 7, Performance Monitoring, is to identify any necessary changes to performance monitoring with the implementation of RTC: 1. Changes to Generation/Controllable Load Resource Energy Deployment Performance (GREDP/CLREDP) formulation. 2. Performance metrics related to a release of a Resource's High Ancillary Service Limit (HASL) through a change in AS Schedule. 3. Removal of the concept of AS Responsibility in Real-Time and associated Settlements. |  |
| RTCTF Discussion |  | On 10/30/19, ERCOT Staff presented language to introduce KP7 subsection (1). On 11/19/19, ERCOT Staff presented language to introduce KP7 subsections (2) and (3). |  |
| TAC Action Requested |  |  |  |
| TAC Action Summary |  |  |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC Endorsement

None

# Principle concepts Previously Endorsed by TAC

None

# Principle Concepts in DisCussion at RTCTF

1)	Generation Resource Energy Deployment Performance (GREDP) and Controllable Load Resource Energy Deployment Performance (CLREDP) calculations will updated to account changes made to deployments instructions from ERCOT.  The “ABP” and “ARI” components of the GREDP and CLREDP calculations will be replaced with an Average Set Point (ASP) component, where ASP is equal to the time-weighted average of the Updated Desired Set Point (UDSP), which is sum of a linearly ramped Base Point (Base Ramp) and Regulation Service instruction that the Resource should have produced during a five-minute clock interval.

2)	Currently, ERCOT measures a QSE’s total telemetered Ancillary Service (AS) Responsibilities and total AS Responsibilities submitted in Current Operating Plans (COPs) against its total AS Obligation in order to identify any non-compliance with the Qualified Scheduling Entity (QSE) meeting its obligation.  With RTC, the concept of an AS Obligation based on Day-Ahead Market (DAM) awards, AS self-arrangement, and AS trades is no longer relevant.  As such, related compliance requirements and associated reporting will be removed.

3)	With RTC, the updating on AS Schedules following a deployment instruction for the purpose of adjusting High Ancillary Service Limits (HASLs) is no longer relevant.  As such, the compliance requirement and associated reporting will be removed.

# Future Decision Points and Issues for Developing Principle Concepts

1)	Changes to base point deviation charge calculations that align with changes to GREDP.

2)	Performance monitoring during the transition into RTC during the time period immediately following implementation.

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |