| Key Topic Concept (KTC) Number | 2 | KTC Title | Physical Responsive Capability, and ORDC Reserve |
| --- | --- | --- | --- |
| Date Posted |  | November 21, 2019 |  |
| Executive Summary |  | This KTC recommends how Energy Storage Resources (ESRs) shall be treated in the calculation of Physical Responsive Capability (PRC) and Operating Reserve Demand Curve (ORDC) reserves. |  |
| Recommendation Description |  | PRC provided by ESRs should consider energy limitations of the Storage Resource as well as droop settings and potential higher PRC contribution when charging. RT On-Line Reserve Capacity (RTOLCAP) provided by ESRs should also consider energy limitations of the ESR besides potential higher RTOLCAP contributions when charging. |  |
| BESTF Discussion |  | On 10/18/19, ERCOT staff presented material related to proposed contributions from ESRs to PRC, and ORDC Reserve. On 11/4/19, the BESTF reached consensus on KTC 2 items 1 and 2. |  |
| TAC Action Requested |  | BESTF plans to request at the 11/20/19 TAC meeting a vote to approve KTC 2. |  |
| TAC Action Summary |  | On 11/20/19, TAC approved KTC 2. |  |

Proposed KTC Recommendation Language

# Key Topic/Concept recommendation Language for TAC Approval

None.

# Key Topic/Concept recommendation Language Previously approved by tac

11/20/19 TAC Approved (Consensus from 11/4/19 BESTF meeting):

- PRC provided by ESRs should also consider energy limitations of the Storage Resources besides droop settings and potential higher PRC contribution when charging. To consider energy limitations, a specific time period is required.  This time period is currently recommended to be 15 minutes.

When online and sitting idle or discharging;

PRC=Min(X% of HSL based on droop, HSL-"ESR-Gen"injection,SOCsTelem-SOCsOperMin∆t)

When Charging;

PRC=Min(X% of {HSL+"ESR-CLR" MW}based on droop,  "ESR-CLR" MW+SOCsTelem-SOCsOperMin∆t)

Where ∆t=14 hour

ESR-Gen: Energy Storage Resource modeled as Generation Resource

ESR-CLR: Energy Storage Resource modeled as Controllable Load Resource (CLR)

SOC: State of Charge

- RTOLCAP provided by ESRs should also consider energy limitations of the Storage Resources besides potential higher RTOLCAP contribution when charging. To consider energy limitations, a specific time period is required.  This time period is currently recommended to be 15 minutes.

When online and sitting idle or discharging;

RTOLCAP=Min(HSL-"ESR-Gen" Base Point,SOCsTelem-SOCsOperMin∆t)

When Charging;

RTOLCAP=Min(HSL+"ESR-CLR" Base Point,  "ESR-CLR" Base Point+SOCsTelem-SOCsOperMin∆t)

Where ∆t=14 hour

ESR-Gen: Energy Storage Resource modeled as Generation Resource

ESR-CLR: Energy Storage Resource modeled as Controllable Load Resource (CLR)

SOC: State of Charge

# Key Topic/Concept recommendation Language IN DISCUSSION AT BESTF

None.

# Future Decision Points and Issues for Developing Key topic/Concept recommendation Language

None.

| Applicable Protocol Section(s) |  |
| --- | --- |
| Impacted System(s) / Application(s) |  |