| NPRR Number | 955 | NPRR Title | Define Limited Impact Remedial Action Scheme (RAS) |
| --- | --- | --- | --- |
| Date |  | December 3, 2019 |  |
| Submitter’s Information |  |  |  |
| Name |  | Freddy Garcia |  |
| E-mail Address |  | Freddy.Garcia@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-4245 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not Applicable |  |

Comments

ERCOT submits these comments to narrow the definition of Remedial Action Scheme (RAS) to include only those Facilities that are part of the Bulk Electric System, as that term is defined in the North American Reliability Corporation (NERC) Glossary of Terms.  These comments also define the new term RAS Entity, which is used throughout Nodal Operating Guide Revision Request (NOGRR) 183, Remedial Action Scheme (RAS) Submittal and Review Requirements.  The use of RAS Entity is intended to align with NERC Reliability Standard PRC-012-2, Remedial Action Schemes, which replaced RAS owner with the term RAS-entity.

Revised Cover Page Language

None

Revised Proposed Protocol Language

## 2.1	DEFINITIONS

Limited Impact Remedial Action Scheme (RAS) (see Remedial Action Scheme (RAS))

- Remedial Action Scheme (RAS)

A scheme designed to detect predetermined ERCOT System conditions and automatically take corrective actions on areas of the ERCOT System that are part of the Bulk Electric System, as that term is defined in the NERC Glossary of Terms.  These corrective actions include, but are not limited to, adjusting or tripping generation (MW and MVAr), tripping Load, or reconfiguring a System(s) to maintain a secure system.  RASs do not include under-frequency or under voltage Load shedding, the isolation of fault conditions, or out-of-step relaying (not designed as an integral part of an RAS).  RASs shall not be implemented on Interconnection Reliability Operating Limits (IROLs).  Additional criteria that are excluded from being classified as RAS are outlined in the Operating Guides.

Limited Impact Remedial Action Scheme (RAS)

A RAS that by inadvertent operation or failure to operate does not cause or contribute to ERCOT System cascading, uncontrolled separation, angular instability, voltage instability, voltage collapse, or unacceptably damped oscillations.

Remedial Action Scheme (RAS) Entity

Any Market Participant that owns Facilities that are included in a RAS.