| Principle Number | 5 | Principle Title | Day-Ahead Market |
| --- | --- | --- | --- |
| Date Posted |  | January 3, 2020 |  |
| Executive Summary |  | This Key Principle identifies what changes are needed to the Day-Ahead Market (DAM) in order to bring Day-Ahead Ancillary Service (AS) procurement into alignment with implementation of Real-Time Co-optimization (RTC). |  |
| Principle Description |  | The purpose of Key Principle 5, Day-Ahead Market, is to identify any necessary changes to DAM with the implementation of RTC: 1. Potential use of Ancillary Service Demand Curves (ASDCs) in the DAM optimization. 2. Changes to the DAM AS insufficiency process 3. Updates to the AS Obligation quantities for settlement based on actual DAM AS quantities procured, if different from the AS Plan quantities. 4. Virtual Offers for AS in the DAM. |  |
| RTCTF Discussion |  | On 8/27/19, ERCOT staff presented material introducing KP5 subsections (1) through (6). On 9/19/19, Morgan Stanley proposed virtual AS offers in the DAM, and the concept will be discussed further at future RTCTF meetings. On 10/9/19, RTCTF discussed KP5 subsections (1) through (7) and reached consensus on KP5 subsections (1) through (6). On 10/30/19, RTCTF reached consensus on KP5 subsection (7). On 12/3/19, RTCTF reached consensus on KP5 subsection (7)(j). On 12/19/19, RTCTF reached consensus on KP5 subsections (2)(a) and (7)(b). |  |
| TAC Action Requested |  | On 10/23/19, TAC vote to endorse KP5 subsections (1) through (6) for purposes of informing the Board. On 11/20/19, TAC vote to endorse KP5 subsection (7) for purposes of informing the Board. On 1/29/20, TAC vote to endorse KP5 subsections (2)(a), (7)(b), and (7)(j) for purposes of informing the Board. |  |
| TAC Action Summary |  | On 10/23/19, TAC voted to endorse KP5 subsections (1) through (6) for purposes of informing the Board. On 11/20/19, TAC voted to endorse KP5 subsection (7) for purposes of informing the Board. |  |
| ERCOT Opinion |  |  |  |
| Board Action Requested |  |  |  |
| Board Action Summary |  |  |  |

Proposed Principle Language

# Principle Concepts for TAC Endorsement

2)	The same ASDCs that are used in Real-Time will be used in DAM.

a. 	In some cases, negative self-arranged quantities may necessitate shifting the DAM ASDC to the right, as they are treated by DAM as bids at the highest price on each corresponding ASDC.

7)	AS Virtual Offers

b.	If awarded, the QSE will be paid the DAM price for the capacity times the quantity awarded.  The awarded QSE will pay the Real-Time price times the quantity awarded.

j.	The Default Uplift Invoice Process will include virtual AS awards to the QSE when calculating the Maximum MWh Activity for the Counter-Party that represents the QSE.

# Principle concepts Previously Endorsed by TAC

1)	ASDCs will be added to the DAM optimization, and will be used as an input affecting the AS quantity procured and the Market Clearing Price for Capacity (MCPC).

2)	The same ASDCs that are used in Real-Time will be used in DAM.

3)	The current DAM AS insufficiency process will be eliminated by removing:

a.	The process of reopening submission window for more offers;

b.	The AS penalty costs; and

c.	The current pricing run used when AS Offers are insufficient to meet the AS Plan.

4)	AS Obligation quantities posted by 06:00 in the Day-Ahead will become an advisory-only number based on the AS Plan.

a.	They will be used to validate self-arranged AS transactions. This validation will include any AS subtype limitations, e.g., Responsive Reserve (RRS) being provided via Under Frequency Relay (UFR).

5)	Minimum AS Obligation quantity will be 0.1 MW.

6)	After DAM is published, updated AS Obligation quantities will be calculated and published based on the actual DAM AS requirement. These quantities may differ from the 06:00 posting, and are the quantities that will be used for DAM Settlement.

a.	“DAM AS requirement” here means the sum of the DAM AS awards plus any self-arrangement.

b.	In the event that a QSE’s self-arranged quantity exceeds the final AS Obligation, the remainder will be paid to the QSE at the DAM MCPC. Self-arranged AS transactions will not be allowed to be submitted or updated after DAM.

7)	AS Virtual Offers

a.	Allow one part, unlinked offers of AS that do not represent an offer from a physical Resource from Qualified Scheduling Entities (QSEs).  The general purpose of adding this new transaction is convergence bidding, in contrast to conventional Resource-specific AS Offers.

b.	If awarded, the QSE will be paid the DAM price for the capacity times the quantity awarded.  The awarded QSE will pay the Real-Time price times the quantity awarded, via the AS Imbalance calculation.

c.	This proposal does not change the quantity of capacity purchased for each AS.  ERCOT will attempt to procure the quantity from its AS Plan from Resource-specific offers as well as virtual offers against respective ASDCs.

d.	Virtual offers can only be submitted for:

i.	Conventional Regulation (not Fast Frequency Response Service);

ii.	RRS – Primary Frequency Response type;

iii.	ECRS – dispatchable; and

iv.	Non-Spin.

e.	The capability to self-arrange AS in excess of a QSE’s AS Obligation will no longer be needed and can be removed.

f.	Virtual AS Offers will automatically expire at the close of the DAM.

g.	The QSE will have the award included in the calculation of the QSE’s position regarding any RUC Capacity-Short Charge.

h.	The credit calculation for Real-Time Liability Estimate (RTLE) will also need to be modified to include this capacity in a fashion similar to the DAM energy short calculations.

i.	The DAM credit exposure calculations will be modified to validate the virtual AS Offers against the available credit limit, similar to how DAM Energy-Only Offers are treated (evaluating the potential DAM/RT price risk).  This will take the form of the 90th percentile of any positive hourly difference between the RT MCPC and the DAM MCPC over the previous 30 days.

# Principle Concepts in DisCussion at RTCTF

None

# Future Decision Points and Issues for Developing Principle Concepts

None

| Applicable Protocol Section(s) | 4.4.7.1, Self-Arranged Ancillary Service Quantities 4.4.7.2, Ancillary Service Offers 4.4.7.2.1, Ancillary Service Offer Criteria 4.4.7.4, Ancillary Service Supply Responsibility 4.4.10, Credit Requirement for DAM Bids and Offers 5.7.4.1.1, Capacity Shortfall Ratio Share 16.11.4.3.2, Real-Time Liability Estimate |
| --- | --- |
| Impacted System(s) / Application(s) |  |