| NPRR Number | 826 | NPRR Title | Mitigated Offer Caps for RMR Resources |
| --- | --- | --- | --- |
| Date of Decision |  | February 11, 2020 |  |
| Action |  | Approved |  |
| Timeline |  | Normal |  |
| Effective Date |  | Upon system implementation |  |
| Priority and Rank Assigned |  | Priority – 2020; Rank – 2970 |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions - 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test 4.4.9.4.1, Mitigated Offer Cap 4.4.9.4.3, Mitigated Offer Cap for RMR Resources (new) 5.6.1, Verifiable Costs |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) creates a new process for determining the Mitigated Offer Cap (MOC) for Reliability Must-Run (RMR) Resources. It uses market design principles to allow the RMR Resource to be Dispatched but still be above other Resources that solve the same constraint. NPRR826 establishes a methodology to set the MOC for RMR Resources based on a “last in line” concept. The objective is to minimize the impact of an RMR Resource (out of market action) on competitive market outcomes. NPRR826 includes criteria that must be met before the methodology will be used. If all of the criteria is not met, the RMR Resource will be mitigated based on cost. - The RMR MOC methodology in NPRR826 will only be applied to an active constraint when all 3 conditions are true: 1) The RMR Resource has a 2% or more unloading Shift Factor on the Transmission Facility; 2) The RMR Resource has a 5% unloading impact* on the transmission facility; and 3) At least one other non-RMR Resource has a 5% or more unloading Shift Factor on the Transmission Facility. *Unloading impact is reduction of flow on the Transmission Facility and is used in ERCOT’s evaluation of whether the unit would be an effective RMR Resource. - The RMR MOC methodology in NPRR826 works as follows (i.e. to create the line): 1) Identify all non-RMR Resources that have a 5%* or more unloading Shift Factor on the Transmission Facility; 2) Collect the highest energy offer point (mitigated if applicable) for each Resource in step 1 and convert to a Shadow Price equivalent; 3) Select the highest value from step 2 and ensure it is less than the Shadow Price cap for the constraint by $1; 4) Add $0.01 to the value from step 3, multiply by the Shift Factor of the RMR Resource to convert back to an energy offer equivalent, and set as the RMR MOC; and 5) If there are multiple constraints, use the lowest value from step 4. *5% is set as a default value and can be updated by the Technical Advisory Committee (TAC). |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | It is essential to good market outcomes that ERCOT’s decision to procure an RMR Resource does not interfere with market fundamentals. Therefore, every effort must be made to identify what offers the RMR could undercut, and then set the RMR MOC to a reasonable level that will help prevent that impact, while still ensuring that the offer from the RMR Resource is Dispatchable to solve the constraint is was procured for. The benefits of locational pricing include sending a long-term investment price signal and a shorter term operational price signal. A long-term price signal helps identify where new generation should be sited, in consideration with other constraints, such as land availability, fuel availability, permit requirements, and other concerns. A shorter term signal is just as important. A more effective price signal will encourage supply and demand Resource behavior to address the local reliability need. Stakeholders should support this NPRR because it offers a solution to protect our market design here in Texas and recognizes that but for ERCOT’s intervention in creating an RMR Agreement, market fundamentals would be very different. |  |
| Credit Work Group Review |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR826 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| PRS Decision |  | On 5/11/17, PRS unanimously voted to table NPRR826 and refer the issue to WMS. All Market Segments were present for the vote. On 7/20/17, PRS voted to table NPRR826. There was one abstention from the Independent Power Marketer (IPM) (Morgan Stanley) Market Segment. All Market Segments were present for the vote. On 8/11/17, PRS voted to table NPRR826. There were two opposing votes from the Independent Retail Electric Provider (IREP) (Reliant) and IPM (Morgan Stanley) Market Segments and three abstentions from the Municipal (DME) and Independent Generator (2) (Dynegy, Exelon) Market Segments. All Market Segments were present for the vote. On 5/9/19, PRS voted to table NPRR826 for one month. There were three opposing votes from the IPM (2) (Morgan Stanley, Tenaska) and IREP (Reliant) Market Segments. All Market Segments were present for the vote. On 6/13/19, PRS unanimously voted to table NPRR826. All Market Segments were present for the vote. On 11/13/19, PRS unanimously voted to recommend approval of NPRR826 as amended by the 11/12/19 Joint comments. The IPM Market Segment was not present for the vote. On 12/12/19, PRS unanimously voted to table NPRR826 for one month. All Market Segments were present for the vote. On 1/16/20, PRS unanimously voted to endorse and forward to TAC the 12/12/19 PRS Report and Impact Analysis for NPRR826 with a recommended priority of 2020 and rank of 2970. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 5/11/17, participants noted the report referenced in the 5/10/17 NRG comments and requested tabling of NPRR826 for review by WMS. On 7/20/17, supporters reiterated the benefits of NPRR826 and urged PRS to recommend its approval. Other participants requested tabling of NPRR826 pending Public Utility Commission of Texas (PUCT) direction on several issues, including local scarcity and RMR. On 8/11/17, participants acknowledged the feedback from the 7/28/17 PUCT Open Meeting not to explicitly hold consideration of this issue awaiting PUCT direction, and recapped several topics discussed at the 8/10/17 PUCT workshop on price formation, including RMR and Reliability Unit Commitment (RUC). Participants requested tabling of NPRR826 to allow for continued review and discussion of these issues. On 5/9/19, participants reviewed the history of NPRR826, noted the recently filed comments, and requested PRS table NPRR826 for one additional month. On 6/13/19, participants reviewed the 5/7/19 TCPA comments and 6/10/19 Joint comments; discussed the appropriate Shift Factor cutoff for RMR Resources; and requested PRS table NPRR826 to allow for continued review. On 11/13/19, participants reviewed the 11/12/19 Joint comments, noting they represent an appropriate compromise to address stakeholder concerns regarding the treatment of RMR Resources. On 12/12/19, participants noted the request for additional time for development of an Impact Analysis for NPRR826 as outlined in the 12/11/19 ERCOT comments. On 1/16/20, there was no discussion. |  |
| TAC Decision |  | On 1/29/20, TAC voted to recommend approval of NPRR826 as recommended by PRS in the 1/16/20 PRS Report as amended by the 11/21/19 Joint comments. There was one abstention from the Consumer (Residential) Market Segment. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 1/29/20, participants reviewed the 11/21/19 Joint comments and Business Case for NPRR826. |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR826. |  |
| Board Decision |  | On 2/11/20, the ERCOT Board approved NPRR826 as recommended by TAC in the 1/29/20 TAC Report. |  |

| Sponsor |  |
| --- | --- |
| Name | Eric Goff |
| E-mail Address | Eric.Goff@citi.com |
| Company | Citigroup Energy Inc. |
| Phone Number |  |
| Cell Number | 512-632-7013 |
| Market Segment | IPM |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | cory.phillips@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| NRG 051017 | Noted the joint Calpine and NRG report, posted to several PUCT dockets, which attempts to provide independent analysis of, and recommended treatment of, RMR Units |
| WMS 060817 | Requested PRS continue to table NPRR826 for further review by WMS |
| ERCOT 062317 | Proposed edits to better align the revisions proposed within NPRR826 with the sponsor’s intent |
| Luminant 071217 | Requested PRS reject NPRR826 |
| WMS 071317 | Endorsed NPRR826 as amended by the 6/23/17 ERCOT comments |
| TIEC 071817 | Requested PRS reject NPRR826 |
| Citigroup 071917 | Reiterated the benefits of NPRR826 |
| Luminant 081017 | Responded to the 5/10/17 NRG comments; reiterated opposition to NPRR826 |
| Joint 111318 | Proposed the initial concept in NPRR826 be a manual phase 1 until an automated phase 2 would automatically create MOCs within Security-Constrained Economic Dispatch (SCED) for RMR Resources |
| Crescent Power 113018 | Proposed additional revisions to the 11/13/18 Joint comments to include RUC-committed Resources within the treatment proposed by NPRR826 |
| ERCOT 042619 | Proposed additional revisions to address technical limitations within the 11/30/18 Crescent Power comments, and other clarifications |
| TCPA 050719 | Proposed revisions to the 4/26/19 ERCOT comments to remove RUC from NPRR826 and eliminate any system changes within phase 1 |
| WMS 060619 | Endorsed NPRR826 as amended by the 5/7/19 TCPA comments |
| Joint 061019 | Proposed additional revisions to the 5/7/19 TCPA comments to remove a phase 1 implementation and add a -5% Shift Factor threshold for applying a different MOC for an RMR Resource |
| Reliant 062719 | Proposed additional revisions to the 6/10/19 Joint comments to utilize a 2% unloading Shift Factor within Section 4.4.9.4.3 |
| ERCOT 071619 | Noted that NPRR826 falls within the scope of P.U.C. SUBST. R. 25.502, Pricing Safeguards in Markets Operated by the Electric Reliability Council of Texas, and will not become effective until after the PUCT issues an order approving of the NPRR |
| Reliant 110719 | Proposed additional revisions to the 6/27/19 Reliant comments to raise the unloading Shift Factor within Section 4.4.9.4.3 from 2% to 5% |
| Joint 111219 | Proposed additional revisions to the 11/7/19 Reliant comments to address a situation within Section 4.4.9.4.3 where system lambda from step 1 could exceed the Energy Offer Curve at High Sustained Limit (HSL) from step 2 for non-RMR Resources |
| Joint 112119 | Proposed additional revisions to align the Revision Description and Business Case for NPRR826, and to maintain consistency with the revisions proposed in NPRR838, Updated O&M Cost for RMR Resources |
| ERCOT 121119 | Proposed an alternative schedule for development of an Impact Analysis for NPRR826 |

Market Rules Notes

Please note the baseline Protocol language in the following sections has been updated to reflect the incorporation of the following NPRR(s) into the Protocols:

- NPRR847, Exceptional Fuel Cost Included in the Mitigated Offer Cap (unboxed 4/5/19)

  - Section 2.1

  - Section 4.4.9.4.1

- NPRR906, Clarifying the Decision Making Entity Process (incorporated 5/1/19)

  - Section 3.19.4

- NPRR940, Removal of Language Related to NPRR664, Fuel Index Price for Resource Definition and Real-Time Make-Whole Payments for Exceptional Fuel Cost Events (incorporated 11/1/19)

  - Section 4.4.9.4.1

  - Section 5.6.1

Please note the following NPRR(s) also propose revisions to the following section(s):

- NPRR838, Updated O&M Cost for RMR Resources

  - Section 5.6.1

- NPRR986, BESTF-2 Energy Storage Resource Energy Offer Curves, Pricing, Dispatch, and Mitigation

  - Section 4.4.9.4.1

  - Section 5.6.1

Please note that administrative corrections have been made to the language below and authored as “ERCOT Market Rules”.

Proposed Protocol Language Revision

- 2.1	DEFINITIONS

- Mitigated Offer Cap (MOC)

An upper limit on the price of an offer as detailed in Section 4.4.9.4.1, Mitigated Offer Cap, and Section 4.4.9.4.3, Mitigated Offer Cap for RMR Resources.

- 3.19.4	Security-Constrained Economic Dispatch Constraint Competitiveness Test

(1)	The SCED CCT uses current system conditions to evaluate the competitiveness of a constraint.

(2)	Before each SCED execution, CCT is performed for all active constraints in SCED.  The SCED CCT shall classify a constraint as competitive for the current SCED execution if the constraint meets all of the following conditions:

(a)	The ECI is less than ECIT2 on the import side;

(b)	The constraint can be resolved by eliminating all Available Capacity for a Resource on the import side, except nuclear capacity and minimum-energy amounts of coal and lignite capacity, that is Managed Capacity for a DME.  If the constraint cannot be resolved, then the DME will be marked as the pivotal player for resolving the constraint;

(c)	There are negative Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have an absolute value greater than or equal to SFP3; and

(d)	The constraint was not designated as non-competitive by a previous SCED CCT execution within the current Operating Hour.

(3)	Any constraint that is analyzed and is not designated as a Competitive Constraint under the conditions outlined in paragraph (2) above shall be designated as a Non-Competitive Constraint by the SCED CCT.

(4)	A constraint that is determined to be a Non-Competitive Constraint by the SCED CCT within an Operating Hour will not be re-evaluated for its competitiveness status for the remainder of that Operating Hour.  However, the SCED CCT will reevaluate the percentage of the ECI Effective Capacity on the import side for each DME and whether the DME is a pivotal player for the constraint.  SCED will re-evaluate the competitiveness of the Non-Competitive Constraint starting with the first SCED interval of the next Operating Hour if the constraint remains active in SCED.

(5)	The Independent Market Monitor (IMM) may designate any constraint as a Competitive Constraint or a Non-Competitive Constraint.  ERCOT shall provide notice describing any such designation by the IMM.  The notice shall include an effective date, justification for the constraint designation by the IMM and the duration for which the IMM designation will be applied.  Any such designation from the IMM shall override the competitiveness status determined by the SCED CCT for the dates for which the IMM override is effective.

(6)	Each hour, ERCOT shall post on the MIS Public Area whether each binding constraint was designated as a Competitive Constraint or as a Non-Competitive Constraint for each of the SCED executions during the previous Operating Hour.

(7)	Mitigation will be applied to a Resource in the SCED Step 2, as described in Section 6.5.7.3, Security Constrained Economic Dispatch, when all of the following conditions are met:

(a)	A constraint has been determined to be a Non-Competitive Constraint by either the SCED CCT or the IMM;

(b)	The DME for the Resource is either identified as a pivotal player for the constraint as described in paragraph (4) above or has a percentage of ECI Effective Capacity on the import side for the constraint greater than DMEECP; and

(c)	The Resource has a Shift Factor on the import side of the constraint with an absolute value greater than SFP4;

(8)	Once mitigation has been applied to a Resource for a SCED interval, it shall remain applied for the remainder of the Operating Hour regardless of the conditions listed in paragraph (7) above.

- 4.4.9.4.1	Mitigated Offer Cap

(1)	Energy Offer Curves may be subject to mitigation in Real-Time operations under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Cap (MOC).  ERCOT shall construct an incremental MOC curve in accordance with Section 6.5.7.3 such that each point on the MOC curve is calculated as follows:

MOC q, r, h = Max [GIHR q, r * Max(FIP, WAFP q, r, h), (IHR q, r * FPRC q, r + OM q, r) * CFMLT q, r]

Where,

If a QSE has submitted an Energy Offer Curve on behalf of a Generation Resource and the Generation Resource has approved verifiable costs, then

FPRC q, r = Max(WAFP q, r, h, FIP + FA q, r) * RTPERFIP q, r / 100 + FOP * RTPERFOP q, r / 100

If a QSE has not submitted an Energy Offer Curve on behalf of a Generation Resource and the Generation Resource has approved verifiable costs, then

FPRC q, r = Max(WAFP q, r, h, FIP + FA q, r) * GASPEROL q, r / 100 + FOP * OILPEROL q, r / 100 + (SFP + FA q, r) * SFPEROL q, r / 100

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| MOC q, r, h | $/MWh | Mitigated Offer Cap per Resource—The MOC for Resource r, for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| GIHR q, r | MMBtu/MWh | Generic Incremental Heat Rate—The generic, single-value, incremental heat rate. For Generation Resources with a Commercial Operations Date on or before January 1, 2004, the generic incremental heat rate shall be set to 10.5. For Generation Resources that have a Commercial Operations Date after January 1, 2004, this value shall be set to 14.5. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| IHR q, r | MMBtu/MWh | Verifiable Incremental Heat Rate per Resource—The verifiable incremental heat rate curve for Resource r, as approved in the verifiable cost process. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| FIP | $/MMBtu | Fuel Index Price—The natural gas index price as defined in Section 2.1, Definitions. |
| RTPERFIP q, r | none | Fuel Index Price Percentage—The percentage of natural gas used by Resource r to operate above LSL, as submitted with the energy offer curve. |
| FOP | $/MMBtu | Fuel Oil Price—The fuel oil index price as defined in Section 2.1. |
| RTPERFOP q, r | none | Fuel Oil Price Percentage—The percentage of fuel oil used by Resource r to operate above LSL, as submitted with the energy offer curve. |
| SFP | $/MMBtu | Solid Fuel Price—The solid fuel index price is $1.50. |
| FPRC q, r | $/MMBtu | Fuel Price Calculated per Resource—The calculated index price for fuel for the Resource based on the Resources fuel mix. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| GASPEROL q, r | none | Percent of Natural Gas to Operate Above LSL—The percentage of natural gas used by Resource r to operate above LSL, as approved in the verifiable cost process. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| OILPEROL q, r | none | Percent of Oil to Operate Above LSL—The percentage of fuel oil used by Resource r to operate above LSL, as approved in the verifiable cost process. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SFPEROL q, r | none | Percent of Solid Fuel to Operate Above LSL—The percentage of solid fuel used by Resource r to operate above LSL, as approved in the verifiable cost process. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| FA q, r | $/MMBtu | Fuel Adder—The fuel adder is the average cost above the index price Resource r has paid to obtain fuel. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost Manual for additional information. |
| OM q, r | $/MWh | Variable Operations and Maintenance Cost above LSL—The O&M cost for Resource r to operate above LSL, including an adjustment for emissions costs, as approved in the verifiable cost process. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost Manual for additional information. |
| CFMLT q, r | none | Capacity Factor Multiplier—A multiplier based on the corresponding monthly capacity factor as described in paragraph (1)(d) below. |
| WAFP q, r, h | $/MMBtu | Weighted Average Fuel Price—The volume-weighted average intraday, same-day and spot price of fuel submitted to ERCOT during the Adjustment Period for a specific Resource and specific hour within the Operating Day, as described in paragraph (1)(f) below. |
| q | none | A QSE. |
| r | none | A Generation Resource. |
| h | none | The Operating Hour. |

- For a Resource contracted by ERCOT under paragraph (2) of Section 6.5.1.1, ERCOT Control Area Authority, ERCOT shall increase the O&M cost such that every point on the MOC curve is greater than the SWCAP in $/MWh.

(b)	The MOC for Energy Storage Resources shall be calculated in accordance with Verifiable Cost Manual Appendix 10, Procedures for Evaluating Costs and Caps for Energy Storage Resources.

(c)	For Quick Start Generation Resources (QSGRs) the MOC shall be adjusted in accordance with Verifiable Cost Manual Appendix 7, Calculation of the Variable O&M Value and Incremental Heat Rate used in Real Time Mitigation for Quick Start Generation Resources (QSGRs).

(d)	The multipliers for the MOC calculation above are as follows:

(i)	1.10 for Resources running at a ≥ 50% capacity factor for the previous 12 months;

(ii)	1.15 for Resources running at a ≥ 30 and < 50% capacity factor for the previous 12 months;

(iii)	1.20 for Resources running at a ≥ 20 and < 30% capacity factor for the previous 12 months;

(iv)	1.25 for Resources running at a ≥ 10 and < 20% capacity factor for the previous 12 months;

(v)	1.30 for Resources running at a ≥ 5 and < 10% capacity factor for the previous 12 months;

(vi)	1.40 for Resources running at a ≥ 1 and < 5% capacity factor for the previous 12 months; and

(vii)	1.50 for Resources running at a less than 1% capacity factor for the previous 12 months.

(e)	The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month.  ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

(f)	During the Adjustment Period, a QSE representing a Resource may submit Exceptional Fuel Cost as a volume-weighted average fuel price for use in the MOC calculation for that Resource.  To qualify as Exceptional Fuel Cost, the submission must meet the following conditions:

(i)	For all Resources, the weighted average fuel price must exceed FIP for the applicable Operating Day, plus a threshold parameter value of $1/MMBtu, plus the applicable fuel adder.  For Resources without approved verifiable costs, the fuel adder will be set to the default value assigned to Resources with approved verifiable costs, as defined in the Verifiable Cost Manual.  The threshold parameter value in this paragraph shall be recommended by the Wholesale Market Subcommittee (WMS) and approved by the Technical Advisory Committee (TAC).  ERCOT shall update the threshold value on the first day of the month following TAC approval unless otherwise directed by the TAC.  ERCOT shall provide a Market Notice prior to implementation of a revised parameter value.

(ii)	Fixed cost (fees, penalties and similar non-gas costs) may not be included in the calculation of the weighted average fuel price.

(iii)	All intra-day, same day, and spot fuel purchases must be included in the calculation of the weighted average fuel price in paragraph (1) above.  These must account for at least 10% of the total fuel volume burned by the applicable Resource for the hour for which the weighted average fuel price is computed.  As noted in paragraph (l) below, the methodology used in the allocation of the cost and volume of purchased fuel to the Resource for the hour is subject to validation by ERCOT.

(iv)	Weighted average fuel prices must be submitted individually for each Operating Hour for which they are applicable.  Values submitted outside of the Adjustment Period will be rejected and not used in the calculation of the MOC for the designated Operating Hour.

(g)	ERCOT may notify the Independent Market Monitor (IMM) if a QSE submits an Exceptional Fuel Cost.

(h)	No later than five Business Days after an Operating Day for which an Exceptional Fuel Cost is submitted, ERCOT shall issue a Market Notice indicating the affected Operating Hours and the number of Resources for which a QSE submitted Exceptional Fuel Cost for a particular Operating Day.

(i)	No later than 1700 Central Prevailing Time (CPT) on the 15th day following an Exceptional Fuel Cost submission, the submitting QSE shall provide ERCOT with the calculation of the weighted average fuel price, intraday or same-day fuel purchases, and any available supporting documentation.  Such information may include, but is not be limited to, documents of the following nature: relevant contracts between the QSE or Resource Entity and fuel supplier, trade logs, transportation, storage, balancing and distribution agreements, calculation of the weighted average fuel price, or any other documentation necessary to support the Exceptional Fuel Cost price and volume for the applicable period(s).

(j)	No later than 1700 Central Prevailing Time (CPT) on the 60th day following an Exceptional Fuel Cost submission, the submitting QSE shall provide ERCOT with all supporting documentation not previously provided to ERCOT.  No supporting documentation will be accepted after the 60th day.

(k)	The accuracy of submitted Exceptional Fuel Cost and the need for purchasing intraday or same-day gas must be attested to by a duly authorized officer or agent of the QSE representing the Resource.  The attestation must be provided in a standardized format acceptable to ERCOT and submitted with the other documentation described in paragraph (i) above.

(l)	ERCOT will use the supporting documentation to validate the Exceptional Fuel Cost for the applicable period. Validation will include, but not be limited to, the cost and the quantity of purchased fuel, Resource-specific heat rates, and the methodology used in the allocation of the cost and volume of purchased fuel to the Resource for the applicable hour used in the weighted average fuel price calculation.  In connection with the validation process ERCOT may request additional documentation or clarification of previously submitted documentation.  Such requests must be honored within ten Business Days.

(m)	At ERCOT’s sole discretion, submission and follow-up information deadlines may be extended on a case-by-case basis.

4.4.9.4.3	Mitigated Offer Cap for RMR Resources

(1)	For each Resource contracted by ERCOT under Section 3.14.1, Reliability Must Run, the Resource’s MOC curve for use in the SCED process is determined using the steps below when the Resource’s offer is subject to mitigation in accordance with Section 3.19, Constraint Competitiveness Tests.  The single price ($/MWh) value determined below will be used as the MOC curve for the full operating range of the Resource.  The calculations will occur between the first and second step within the SCED process as well as during the process for determining Real-Time On-Line Reliability Deployment Price Adder.  This analysis will only be applied to active constraints for which the contracted Resource has a more than 2% unloading Shift Factor on the Transmission Facility(s), more than 5% unloading impact on the Transmission Facility(s) based on telemetered HSL, and if at least one other Resource not contracted by ERCOT under Section 3.14.1 has an unloading Shift Factor of 5% or more relative to the constraint(s).  If this criteria is not met, the MOC curve for the Resource shall be calculated in accordance with Section 4.4.9.4.1, Mitigated Offer Cap, and Section 5.6.1, Verifiable Costs.

(a) 	For each Resource that is not a Resource contracted by ERCOT under Section 3.14.1 or paragraph (2) of Section 6.5.1.1 and that has an unloading Shift Factor of at least RMRSF percent relative to the constraint(s), determine the higher of zero or the difference between the price ($/MWh) at HSL from the Energy Offer Curves determined for use in SCED Step 2, which may or may not be mitigated, and system lambda from SCED Step 1 and divide that difference by the absolute value of that Resource’s Shift Factor for the corresponding constraint.  The value of RMRSF will default to 5% until a different value is approved by TAC considering the analysis and data used by ERCOT to determine the need for the contracted Resource under Section 3.14.1.  ERCOT shall post the current TAC-approved value(s) of RMRSF on the MIS Public Area.

(b) 	For each constraint, identify the largest value that is less than maximum Shadow Price for the specific constraint.

(c) 	For each SCED interval for each constraint, determine a value equal to the minimum of:

(i)	The value determined in paragraph (b) above plus $0.01/MWh; and

(ii)	The maximum Shadow Price for the constraint minus $1/MWh.

(d) 	For each SCED interval for each constraint, multiply the resulting value from paragraph (c) above by the absolute value of the Shift Factor of the Resource contracted by ERCOT to the corresponding constraint.  For SCED intervals in which there are multiple constraints which are being analyzed, the lowest value is used for the SCED interval.

(e)	If the value from paragraph (b) above for any constraint analyzed is zero, the MOC curve for the RMR Resource shall be calculated in accordance with Section 4.4.9.4.1 and Section 5.6.1.  If the value from paragraph (b) above for every constraint analyzed is greater than zero, the RMR Resource’s MOC curve for use in Step 2 of the SCED process is the sum of system lambda from Step 1 of SCED in the interval and the value from (d) above.

5.6.1	Verifiable Costs

(1)	The Qualified Scheduling Entity (QSE) is responsible for submitting verifiable costs unless both the QSE and Resource Entity agree that the Resource Entity will have this responsibility, in which case both the QSE and Resource Entity shall submit an affidavit to ERCOT stating this arrangement.  Notwithstanding the foregoing, QSEs that submit Power Purchase or Tolling Agreements (PPAs) do not have the option of allowing Resource Entities to file verifiable costs.

(2)	Make-Whole Payments for a Resource are based on the Startup Offers and Minimum-Energy Offers for the Resource, limited by caps.  Until ERCOT approves verifiable unit-specific costs for that Resource, the caps are the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  When ERCOT approves verifiable unit-specific costs for that Resource the caps are those verifiable unit-specific costs.  A QSE or Resource Entity may file verifiable unit-specific costs for a Resource at any time, but it must file those costs no later than 30 days after five Reliability Unit Commitment (RUC) events for that Resource in a calendar year.  A RUC event begins when a Resource receives a RUC instruction to come or stay On-Line and ends the later of when the Resource shuts down or the end of the Operating Day.  The most recent ERCOT-approved verifiable costs must be used going forward.

(3)	These unit-specific verifiable costs may include and are limited to the following average incremental costs:

(a)	Allocation of maintenance requirements based on number of starts between maintenance events using, at the option of the QSE or Resource Entity, either:

(i)	Manufacturer-recommended maintenance schedule;

(ii)	Historical data for the unit and actual maintenance practices; or

(iii)	Another method approved in advance by ERCOT in writing;

(b)	Startup fuel calculations based on recorded actual measured flows when the data is available or based on averages of historical flows for similar starts (for example, hot, cold, intermediate) when actual data is not available.  Startup fuel will include filing separately the startup fuel required to reach breaker close and fuel after breaker close to Low Sustained Limit (LSL).  Any fuel required to shutdown a Resource will be submitted as the fuel from breaker open to shutdown;

(c)	Operation costs;

(d)	Chemical costs;

(e)	Water costs; and

(f)	Emission credits.

(4)	Standard Operations and Maintenance (O&M) costs pursuant to paragraph (6) below may be used in lieu of the incremental O&M costs set forth in items (3)(a), (c), (d) and (e) above.

(5)	These unit-specific verifiable costs may not include:

(a)	Fixed costs, which are any cost that is incurred regardless of whether the unit is deployed or not; and

(b)	Costs for which the QSE or Resource Entity cannot provide sufficient documentation for ERCOT to verify the costs.

(6)	At their election, QSEs or Resource Entities may receive standard O&M costs for both startup and minimum energy.  This election may be made by submitting an election form to ERCOT.  If a QSE or Resource has received final approval for actual verifiable O&M costs under the verifiable cost process, it may not elect to receive standard O&M costs.

(a)	Until December 31, 2011, standard O&M costs are defined as follows:

| Resource Category Start Year = 2009 | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Aeroderivative simple cycle commissioned after 1996 | 1,000.00 | 1,000.00 | 1,000.00 | 3.94 |
| Reciprocating Engine | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | 5.09 |
| Simple cycle ≤ 90 MW | 2,300.00 | 2,300.00 | 2,300.00 | 3.94 |
| Simple cycle ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 | 3.94 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 3.19 |
| Combustion turbine < 90 MW | 2,300.00 | 2,300.00 | 2,300.00 |  |
| Combustion turbine ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 |  |
| Steam turbine | 3,000.00 | 2,250.00 | 1,250.00 |  |
| Gas-steam non-reheat boiler | 2,310.00 | 1,732.50 | 866.25 | 7.08 |
| Gas-steam reheat boiler | 3,000.00 | 2,250.00 | 1,125.00 | 7.08 |
| Gas-steam supercritical boiler | 4,800.00 | 3,600.00 | 1,800.00 | 7.08 |
| Nuclear, coal, lignite and hydro | 7,200.00 | 5,400.00 | 2,700.00 | 5.02 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 5.50 |

(b)	For the period beginning January 1, 2012 and ending December 31, 2012, standard O&M costs shall be reduced by 10% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 900.00 | 900.00 | 900.00 | 3.55 |
| Reciprocating Engine | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | 4.58 |
| Simple cycle ≤ 90 MW | 2,070.00 | 2,070.00 | 2,070.00 | 3.55 |
| Simple cycle ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 | 3.55 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.87 |
| Combustion turbine < 90 MW | 2,070.00 | 2,070.00 | 2,070.00 |  |
| Combustion turbine ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 |  |
| Steam turbine | 2,700.00 | 2,025.00 | 1,125.00 |  |
| Gas-steam non-reheat boiler | 2,079.00 | 1,559.25 | 779.63 | 6.37 |
| Gas-steam reheat boiler | 2,700.00 | 2,025.00 | 1,012.50 | 6.37 |
| Gas-steam supercritical boiler | 4,320.00 | 3,240.00 | 1,620.00 | 6.37 |
| Nuclear, coal, lignite and hydro | 6,480.00 | 4,860.00 | 2,430.00 | 4.52 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.95 |

(c)	Beginning January 1, 2013 and going forward, standard O&M costs shall be reduced by 20% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 800.00 | 800.00 | 800.00 | 3.15 |
| Reciprocating Engine | $46.40/MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | 4.07 |
| Simple cycle ≤ 90 MW | 1,840.00 | 1,840.00 | 1,840.00 | 3.15 |
| Simple cycle ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 | 3.15 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.55 |
| Combustion turbine < 90 MW | 1,840.00 | 1,840.00 | 1,840.00 |  |
| Combustion turbine ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 |  |
| Steam turbine | 2,400.00 | 1,800.00 | 1,000.00 |  |
| Gas-steam non-reheat boiler | 1,848.00 | 1,386.00 | 693.00 | 5.66 |
| Gas-steam reheat boiler | 2,400.00 | 1,800.00 | 900.00 | 5.66 |
| Gas-steam supercritical boiler | 3,840.00 | 2,880.00 | 1,440.00 | 5.66 |
| Nuclear, coal, lignite and hydro | 5,760.00 | 4,320.00 | 2,160.00 | 4.02 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.40 |

(d)	If the QSE or Resource Entity chooses to utilize the standard O&M costs for O&M, standard O&M costs will be used by ERCOT going forward until either:

(i)	Verifiable variable O&M costs are filed; or

(ii)	ERCOT notifies the QSE or Resource Entity to update its verifiable costs as set forth in either paragraph (9) or (10) below.  If a Resource is receiving standard O&M costs, it may reelect standard O&M costs when resubmitting verifiable costs.

(7)	When submitting verifiable costs for combined cycle Resources, the QSE or Resource Entity must elect standard O&M costs for all Combined-Cycle Configurations or verifiable costs for all Combined-Cycle Configurations within the combined cycle train.

(8)	QSEs submitting PPAs as Resource-specific verifiable costs documentation are subject to the guidelines detailed below and in the Verifiable Cost Manual.

(a)	Only QSEs offering Three-Part Supply Offers for a specific Resource may submit a PPA as verifiable costs documentation.

(b)	A QSE submitting a PPA as verifiable costs documentation must represent 100% of the Resource’s capacity.

(c)	Only PPAs:

(i) 	Signed prior to July 16, 2008; and

(ii)	Not between Affiliates, subsidiaries or partners will be accepted as verifiable cost documentation.

(d)	Verifiable costs for PPAs shall be capped at the level of the highest comparable Resource (referred to as the reference Resource) specific verifiable costs approved by ERCOT without a PPA.  The ERCOT approved verifiable costs for a PPA shall be equal to the lesser of:

(i)	The cap as described in paragraph (d) above; and

(ii)	The costs from the PPA.

(e)	ERCOT shall use the Resource actual fuel costs submitted by the QSE for startup and operation at minimum-energy level (LSL), and shall use the Resource Category Startup Offer Generic Costs as the cap for the O&M portion of the Startup Costs until ERCOT receives and approves comparable Resource specific verifiable costs.

(f)	PPAs will no longer be accepted as verifiable cost documentation after the primary term of the contract expires.

(g)	ERCOT shall produce a report each April that provides the percentage of RUC Make-Whole Payments for Resources with PPAs during the 12 months of the previous calendar year.  If there are no Make-Whole Payments for Resources with PPAs, ERCOT shall not produce the annual report.  The report shall be based on the final Settlements and include the total number of Resources that used a PPA for their most recent verifiable cost submission that was approved by ERCOT.  ERCOT shall present the results of this study to the appropriate Technical Advisory Committee (TAC) subcommittee.

(h)	Notwithstanding anything to the contrary in this Section 5.6.1, QSEs representing PPAs may, at any time, submit data from a Resource as verifiable costs documentation and such documentation will be accepted for consideration by ERCOT.  A QSE submitting verifiable costs documentation pursuant to this paragraph shall not be required to submit a PPA to ERCOT for consideration for verifiable cost recovery.

(9)	ERCOT shall notify a QSE to update verifiable cost data of a Resource when the Resource has received more than 50 RUC instructions meeting the criteria in Section 5.6.2, RUC Startup Cost Eligibility, in a year, but ERCOT may not request an update more frequently than annually.

(10)	ERCOT shall notify a QSE to update verifiable cost data of a Resource if at least five years have passed since ERCOT previously approved verifiable cost data for that Resource.

(11)	Within 30 days after receiving an update Notice from ERCOT under either paragraph (9) or (10) above, a QSE or Resource Entity must submit verifiable cost data for the Resource.  Despite the provisions in paragraph (2) above, if the QSE or Resource Entity does not submit verifiable cost data within 30 days after receiving an update Notice, then ERCOT shall determine payment using the Resource Category Startup Offer Generic Cap, Resource Category Minimum-Energy Offer Generic Cap, and a zeroed value for variable O&M Cost as described in Section 4.4.9.4.1, Mitigated Offer Cap, in accordance with the schedule established in this section until updated verifiable costs are approved.  If the 30-day deadline has been reached before the start of the tenth day before the end of the month, the Resource’s verifiable costs will revert back to generic costs beginning on the first day of the following month.  If the 30-day deadline falls within the last ten days of the month, the Resource’s verifiable costs will revert back to generic costs on the first day of the second month following the deadline month.