| LPGRR Number | XXX | LPGRR Title | Profile Decision Tree Revisions |
| --- | --- | --- | --- |
| Date Posted |  | February TBD, 2020 |  |
| Requested Resolution |  | Normal |  |
| Load Profiling Guide Sections Requiring Revision |  | Appendix D, Profile Decision Tree – Segment Assignment Appendix D, Profile Decision Tree – TOU Schedules Appendix D, Profile Decision Tree – Oil & Gas Appendix D, Profile Decision Tree – Non-ERCOT Profile IDs |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | The original process for assigning oil and gas profiles was created before Advanced Meters were adopted. Seeing that the vast majority of new meter installations use Advanced Meters, this Load Profiling Guide Revision Request (LPGRR) proposes to update the Oil & Gas Flat (OGFLT) Profile assignment to reflect this change. The proposed changes will streamline the assignment of oil and gas profiles by eliminating current processes which are no longer applicable. Some of the details include: - ERCOT will only validate weather sensitivity for Non-Interval Data Recorder (NIDR) Electric Service Identifiers (ESI IDs) that request the OGFLT Profile. - Remove the “TOU Schedules” and “Non-ERCOT Profile IDs” worksheets. - Change the Distributed Generation (DG) Profile Segment assignment process where the Transmission and/or Distribution Service Provider (TDSP) assigns the DG Profile Segment for residential and business profiles and reports the assignment to ERCOT. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Streamline the processes for Oil & Gas and DG Profile requests. The worksheets “TOU Schedules” and “Non-ERCOT Profile IDs” are no longer used. |  |

| Sponsor |  |
| --- | --- |
| Name | Calvin Opheim |
| E-mail Address | Calvin.Opheim@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3944 |
| Cell Number | 512-750-2411 |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jordan Troublefield |
| E-Mail Address | jordan.troublefield@ercot.com |
| Phone Number | 512-248-6521 |

Market Rules Notes

None

Proposed Guide Language Revision

Appendix D, Profile Decision Tree – “Segment Assignment”  worksheet

|  | II. Residential (RES) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | For each ESI ID, step through 'A' and then 'B' or 'C' below as appropriate to determine the applicable Profile Segment. Then follow the instructions in 'D' below for ESI IDs that have Distributed Generation (per the DG tab). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | A. | Default Residential ESI ID Profile Segment |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Assign the default Profile Segment for the initial assignment of all new Residential ESI IDs as directed below. Please refer to the Load Profiling Guide for Opt-In and transition assignments. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | If Weather Zone = COAST or if Weather Zone = FWEST then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | assign LOWR |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else assign HIWR. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | COAST default = LOWR |  |  |  | NORTH default = HIWR |  |  |  |  |  |  |
|  |  |  |  |  |  | EAST default = HIWR |  |  |  | SCENT default = HIWR |  |  |  |  |  |  |
|  |  |  |  |  |  | FWEST default = LOWR |  |  |  | SOUTH default = HIWR |  |  |  |  |  |  |
|  |  |  |  |  |  | NCENT default = HIWR |  |  |  | WEST default = HIWR |  |  |  |  |  |  |
|  |  | B. | Residential Annual Validation for NIDR ESI IDs |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | For Annual Validation, assign the applicable Profile Segment for each ESI ID based on the steps below. Because the steps below are not mutually exclusive, it is necessary to step through each of the following in the order listed, for each ESI ID, until an applicable case is found. Once an applicable case has been found follow the instructions below in Section C. Distributed Generation Profile Segment determination. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1. Usage Time Period: The four-year period inclusive of the applicable calendar year. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2. For each RES ESI ID, identify all usage data for which the entire meter read period falls between Sep 20 and May 10 (inclusive) for the various years of the Usage Time Period and spans no longer than 44 days. Use the resulting data from this step in the subsequent steps. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Note: Step 2 allows for only the Winter and Shoulder meter reads to be included in the subsequent steps to determine the Segment assignment. Meter reads are to be classified as Winter or Shoulder reads in step 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3. For each meter read usage value determine if the read is a Winter or a Shoulder season for each RES ESI ID. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | If start date ≥ Dec 01 or stop date ≤ Mar 01 then season = Winter |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if start date ≥ Sept 20 and stop date ≤ Dec 01 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | season = Shoulder |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if start date ≥ Mar 01 and stop date ≤ May 11 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | season = Shoulder |  |  |  |  |  |  |  |  |
|  |  |  |  |  | otherwise calculate shoulder percent as follows: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the read spans Dec 01 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | shoulder_percent = (Dec 01 - start date) / (stop date - start date) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the read spans Mar 01 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | shoulder_percent = (Mar 01 - start date) / (stop date - start date) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if shoulder_percent >= 0.6 then season = Shoulder |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if shoulder_percent <= 0.4 then season = Winter |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if a meter read has not been classified by season above then disregard the read |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 4. Calculate Average Daily Use for the meter read period (ADUsep) and assign a year value for each reading for each ESI ID. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Let |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | yy = July 1st of current year |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | yy-1 = July 1st of last year |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | yy-2 = July 1st of two years ago |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | yy-3 = July 1st of three years ago |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | If stop date < yy-3 then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | year value = 1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if stop date < yy-2 then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | year value = 2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if stop date < yy-1 then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | year value = 3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else year value = 4 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 5. Calculate the mean and standard deviation (stdev) of the Average Daily Use (ADUsep) for each ESI ID. If stdev > 0 then create a normalized ADUsep (NADUsep), use the NADUsep to identify outliers and set their reading usage values and ADUsep to null. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | where |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | and n = number of meter readings for the ESI ID |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | If NADUsep > 3.5 or |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if NADUsep > 3 and ADUsep >100 or |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if NADUsep < -2 or |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if NADUsep < 5 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | kWhp = null |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | ADUsep = null |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else |  | kWhp = kWhp |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | ADUsep = ADUsep |  |  |  |  |  |  |  |  |
|  |  |  | 6. Count the number of readings that have a usage value (not null) for each Season and continue with Step 6 for those ESI IDs that have more than two Winter readings and more than two Shoulder readings. For those ESI IDs that do not have sufficient number of readings do not change the current Load Profile Type assignment. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 7. For each meter reading time period compute the RESHIWR kWhp and the RESLOWR kWhp by summing the kWh interval values separately for the RESHIWR and RESLOWR backcasted profiles (available on the Load Profiling page at www.ercot.com) corresponding to the specific days in meter reading period "p". |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8. Compute the ESI ID year use, RESHIWR year use, and RESLOWR year use by summing the kWhp, RESHIWR kWhp, and RESLOWR kWhp respectively for each year value as determined in Step 3 above. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 9. For each year value compute a scaled RESHIWR kWh use and a scaled RESLOWR kWh use for each meter reading. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 10. Determine the correlation (R2) to the RESHIWR and RESLOWR profiles for each ESI ID. The correlations are determined with a weighted linear regression analysis. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Each reading is weighted as follows: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | If season = Winter |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | and RESLOWR kWhp > 0 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | and RESHIWR kWhp > RESLOWR kWhp then |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | weightp = 2(RESHIWR kWhp / RESLOWR kWhp) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else weightp = 1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | where |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | wi = weightp |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | yi = kWhp |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | xi = S RESHIWR kWhp or S RESLOWR kWhp |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 11. Identify the Winter Max ADUsep for each ESI ID for the entire usage time period. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 12. For each ESI ID, assign either HIWR (High Winter Ratio) or LOWR (Low Winter Ratio) based on the results of the previous steps, as follows. Because A thru D below are not mutually exclusive, it is necessary to step through each of the following in the order listed, for each ESI ID, until an applicable case is found. (Please note that the breakpoint values below are subject to change periodically.) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | If the ESI ID's Winter Max ADUsep < 20 kWh/day then assign LOWR |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if the ESI ID's correlation to the RESHIWR profile > 0.60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | and the correlation to the RESHIWR profile > correlation to the RESLOWR profile |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | then assign HIWR |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if the number of readings available > 9 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | and the correlation to the RESHIWR profile > 0.90 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | and (the correlation to the RESHIWR profile + 0.009) > the correlation to the RESLOWR profile |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | and Winter Max ADUsep > 53 kWh/day then assign HIWR |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else assign LOWR |  |  |  |  |  |  |  |  |  |  |  |
|  |  | C. | Residential Annual Validation for IDR ESI IDs |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | For each ESI ID with a meter data type of IDR, perform the following. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1. Determine a list of ESI IDs that were active on January 1 of the year two years prior to the current year. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2. Calculate two variables for each day on which the ESI ID is active and for which the actual interval data is available for the following months. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | January of the current year |  |  |  |  | February of the current year |  |  |  |  |  |
|  |  |  |  |  |  | January of the previous year |  |  |  |  | February of the previous year |  |  |  |  |  |
|  |  |  |  |  |  | January from two years ago |  |  |  |  | February from two years ago |  |  |  |  |  |
|  |  |  |  |  | The two variables are: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | (a) Daily kWh; and |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | (b) Average Weather Zone daily dry bulb temperature. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ESI IDs must have at least 90% of the total monthly intervals for all six months to proceed to step 3. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3. Calculate R-square (Pearson Product Moment Coefficient of Determination) values between these two variables, for each of the six months listed above. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 4. For each ESI ID, assign the appropriate Profile Segment based on A thru B below. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | a. | If the existing assignment is LOWR (or a DG variation such as LOPV) then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate R-square values for any of the six |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | months then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | do not change assignment; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if any three of the six R-square values is ≥ 0.6 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign HIWR; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else do not change assignment. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | b. | If the existing assignment is HIWR (or a DG variation) then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate R-square values for each of the six |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | months then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | do not change assignment; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if all of the six R-square values are ≤ 0.4 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOWR; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else do not change assignment. |  |  |  |  |  |  |  |  |  |  |
|  |  | D. | Assign a DG Profile Segment per the DG tab and report the assignment to ERCOT. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1. If the ESI ID has any PV generation then |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if segment is determined to be HIWR then assign HIPV; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be LOWR then assign LOPV; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2. Else if the ESI ID has wind generation then |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if segment is determined to be HIWR then assign HIWD; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be LOWR then assign LOWD; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3. Else if the ESI ID has other DG then |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if segment is determined to be HIWR then assign HIDG; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be LOWR then assign LODG. |  |  |  |  |  |  |  |  |  |  |  |

|  | III. Business (BUS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Assignment Year for Average Load Factor calculations: The previous calendar year (January through December) will be used to calculate the Average Load Factor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | For each ESI ID, assign the applicable Profile Segment based on the steps below. Because the steps below are not mutually exclusive, it is necessary to step through each of the following in the order listed, for each ESI ID, until an applicable case is found. Once an applicable case has been found follow the instructions in 'E' below for ESI IDs that have Distributed Generation (per the DG tab). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | A. Assign the IDRRQ ('IDR required') Profile Segment to all BUS ESI IDs required to have an IDR Meter, per Section 18 of the ERCOT Nodal Protocols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | B. Assign the OGFLT (Oil & Gas Flat) Profile Segment to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ESI IDs for which ERCOT has informed the TDSP that OGFLT should be assigned per the Oil & Gas tab. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | C. Assign the NODEM Profile Segment for non-residential ESI IDs which are not billed demand. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | D. Determine the Average Load Factor (AvgLF) for ESI IDs that were not assigned a Profile Segment in Steps 1, 2, or 3 above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1. Determine Usage Month values (ActiveDaysm, kWDaysm, kWhm, MaxkWm, and ADUsem) for each ESI ID for the 12 months of the Assignment Year, which is listed near the beginning of Section III. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2. Compute the Average Hourly Usage (AHUsem) for the Usage Months of the Assignment Year. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | where | kWhm = consumption in kilowatt hours in Usage Month m, and |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ActiveDaysm = Number of Active Days in Usage Month m. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | * Round to two decimal places, per the Rounding instructions on the Definitions tab. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3. Compute the Average Load Factor (AvgLF) as shown below for the Usage Months of the current Assignment Year. TDSPs that measure kVA at the ESI ID level should reference the 'kVA to kW' tab before proceeding. The Average Load Factor is a weighted average of the individual monthly load factors, where demand levels are used to define the weights (presented in a mathematically equivalent calculation below). AHUsem and MaxkWm values are required for each of the 12 months of the current Assignment Year in order to calculate AvgLF. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | where | AHUsem = Average Hourly Use in Usage Month m as previously defined, and |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | MaxkWm = Maximum metered kW Demand in Usage Month m, as defined on the Usage Month methodology tab. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | * Round to two decimal places, per the Rounding instructions on the Definitions tab. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 4. For each ESI ID, assign the appropriate Profile Segment based on A thru G below. Because A thru G below are not mutually exclusive, it is necessary to step through each of the following in the order listed, for each ESI ID, until an applicable case is found. (Please note that the breakpoint values below are subject to change periodically.) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | a. | If there is no existing assignment then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate the AvgLF** then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if data were available (e.g., for Opt-in entities) to calculate the AvgLF then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | if the AvgLF < 0.40 then |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | else if 0.40 ≤ AvgLF ≤ 0.60 then |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | assign MEDLF; |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | else assign HILF. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | b. | If the existing assignment is LOLF (or a DG variation, such as LOWD) then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate the AvgLF** then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | do not change assignment from LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if the AvgLF < 0.40 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | do not change assignment from LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if 0.40 ≤ AvgLF ≤ 0.60 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign MEDLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else assign HILF. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | c. | If the existing assignment is MEDLF (or a DG variation) then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate the AvgLF** then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | do not change assignment from MEDLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if the AvgLF < 0.40 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if 0.40 ≤ AvgLF ≤ 0.60 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | do not change assignment from MEDLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else assign HILF. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | d. | If the existing assignment is HILF (or a DG variation) then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate the AvgLF** then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | do not change assignment from HILF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if the AvgLF < 0.40 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if 0.40 ≤ AvgLF ≤ 0.60 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign MEDLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else do not change assignment from HILF. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e. | If the existing assignment is IDRRQ then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate the AvgLF** then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if the AvgLF < 0.40 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if 0.40 ≤ AvgLF ≤ 0.60 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign MEDLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else assign HILF. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | f. | If the existing assignment is NODEM (or a DG variation) then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate the AvgLF** then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if the AvgLF < 0.40 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if 0.40 ≤ AvgLF ≤ 0.60 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign MEDLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else assign HILF. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | g. | If the existing assignment is neither LOLF, MEDLF, HILF, IDRRQ, nor NODEM then |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | if the required data were not available to calculate the AvgLF** then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if the AvgLF < 0.40 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign LOLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else if 0.40 ≤ AvgLF ≤ 0.60 then |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | assign MEDLF; |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | else assign HILF. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ** or if the mathematical calculation of the AvgLF is undefined due to a zero (0) in the denominator |  |  |  |  |  |  |  |  |  |  |  |
|  |  | E. | Assign a DG Profile Segment per the DG tab and report the assignment to ERCOT. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1. | If the ESI ID would otherwise be assigned IDRRQ then |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | assign IDRRQ; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2. | Else if the ESI ID has any PV generation then |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if segment is determined to be HILF then assign HIPV; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be MEDLF then assign MEDPV; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be LOLF then assign LOPV; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be NODEM then assign NODPV; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be OGFLT then assign OGFPV; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3. | Else if the ESI ID has wind generation then |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if segment is determined to be HILF then assign HIWD; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be MEDLF then assign MEDWD; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be LOLF then assign LOWD; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be NODEM then assign NODWD; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be OGFLT then assign OGFWD.; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 4. | Else if the ESI ID has other DG then |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | if segment is determined to be HILF then assign HIDG; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be MEDLF then assign MEDDG; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be LOLF then assign LODG; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be NODEM then assign NODDG; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | else if segment is determined to be OGFLT then assign OGFDG. |  |  |  |  |  |  |  |  |  |  |  |

Appendix D, Profile Decision Tree – “TOU Schedules” worksheet

Appendix D, Profile Decision Tree – “Oil & Gas”  worksheet

|  | Only the current CR of Record may seek to have the Oil & Gas Flat (OGFLT) Profile Segment assigned to one of their ESI IDs and are required to provide ERCOT (ERCOTLoadProfilingDepartment@ercot.com) with documentation, either electronic (preferred) or hard copy, of the following: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1. | Sales Tax Exemption Certificate (on file with the CR for the ESI ID) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2. | Customer Certification to the CR that the Customer holds an official Railroad Commission of Texas Operator Number and uses electricity at the premises identified by the ESI ID for the purpose of exploring, producing or transporting oil and/or natural gas extracted from the earth. This Customer Certification must be signed by an official company representative and shall list the Railroad Commission of Texas Operator Number, the name of the entity holding the Operator Number, the ESI ID and the service address of the ESI ID. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | In addition, for NIDR ESI IDs to be assigned the Oil & Gas Flat (OGFLT) profile, ERCOT shall validate that they are not weather sensitive. |  |  |  |  |  |  |  |  |  |  |  |
|  | The Oil & Gas Flat segment assignment shall not change simply due to a switch in CRs. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Should a previously approved ESI ID become ineligible to be served under an Oil & Gas Flat segment, then ERCOT shall notify the appropriate TDSP to change the Profile Type to the current appropriate default Profile Type as if the ESI ID represented a new premise. TDSPs shall make reasonable effort to effect the change using the appropriate Texas SET process as soon as possible after notification. ERCOT shall notify the CR of Record of any such changes to the Profile Type. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Appendix D, Profile Decision Tree – “Non-ERCOT Profile IDs” worksheet