| NOGRR Number | 214 | NOGRR Title | Geomagnetic Disturbance (GMD) Measurement Data Reporting |
| --- | --- | --- | --- |
| Date of Decision |  | June 4, 2020 |  |
| Action |  | Tabled |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Nodal Operating Guide Sections Requiring Revision |  | 6.1.1, Introduction 6.1.5.1, Geomagnetic Disturbance (GMD) Measurement Data Processes (new) |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Operating Guide Revision Request (NOGRR) describes the process for ERCOT to collect Geomagnetically-Induced Current (GIC) monitor and magnetometer data to satisfy Requirements R11 and R12 of North American Electric Reliability Corporation (NERC) Reliability Standard TPL-007-3, Transmission System Planned Performance for Geomagnetic Disturbance Events. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | NERC Reliability Standard TPL-007-3 requires processes for obtaining GIC monitor data and geomagnetic field data (Requirements R11 and R12). Planning Guide Revision Request (PGRR) 070, Revised Responsibilities for Performing Geomagnetic Disturbance (GMD) Vulnerability Assessments, which was approved by the ERCOT Board in June 2019, requires collaboration between ERCOT, Transmission Service Providers (TSPs), and Resource Entities to implement the processes required for TPL-007-3, Requirements R11 and R12. This NOGRR describes the process for ERCOT to collect GIC monitor and magnetometer data to satisfy Requirements R11 and R12 of NERC Reliability Standard TPL-007-3. |  |
| ROS Decision |  | On 6/4/20, ROS unanimously voted via roll call to table NOGRR214 and refer the issue to the Planning Geomagnetic Disturbance Task Force (PGDTF). All Market Segments were present for the vote. |  |
| Summary of ROS Discussion |  | On 6/4/20, there was no discussion. |  |

| Sponsor |  |
| --- | --- |
| Name | John Bernecker |
| E-mail Address | John.Bernecker@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4274 |
| Cell Number |  |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Phillip Bracy |
| E-Mail Address | Phillip.Bracy@ercot.com |
| Phone Number | 512-248-6917 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| AEPSC 060220 | Proposed extending response deadlines in Section 6.1.5.1 to 60 days, modified requirements in paragraph (2) such that TSPs and Resource Entities would only be required to provide GMD measurement data for events when the global geomagnetic activity index for GMD events indicates a value of KP7 or greater |

Market Rules Notes

Please note that administrative clarifications have been made to the language below and authored as “ERCOT Market Rules”.

Proposed Guide Language Revision

6.1.1	Introduction

(1)	Disturbance monitoring is necessary to:

(a)	Determine performance of the ERCOT System;

(b)	Determine effectiveness of protective relaying systems;

(c)	Verify ERCOT System models;

(d)	Determine causes of ERCOT System disturbances (trips, faults, and protective relay system actions); and

(e)	Meet the requirements of North American Reliability Corporation (NERC) Reliability Standards.

(2)	To ensure that adequate data is available for these activities, the disturbance monitoring requirements and procedures discussed in these Operating Guides have been established by ERCOT for the following:

(a)	Fault recording, sequence of events recording, and dynamic disturbance recording equipment owners in the ERCOT System; and

(b)	Transmission Service Providers (TSPs) and Resource Entities with equipment for recording Geomagnetic Disturbance (GMD) measurement data, including Geomagnetically-Induced Current (GIC) monitors and/or magnetometers for recording geomagnetic field data, installed at their facilities.

6.1.5.1	Geomagnetic Disturbance (GMD) Measurement Data Processes

(1)	TSPs and Resource Entities shall notify ERCOT on an annual basis, by October 31, of any addition of, or changes to, equipment for recording GMD measurement data installed at their facilities.  Additionally, a complete list of GMD measurement equipment shall be provided, when specifically requested by ERCOT, within 30 days.

(2)	TSPs and Resource Entities with GMD measurement equipment installed at their facilities shall provide GMD measurement data, when requested specifically by ERCOT, within 30 days.

(3)	ERCOT may, at the request of TSPs, post GMD measurement data obtained from TSPs, Resource Entities, or publicly available sources to the Market Information System (MIS) Certified Area for TSPs.