| SCR Number | 810 | SCR Title | EMS System Change to Count DC Ties toward the 2% Constraint Activation Criterion |
| --- | --- | --- | --- |
| Date of Decision |  | May 15, 2020 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Supporting Protocol or Guide Section(s) / Related Documents |  | None |  |
| System Change Description |  | This System Change Request (SCR) adds logic to ERCOT’s Energy Management System (EMS) system to remove the flag that indicates to the ERCOT Operator that a unit representing a Direct Current Tie (DC Tie) does not count toward the 2% criterion for activating transmission constraints. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This SCR addresses the financial harm to DC Tie exporters when DC Tie Load is curtailed but the DC Tie Load Zone price does not reflect the Shadow Price of the constraint due to DC Tie not counting toward the EMS 2% constraint activation criterion. These same constraints are activated in the Day-Ahead Market (DAM) – this is also a source of inconsistency between DAM and Real-Time Market (RTM) pricing outcomes. Thus, if an exporter procures energy in DAM for export and in RTM the export is curtailed due to a constraint that only has the DC Tie meeting the 2% constraint activation criterion, the DC Tie Load Zone price in RTM will likely be lower than in DAM and the exporter will not only lose any profits from the export but also incur a loss by having to sell energy bought in DAM at a lower price in RTM. Additionally, the relatively low DC Tie Load Zone RTM price does not reflect the local scarcity that resulted in DC Tie Load curtailment. This SCR corrects these issues by changing the EMS system to count DC Ties toward the 2% constraint activation criterion. |  |
| PRS Decision |  | On 5/15/20, PRS voted unanimously via email to recommend approval of SCR810 as submitted. All Market Segments participated in the email vote. |  |
| Summary of PRS Discussion |  | On 5/15/20, there was no discussion. |  |

| Sponsor |  |
| --- | --- |
| Name | Shams Siddiqi |
| E-mail Address | shams@crescentpower.net |
| Company | Rainbow Energy Marketing Corporation |
| Phone Number | 512-619-3532 |
| Cell Number | 512-619-3532 |
| Market Segment | Independent Power Marketer (IPM) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Brittney Albracht |
| E-Mail Address | Brittney.Albracht@ercot.com |
| Phone Number | 512-225-7027 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| WMS 050820 | Endorsed SCR810 as submitted |

Market Rules Notes

None

Proposed System Change

Issue:

Currently, transmission constraints are only activated if there is a Generation Resource with a Shift Factor with an absolute value greater than 2% to the constraint.  In the EMS Transmission Constraint Manager (TCM), all units that are not dispatched by Security-Constrained Economic Dispatch (SCED), including DC Ties, have a flag to indicate to the ERCOT Operator that that unit does not count toward the 2% activation criterion.  In order to include DC Ties along with Generation Resources in the 2% activation criterion, logic needs to be added to the EMS to remove the flag that indicates to the operator that the unit does not count toward the 2% activation criterion.

Resolution:

Logic will be added to ERCOT’s EMS system to remove the flag that indicates to the ERCOT Operator that a unit representing a DC Tie does not count toward the 2% activation criterion.