| NPRR Number | 996 | NPRR Title | Alignment of Hub Bus Names Between Protocols and ERCOT Model |
| --- | --- | --- | --- |
| Date of Decision |  | June 11, 2020 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Nodal Protocol Sections Requiring Revision |  | 3.5.2.1, North 345 kV Hub (North 345) 3.5.2.3, Houston 345 kV Hub (Houston 345) 3.5.2.4, West 345 kV Hub (West 345) |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) aligns the Hub Bus names within Protocols with the substation names within the ERCOT model. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This NPRR clarifies Hub Bus names between Protocols and the ERCOT model to help avoid confusion. The name change will not impact the Hub Bus treatment. |  |
| PRS Decision |  | On 6/11/20, PRS voted unanimously via roll call to recommend approval of NPRR996 as submitted. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 6/11/20, there was no discussion. |  |

| Sponsor |  |
| --- | --- |
| Name | Alfredo Moreno |
| E-mail Address | Alfredo.Moreno@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6977 |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jordan Troublefield |
| E-Mail Address | jordan.troublefield@ercot.com |
| Phone Number | 512-248-6521 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None |  |

Market Rules Notes

Please note the following NPRR(s) also propose revisions to the following section(s):

- NPRR1007, RTC – NP 3: Management Activities for the ERCOT System

  - Section 3.5.2.1

  - Section 3.5.2.3

  - Section 3.5.2.4

Proposed Protocol Language Revision

#### 3.5.2.1	North 345 kV Hub (North 345)

(1)	The North 345 kV Hub is composed of the following Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | ANASW | 345 | NORTH |
| 2 | CN345 | 345 | NORTH |
| 3 | WLSH | 345 | NORTH |
| 4 | FMRVL | 345 | NORTH |
| 5 | LPCCS | 345 | NORTH |
| 6 | MNSES | 345 | NORTH |
| 7 | PRSSW | 345 | NORTH |
| 8 | SSPSW | 345 | NORTH |
| 9 | VLSES | 345 | NORTH |
| 10 | ALNSW | 345 | NORTH |
| 11 | ALLNC | 345 | NORTH |
| 12 | BNDVS | 345 | NORTH |
| 13 | BNBSW | 345 | NORTH |
| 14 | BBSES | 345 | NORTH |
| 15 | BOSQUESW | 345 | NORTH |
| 16 | CDHSW | 345 | NORTH |
| 17 | CNTRY | 345 | NORTH |
| 18 | CRLNW | 345 | NORTH |
| 19 | CMNSW | 345 | NORTH |
| 20 | CNRSW | 345 | NORTH |
| 21 | CRTLD | 345 | NORTH |
| 22 | DCSES | 345 | NORTH |
| 23 | EMSES | 345 | NORTH |
| 24 | ELKTN | 345 | NORTH |
| 25 | ELMOT | 345 | NORTH |
| 26 | EVRSW | 345 | NORTH |
| 27 | KWASS | 345 | NORTH |
| 28 | FGRSW | 345 | NORTH |
| 29 | FORSW | 345 | NORTH |
| 30 | FRNYPP | 345 | NORTH |
| 31 | GIBCRK | 345 | NORTH |
| 32 | HKBRY | 345 | NORTH |
| 33 | VLYRN | 345 | NORTH |
| 34 | JEWET | 345 | NORTH |
| 35 | KNEDL | 345 | NORTH |
| 36 | KLNSW | 345 | NORTH |
| 37 | LCSES | 345 | NORTH |
| 38 | LIGSW | 345 | NORTH |
| 39 | LEG | 345 | NORTH |
| 40 | LFKSW | 345 | NORTH |
| 41 | LWSSW | 345 | NORTH |
| 42 | MLSES | 345 | NORTH |
| 43 | MCCREE | 345 | NORTH |
| 44 | MDANP | 345 | NORTH |
| 45 | ENTPR | 345 | NORTH |
| 46 | NCDSE | 345 | NORTH |
| 47 | NORSW | 345 | NORTH |
| 48 | NUCOR | 345 | NORTH |
| 49 | PKRSW | 345 | NORTH |
| 50 | KMCHI | 345 | NORTH |
| 51 | PTENN | 345 | NORTH |
| 52 | RENSW | 345 | NORTH |
| 53 | RCHBR | 345 | NORTH |
| 54 | RNKSW | 345 | NORTH |
| 55 | RKCRK | 345 | NORTH |
| 56 | RYSSW | 345 | NORTH |
| 57 | SGVSW | 345 | NORTH |
| 58 | SHBSW | 345 | NORTH |
| 59 | SHRSW | 345 | NORTH |
| 60 | SCSES | 345 | NORTH |
| 61 | SYCRK | 345 | NORTH |
| 62 | THSES | 345 | NORTH |
| 63 | TMPSW | 345 | NORTH |
| 64 | TNP_ONE | 345 | NORTH |
| 65 | TRCNR | 345 | NORTH |
| 66 | TRSES | 345 | NORTH |
| 67 | TOKSW | 345 | NORTH |
| 68 | VENSW | 345 | NORTH |
| 69 | WLVEE | 345 | NORTH |
| 70 | W_DENT | 345 | NORTH |
| 71 | WTRML | 345 | NORTH |
| 72 | WCSWS | 345 | NORTH |
| 73 | WEBBS | 345 | NORTH |
| 74 | WHTNY | 345 | NORTH |
| 75 | WCPP | 345 | NORTH |

(2)	The North 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the Day-Ahead Market (DAM) in the Day-Ahead and is the simple average of the time-weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP North345 =	DASL – Σc(DAHUBSF North345, c * DASP c),

if HBBC North345≠0

DASPP North345 =	DASPP ERCOT345Bus, if HBBC North345=0

Where:

DAHUBSF North345, c	=	Σhb(HUBDF hb, North345, c * DAHBSF hb, North345, c)

DAHBSF hb, North345, c	=	Σpb(HBDF pb, hb, North345, c * DASF pb, hb, North345, c)

HUBDF hb, North345, c	=	IF(HB North345, c=0, 0, 1 / HB North345, c)

HBDF pb, hb, North345, c	=	IF(PB hb, North345, c=0, 0, 1 / PB hb, North345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP North345 | $/MWh | Day-Ahead Settlement Point PriceThe DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System LambdaThe DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraintThe DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF North345,c | none | Day-Ahead Shift Factor of the Hub The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,North345,c | none | Day-Ahead Shift Factor of the Hub BusThe DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,North345,c | none | Day-Ahead Shift Factor of the power flow busThe DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, North345,c | none | Hub Distribution Factor per Hub Bus in a constraintThe distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, North345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraintThe distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, North345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC North345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB North345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP North345	=	Max [-$251, (RTRSVPOR + RTRDP +

(HUBDF hb, North345 * ((RTHBP hb, North345, y *

TLMP y) / (TLMP y))))], if HB North345≠0

RTSPP North345	=	RTSPP ERCOT345Bus, if HB North345=0

Where:

RTRSVPOR 		=	(RNWF y * RTORPA y)

RTRDP                       =           (RNWF y * RTORDPA y)

RNWF y		=	TLMP y / TLMP y

RTHBP hb, North345, y	=	(HBDF b, hb, North345 * RTLMP b, hb, North345, y)

HUBDF hb, North345	=	IF(HB North345=0, 0, 1 / HB North345)

HBDF b, hb, North345	=	IF(B hb, North345=0, 0, 1 / B hb, North345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP North345 | $/MWh | Real-Time Settlement Point PriceThe Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RTHBP hb, North345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per Security-Constrained Economic Dispatch (SCED) intervalThe Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RTRSVPOR | $/MWh | Real-Time Reserve Price for On-Line ReservesThe Real-Time Reserve Price for On-Line Reserves for the 15-minute Settlement Interval. |
| RTORPA y | $/MWh | Real-Time On-Line Reserve Price Adder per intervalThe Real-Time price adder for On-Line Reserves for the SCED interval y. |
| RTRDP | $/MWh | Real-Time On-Line Reliability Deployment PriceThe Real-Time price for the 15-minute Settlement Interval, reflecting the impact of reliability deployments on energy prices that are calculated from the Real-Time On-Line Reliability Deployment Price Adder. |
| RTORDPA y | $/MWh | Real-Time On-Line Reliability Deployment Price AdderThe Real-Time price adder that captures the impact of reliability deployments on energy prices for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTLMP b, hb, North345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per intervalThe Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the 15-minute Settlement Interval |
| HUBDF hb, North345 | none | Hub Distribution Factor per Hub BusThe distribution factor of Hub Bus hb. |
| HBDF b, hb, North345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub BusThe distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, North345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB North345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |

#### 3.5.2.3	Houston 345 kV Hub (Houston 345)

(1)	The Houston 345 kV Hub is composed of the following listed Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | ADK | 345 | HOUSTON |
| 2 | BI | 345 | HOUSTON |
| 3 | CBY | 345 | HOUSTON |
| 4 | CTR | 345 | HOUSTON |
| 5 | CHB | 345 | HOUSTON |
| 6 | DPW | 345 | HOUSTON |
| 7 | DOW | 345 | HOUSTON |
| 8 | RNS | 345 | HOUSTON |
| 9 | GBY | 345 | HOUSTON |
| 10 | JN | 345 | HOUSTON |
| 11 | KG | 345 | HOUSTON |
| 12 | KDL | 345 | HOUSTON |
| 13 | NB | 345 | HOUSTON |
| 14 | OB | 345 | HOUSTON |
| 15 | PHR | 345 | HOUSTON |
| 16 | SDN | 345 | HOUSTON |
| 17 | SMITHERS | 345 | HOUSTON |
| 18 | THW | 345 | HOUSTON |
| 19 | WAP | 345 | HOUSTON |
| 20 | WO | 345 | HOUSTON |

(2)	The Houston 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time-weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP Houston345 =	DASL – Σc(DAHUBSF Houston345, c * DASP c),

if HBBC Houston345≠0

DASPP Houston345 =	DASPP ERCOT345Bus, if HBBC Houston345=0

Where:

DAHUBSF Houston345, c	=	Σhb(HUBDF hb, Houston345, c * DAHBSF hb, Houston345, c)

DAHBSF hb, Houston345, c	=	Σpb(HBDF pb, hb, Houston345, c * DASF pb, hb, Houston345, c)

HUBDF hb, Houston345, c	=	IF(HB Houston345, c=0, 0, 1 / HB Houston345, c)

HBDF pb, hb, Houston345, c	=	IF(PB hb, Houston345, c=0, 0, 1 / PB hb, Houston345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP Houston345 | $/MWh | Day-Ahead Settlement Point PriceThe DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System LambdaThe DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraintThe DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF Houston345,c | none | Day-Ahead Shift Factor of the Hub The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,Houston345,c | none | Day-Ahead Shift Factor of the Hub BusThe DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,Houston345,c | none | Day-Ahead Shift Factor of the power flow busThe DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, Houston345,c | none | Hub Distribution Factor per Hub Bus in a constraintThe distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, Houston345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraintThe distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, Houston345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC Houston345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB Houston345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP Houston345	   =	Max [-$251, (RTRSVPOR + RTRDP +

(HUBDF hb, Houston345 * ((RTHBP hb, Houston345, y *

TLMP y) / (TLMP y))))], if HB Houston345≠0

RTSPP Houston345   =	RTSPP ERCOT345Bus, if HB Houston345=0

Where:

RTRSVPOR 		=	(RNWF y * RTORPA y)

RTRDP                       		=           (RNWF y * RTORDPA y)

RNWF y		=	TLMP y / TLMP y

RTHBP hb, Houston345, y	=	(HBDF b, hb, Houston345 * RTLMP b, hb, Houston345, y)

HUBDF hb, Houston345	=	IF(HB Houston345=0, 0, 1 / HB Houston345)

HBDF b, hb, Houston345	=	IF(B hb, Houston345=0, 0, 1 / B hb, Houston345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP Houston345 | $/MWh | Real-Time Settlement Point PriceThe Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RTHBP hb, Houston345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED intervalThe Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RTRSVPOR | $/MWh | Real-Time Reserve Price for On-Line ReservesThe Real-Time Reserve Price for On-Line Reserves for the 15-minute Settlement Interval. |
| RTORPA y | $/MWh | Real-Time On-Line Reserve Price Adder per intervalThe Real-Time On-Line Reserve Price Adder for the SCED interval y. |
| RTRDP | $/MWh | Real-Time On-Line Reliability Deployment PriceThe Real-Time price for the 15-minute Settlement Interval, reflecting the impact of reliability deployments on energy prices that are calculated from the Real-Time On-Line Reliability Deployment Price Adder. |
| RTORDPA y | $/MWh | Real-Time On-Line Reliability Deployment Price AdderThe Real-Time price adder that captures the impact of reliability deployments on energy prices for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTLMP b, hb, Houston345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per intervalThe Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the 15-minute Settlement Interval |
| HUBDF hb, Houston345 | none | Hub Distribution Factor per Hub BusThe distribution factor of Hub Bus hb. |
| HBDF b, hb, Houston345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub BusThe distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, Houston345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB Houston345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |

#### 3.5.2.4	West 345 kV Hub (West 345)

(1)	The West 345 kV Hub is composed of the following listed Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | MULBERRY | 345 | WEST |
| 2 | BOMSW | 345 | WEST |
| 3 | OECCS | 345 | WEST |
| 4 | BITTCR | 345 | WEST |
| 5 | FSHSW | 345 | WEST |
| 6 | FLCNS | 345 | WEST |
| 7 | GRSES | 345 | WEST |
| 8 | JCKSW | 345 | WEST |
| 9 | MDLNE | 345 | WEST |
| 10 | MOSSW | 345 | WEST |
| 11 | MGSES | 345 | WEST |
| 12 | DCTM | 345 | WEST |
| 13 | ODEHV | 345 | WEST |
| 14 | OKLA | 345 | WEST |
| 15 | REDCREEK | 345 | WEST |
| 16 | SWESW | 345 | WEST |
| 17 | TWINBU | 345 | WEST |

(2)	The West 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP West345 =		DASL – Σc(DAHUBSF West345, c * DASP c),

if HBBC West345≠0

DASPP West345 =		DASPP ERCOT345Bus, if HBBC West345=0

Where:

DAHUBSF West345, c	=	Σhb(HUBDF hb, West345, c * DAHBSF hb, West345, c)

DAHBSF hb, West345, c	=	Σpb(HBDF pb, hb, West345, c * DASF pb, hb, West345, c)

HUBDF hb, West345, c	=	IF(HB West345, c=0, 0, 1 / HB West345, c)

HBDF pb, hb, West345, c	=	IF(PB hb, West345, c=0, 0, 1 / PB hb, West345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP West345 | $/MWh | Day-Ahead Settlement Point PriceThe DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System LambdaThe DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraintThe DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF West345,c | none | Day-Ahead Shift Factor of the Hub The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,West345,c | none | Day-Ahead Shift Factor of the Hub BusThe DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,West345,c | none | Day-Ahead Shift Factor of the power flow busThe DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, West345,c | none | Hub Distribution Factor per Hub Bus in a constraintThe distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, West345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraintThe distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, West345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC West345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB West345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP West345	=	Max [-$251, (RTRSVPOR + RTRDP +

(HUBDF hb, West345 * ((RTHBP hb, West345, y * TLMP y) / (TLMP y))))], if HB West345≠0

RTSPP West345	=	RTSPP ERCOT345Bus, if HB West345=0

Where:

RTRSVPOR 		=	(RNWF y * RTORPA y)

RTRDP                      		=           (RNWF y * RTORDPA y)

RNWF y		=	TLMP y / TLMP y

RTHBP hb, West345, y	=	(HBDF b, hb, West345 * RTLMP b, hb, West345, y)

HUBDF hb, West345	=	IF(HB West345=0, 0, 1 / HB West345)

HBDF b, hb, West345	=	IF(B hb, West345=0, 0, 1 / B hb, West345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP West345 | $/MWh | Real-Time Settlement Point PriceThe Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RTRSVPOR | $/MWh | Real-Time Reserve Price for On-Line ReservesThe Real-Time Reserve Price for On-Line Reserves for the 15-minute Settlement Interval. |
| RTORPA y | $/MWh | Real-Time On-Line Reserve Price Adder per intervalThe Real-Time On-Line Reserve Price Adder for the SCED interval y. |
| RTRDP | $/MWh | Real-Time On-Line Reliability Deployment PriceThe Real-Time price for the 15-minute Settlement Interval, reflecting the impact of reliability deployments on energy prices that are calculated from the Real-Time On-Line Reliability Deployment Price Adder. |
| RTORDPA y | $/MWh | Real-Time On-Line Reliability Deployment Price AdderThe Real-Time price adder that captures the impact of reliability deployments on energy prices for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTHBP hb, West345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED intervalThe Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RTLMP b, hb, West345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per intervalThe Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| HUBDF hb, West345 | none | Hub Distribution Factor per Hub BusThe distribution factor of Hub Bus hb. |
| HBDF b, hb, West345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub BusThe distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, West345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB West345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |