| NOGRR Number | 205 | NOGRR Title | Clarification to Grey-boxed Language in Section 4.8.1 |
| --- | --- | --- | --- |
| Date of Decision |  | June 24, 2020 |  |
| Action |  | Approved |  |
| Timeline |  | Normal |  |
| Effective Date |  | July 1, 2020 |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Nodal Operating Guide Sections Requiring Revision |  | 4.8.1, Responsive Reserve Service Manual Deployment |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Operating Guide Revision Request (NOGRR) clarifies grey-boxed language in Section 4.8.1 in order to maintain consistency with revisions adopted via NOGRR197, Align Responsive Reserve (RRS) Manual Deployment Requirements with Current Practice, following the incorporation of NOGRR191, Related to NPRR939, Modification to Load Resources Providing RRS to Maintain Minimum PRC on Generators During Scarcity Conditions, into the 11/1/19 Nodal Operating Guide. This NOGRR also corrects an error in the 9/26/19 ERCOT comments to NOGRR191, which inadvertently changed the language. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This NOGRR is necessary in order to maintain alignment of language in the Nodal Operating Guide with current RRS deployment practices upon implementation of NOGRR191. |  |
| ROS Decision |  | On 2/6/20, ROS unanimously voted to recommend approval of NOGRR205 as amended by the 2/4/20 ERCOT comments. All Market Segments were present for the vote. On 5/11/20 ROS unanimously voted via email to endorse and forward to TAC the 2/6/20 ROS Report and the Impact Analysis for NOGRR205. All Market Segments participated in the email vote. |  |
| Summary of ROS Discussion |  | On 2/6/20, ERCOT Staff highlighted the clarifications provided in NOGRR205 and the 2/4/20 ERCOT comments. On 5/11/20, there was no discussion. |  |
| TAC Decision |  | On 6/24/20, TAC unanimously voted via roll call to approve NOGRR205 as recommended by ROS in the 5/11/20 ROS Report. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 6/24/20, there was no discussion. |  |
| ERCOT Opinion |  | ERCOT supports approval of NOGRR205. |  |

| Sponsor |  |
| --- | --- |
| Name | Nitika Mago |
| E-mail Address | nitika.mago@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6601 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Phillip Bracy |
| E-Mail Address | Phillip.Bracy@ercot.com |
| Phone Number | 512-248-6917 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 020420 | Additional revision to correct an inadvertent error found in the 9/26/19 ERCOT comments to NOGRR191 |

Market Rules Notes

None

Proposed Guide Language Revision

4.8.1	Responsive Reserve Service Manual Deployment

(1)	RRS for capacity may be manually deployed (HASL released) when the system approaches scarcity conditions so that the capacity reserved behind HASL will be released to Security-Constrained Economic Dispatch (SCED). The capacity may be released under any of the following conditions:

(a)	When HASL – (Gen + 5 minute load ramp) <= 200 MW, deploy a portion of the available RRS capacity from Generation Resources and Controllable Load Resources after all the available Non-Spinning Reserve (Non-Spin) service has been deployed;

(b)	Additional RRS capacity from Generation Resources and Controllable Load Resources may be deployed, as available, when HASL – (Gen + 5 minute load ramp) <= 200 MW and Resources have responded to any earlier deployments; or

(c)	When PRC <= 2,000 MW, release all remaining RRS capacity from Generation Resources and Controllable Load Resources after all the available Non-Spin has been deployed.

[NOGRR187 and NOGRR191: Replace applicable portions paragraph (1) above with the following upon system implementation of NPRR863 or NPRR939:] (1) RRS for capacity may be manually deployed (HASL released) when the system approaches scarcity conditions so that the capacity reserved behind HASL will be released to SCED. (a) When HASL – (Gen + 5 minute load ramp) <= 200 MW, ERCOT may deploy a portion of the available RRS capacity from Generation Resources and Controllable Load Resources after all the available ECRS (dispatchable by SCED) and Non-Spinning Reserve (Non-Spin) service has been deployed and Resources have responded to any earlier deployments. (b) When HSL – (Gen + 5 minute load ramp) <= 500 MW, ERCOT may deploy Load Resources controlled by high-set under-frequency relays providing RRS.