| NOGRR Number | 214 | NOGRR Title | Geomagnetic Disturbance (GMD) Measurement Data Reporting |
| --- | --- | --- | --- |

| Date | July 7, 2020 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Amjed Kandah on behalf of the Planning Geomagnetic Disturbance Task Force (PGDTF) |
| E-mail Address | Amjed.kandah@oncor.com |
| Company | Oncor |
| Phone Number | 214-743-6681 |
| Cell Number |  |
| Market Segment | Not applicable |

Comments

PGDTF submits these comments to Nodal Operating Guide Revision Request (NOGRR) 214, Geomagnetic Disturbance (GMD) Measurement Data Reporting, to incorporate revisions discussed at the July 7, 2020 PGDTF meeting.

Revised Cover Page Language

None

Revised Proposed Guide Language

6.1.1	Introduction

(1)	Disturbance monitoring is necessary to:

(a)	Determine performance of the ERCOT System;

(b)	Determine effectiveness of protective relaying systems;

(c)	Verify ERCOT System models;

(d)	Determine causes of ERCOT System disturbances (trips, faults, and protective relay system actions); and

(e)	Meet the requirements of North American Reliability Corporation (NERC) Reliability Standards.

(2)	To ensure that adequate data is available for these activities, the disturbance monitoring requirements and procedures discussed in these Operating Guides have been established by ERCOT for the following:

(a)	Fault recording, sequence of events recording, and dynamic disturbance recording equipment owners in the ERCOT System; and

(b)	Transmission Service Providers (TSPs) and Resource Entities with equipment for recording Geomagnetic Disturbance (GMD) measurement data, including Geomagnetically-Induced Current (GIC) monitors and/or magnetometers for recording geomagnetic field data, installed at their facilities.

6.1.5.1	Geomagnetic Disturbance (GMD) Measurement Data Processes

(1)	When specifically requested by ERCOT, TSPs and Resource Entities shall provide a complete list of GMD measurement equipment installed at their facilities within 30 days.

(2)	When specifically requested by ERCOT, TSPs and Resource Entities with GMD measurement equipment installed at their facilities shall provide GMD measurement data for events meeting the reporting criteria set forth in the NERC Geomagnetic Disturbance Data System Data Reporting Instructions, within 60 days.

(3)	ERCOT may, at the request of TSPs, post GMD measurement data obtained from TSPs, Resource Entities, or publicly available sources to the Market Information System (MIS) Certified area for TSPs.