ERCOT ALREADY HAS SEVERE LIMITS TO INTER-REGIONAL SHIPMENT OF POWER

Currently, there are over a dozen Generic Transmission Constraints (GTCs) in ERCOT.  GTCs require extraordinary effort to ensure that they are not violated.  Major generator and/or transmission outages will likely occur immediately once a GTC is violated.  The bulk of the transmission constraints in ERCOT are thermal constraints.  Unlike GTCs, thermal constraints can be violated for as much as 30 minutes before major generator and/or transmission outages will occur thus providing sufficient time for operator action to relieve the overload.

The latest GTC to be established consists of a 5-circuit interface and is depicted by the red line on the map below.  When activated this GTC has the prospect of effecting well over 10,000 MW of generation.  When all West Texas transmission is in service, the flow on the interface must be kept below 4,178 MW; however, if there are nearby transmission lines out of service, the limit could go down to 2,393 MW.  Recent, monthly peak flows on the interface have all been above 3,000 MW reaching 3,600 MW in May 2020.

So, how bad could it get?  Unfortunately, there is a much larger GTC interface limit consisting of 16 circuits that is looming its ugly ahead.  The purple line on the map above depicts this GTC; if it were to be established, this GTC would affect over 28,000 MW of generation virtually splitting ERCOT in half.  This GTC could be put in place as soon as December 2020 with devastating effects on service to ERCOT customers and West Texas generator revenues.  The only solution is more transmission!