ERCOT INTER REGIONAL TRANSMISSIOIN NEEDED NOW

In the Long Term System Assessment (LTSA) study completed in 2018 ERCOT determined that 10 major transmission lines would be needed, 9 of them by 2028 in order to accommodate their estimate of various scenario renewable generation additions, the Business as Usual scenario renewable fleet totaled 45,026 by 2028.  That is a tall order as we stand here only 8 years away from 2028.

| Table ES. 1: Transmission Upgrades and Additions Index |  |  |
| --- | --- | --- |
| Projects |  | In service date |
| 1 | Oklaunion to Jacksboro new 345-kV line | 2028 |
| 2 | Odessa to Bearkat new 345-kV line | 2028 |
| 3 | Lubbock Loop (North to New Oliver new 345-kV line and Long Draw to Grassland 345-kV line upgrade) | 2028 |
| 4 | Northwest Austin Metro new 345-kV line and 345/138-kV transformer | 2028 |
| 5 | Northwest Dallas-Fort Worth new 345-kV line | 2028 |
| 6 | Faraday to Morgan Creek new 345-kV line | 2028 |
| 7 | Long Draw to Dermott new 345-kV line | 2028 |
| 8 | West Texas to San Antonio new 345-kV line | 2028 |
| 9 | Bergheim 345/138-kV transformer upgrade | 2028 |
| 10 | Odessa to Moss new 345-kV line | 2033 |

So where do we stand now? Not a single transmission line on the list has been submitted for study, much less approval.  In addition, time has moved on.  The latest Generator Interconnect Status Report (GISR) lists over 45,777 MW of renewable capacity to be in place by 2022, only two years from now; that is 700 MW more than the 45,026 MW that LTSA said might be in place by 2028 and for which the LTSA opined that the transmission listed in the table above would be needed.  The 45,777 MW to be in place by 2022 is either on the ground today or has posted millions of dollars of security; these are not speculative generation additions.

The key problem is that we have an extremely high hurdle to getting transmission approved in ERCOT if it is not needed for reliability purposes.  The current rules state that a transmission problem that can be cured by backing down generation is not a reliability issue.  Any transmission that is needed to increase production from lower cost resources needs to have an economic justification.  That is getting very hard since the really high cost generation has been pushed out of the market or is seldom used.  Add to that the lower cost of natural gas and it is much more difficult to make the economics work out.

This catch 22 is further exacerbated by the fact that as more wind and solar power replace high cost generation, there is less high cost generation to replace.  Taken to the extreme, once all generation in ERCOT comes from wind and solar power; how will there be any basis for economic justification for transmission additions?

The real unfortunate possibility is that developers will rethink their plans to build generation.