| NPRR Number | 1045 | NPRR Title | Transmission Operator Definition and Designation |
| --- | --- | --- | --- |
| Date of Decision |  | October 15, 2020 |  |
| Action |  | Tabled |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms and Abbreviations 6.5.7.8, Dispatch Procedures 16.19, Designation of Transmission Operators (new) |  |
| Related Documents Requiring Revision/Related Revision Requests |  | Nodal Operating Guide Revision Request (NOGRR) 219, Related to NPRR1045, Transmission Operator Definition and Designation Transmission Operator Designation Form |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) moves the definition of Transmission Operator (TO) from the Nodal Operating Guide to Section 2.1, Definitions, of the ERCOT Protocols, revises this definition, and adds new Section 16.19, Designation of Transmission Operators, that clarifies the designation process and basic qualifications for TOs. This NPRR also replaces references to “designated agent” in Section 6.5.7.8, Dispatch Procedures, with the term “TO.” |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | The definition of TO, obligation to designate a TO, and the process and requirements relating to the designation of a TO are more appropriately located in the ERCOT Protocols, which address responsibilities relating to registration. The processes and requirements for TO designation were not clearly described in the Nodal Operating Guide language. |  |
| Credit Work Group Review |  | To be determined |  |
| PRS Decision |  | On 10/15/20, PRS unanimously voted via roll call to table NPRR1045 and refer the issue to ROS. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 10/15/20, ERCOT Staff provided an overview of NPRR1045. Market Participants requested additional review by ROS, and noted that some Market Participants may require time to review current contracts prior to implementation of this NPRR. |  |

| Sponsor |  |
| --- | --- |
| Name | Douglas Fohn |
| E-mail Address | Douglas.Fohn@ercot.com |
| Company | ERCOT |
| Phone Number | 512-275-7447 |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Phillip Bracy |
| E-Mail Address | Phillip.Bracy@ercot.com |
| Phone Number | 512-248-6917 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 101420 | Provided administrative clarifications inadvertently overlooked in the original submission of NPRR1045 |

Market Rules Notes

None

Proposed Protocol Language Revision

## 2.1	DEFINITIONS

Transmission Operator (TO)

A Transmission and/or Distribution Service Provider (TDSP) designated by itself or another TDSP or for purposes of communicating with ERCOT and taking action to preserve reliability of a particular portion of the ERCOT System, as provided in the ERCOT Protocols or Other Binding Documents.

[NPRRXXX: Insert the following definition “Transmission Operator (TO)” upon system implementation of NPRR857:] Transmission Operator (TO) A Transmission and/or Distribution Service Provider (TDSP) or Direct Current Tie Operator (DCTO) designated by itself or another TDSP or DCTO for purposes of communicating with ERCOT and taking action to preserve reliability of a particular portion of the ERCOT System, as provided in the ERCOT Protocols or Other Binding Documents.

## 2.2	ACRONYMS AND ABBREVIATIONS

TO	Transmission Operator

UFLS	Under-Frequency Load Shed

UVLS	Under-Voltage Load Shed

6.5.7.8	Dispatch Procedures

(1)	ERCOT shall issue all Resource Dispatch Instructions to the QSE that represents the affected Resource.  ERCOT and QSEs are responsible for complying with Dispatch Instructions as prescribed in the Nodal Operating Guides.  A QSE may provide a Resource Status of ONTEST for a Generation Resource not providing Ancillary Services to indicate that the Resource is currently undergoing unit testing and is blocked from SCED Dispatch.  A QSE may provide a Resource Status of STARTUP for a Generation Resource not providing Ancillary Services to indicate that the Resource is currently undergoing a start-up sequence which requires manual control below or above its telemetered LSL to stabilize the Resource prior to its availability for SCED Dispatch.  Generation Resources with a Resource Status of ONTEST will be provided a Base Point equal to the net real power telemetry at the time of the SCED execution.  ERCOT may not issue Dispatch Instructions to the QSE for Generation Resources with a Resource Status of ONTEST except:

(a)	For Dispatch Instructions that are a part of testing; or

(b)	During conditions when the Resource is the only alternative for solving a transmission constraint; or

(c)	During Force Majeure Events that threaten the reliability of the ERCOT System.

(2)	Each QSE shall immediately forward any valid Dispatch Instruction to the appropriate Resource or group of Resources or identify a reason for non-compliance with the Dispatch Instruction to ERCOT in accordance with Section 6.5.7.9, Compliance with Dispatch Instructions.

(3)	If ERCOT believes that a Resource has inadequately responded to a Dispatch Instruction, ERCOT shall notify the QSE representing the Resource as soon as practicable.

(4)	ERCOT shall record all voice conversations that occur in the communication of Verbal Dispatch Instructions (VDIs).

(5)	By mutual agreement of the TSP and ERCOT, Dispatch Instructions to the TSP may be provided to the TSP’s Transmission Operator (TO).  In that case, issuance of the Dispatch Instruction to the TO is considered issuance to the TSP, and the TSP must comply with the Dispatch Instruction exactly as if it had been issued directly to the TSP, whether or not the TO accurately conveys the Dispatch Instruction to the TSP.

[NPRR857: Replace paragraph (5) above with the following upon system implementation:] (5) By mutual agreement of the TSP, DCTO, and ERCOT, Dispatch Instructions to the TSP or DCTO may be provided to the TSP’s or DCTO’s Transmission Operator (TO). In that case, issuance of the Dispatch Instruction to the TO is considered issuance to the TSP or DCTO, and the TSP or DCTO must comply with the Dispatch Instruction exactly as if it had been issued directly to the TSP or DCTO, whether or not the TO accurately conveys the Dispatch Instruction to the TSP or DCTO.

(6)	ERCOT shall direct VDIs to the Master QSE of a Generation Resource that has been split to function as two or more Split Generation Resources as deemed necessary by ERCOT to effectuate actions for the total Generation Resource for instances in which electronic instructions are not feasible.

16.19	Designation of Transmission Operators

(1)	Each Transmission and/or Distribution Service Provider (TDSP) and Direct Current Tie Operator (DCTO) shall either register as a Transmission Operator (TO) or designate another TDSP as its TO.  A TDSP or DCTO shall designate another Entity as its TO only with the written consent of that other Entity.  A TO shall have full authority to act on behalf of the designating TDSP or DCTO in the performance of all TO responsibilities provided in the ERCOT Protocols or Other Binding Documents, including effectuating any Load-shed that may be necessary during any Energy Emergency Alert (EEA) event, Under-Frequency Load Shed (UFLS) event, or Under-Voltage Load Shed (UVLS) event.

(2)	To qualify for designation as a TO, the TDSP or DCTO shall be connected to the ERCOT Wide Area Network (WAN) and maintain 24x7 operations and Hotline communications with ERCOT.

(3)	ERCOT shall post a Transmission Operator Designation Form on the ERCOT website.  A TDSP or DCTO may designate a TO or make any change to its designation, including revoking a TO designation, by properly completing the Transmission Operator Designation Form and submitting it via email to ERCOT with a copy provided to the TDSP designated as TO.  Any such change shall be submitted to ERCOT within the time required for submitting physical changes to the Network Operations Model specified in paragraph (3) of Section 3.10.1, Time Line for Network Operations Model Changes.

(4)	The designation of a TO shall not be effective unless the Transmission Operator Designation Form has been signed by an officer or other Authorized Representative of the designating TDSP or DCTO and the TDSP designated as TO.

(5)	A TO may unilaterally resign from a designation as TO by submitting a properly completed Transmission Operator Designation Form to ERCOT reflecting the TO’s resignation of its TO status within the time required for submitting physical changes to the Network Operations Model specified in paragraph (3) of Section 3.10.1.  The resigning TO shall provide simultaneous notice of the resignation to the TDSP or DCTO it represents.

(6)	On the effective date of a TO’s resignation, the TDSP or DCTO previously represented by the resigning TO shall be deemed its own TO and shall assume all TO responsibilities unless and until it has properly designated another TDSP as its TO.