| NPRR Number | 1023 | NPRR Title | Change to CRR Repossession Process |
| --- | --- | --- | --- |
| Date |  | January 25, 2021 |  |
| Submitter’s Information |  |  |  |
| Name |  | Donald House |  |
| E-mail Address |  | Donald.House@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-4676 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

ERCOT submits these additional revisions to Nodal Protocol Revision Request (NPRR) 1023 to provide minor, non-substantive edits to the 1/7/21 ERCOT comments.

Market Rules Notes

None

Revised Cover Page Language

| Nodal Protocol Sections Requiring Revision | 7.5.3.1, Data Transparency 7.5.5.3, Auction Process 16.11.6.1.4, Repossession of CRRs by ERCOT 16.11.6.1.5, Declaration of Forfeit of CRRs 16.11.6.1.6, Revocation of a Market Participant’s Rights and Termination of Agreements 16.11.6.1.6, Liquidation of Repossessed or Forfeited CRRs (new) 16.11.7, Release of Market Participant’s Financial Security Requirement 16.11.8, Acceleration 16.11.8, Conversion of Letters of Credit and Surety Bonds to Cash Collateral (new) |
| --- | --- |

Revised Proposed Protocol Language

7.5.3.1	Data Transparency

(1)	Following each CRR Auction, ERCOT shall record and make available to each CRR Account Holder on the MIS Certified Area the following information for each CRR awarded in, sold in, or allocated before, the CRR Auction to the specific CRR Account Holder:

(a)	Unique identifier of each CRR;

(b)	Type of CRR (PTP Option, PTP Obligation, PTP Option with Refund, or PTP Obligation with Refund);

(c)	Clearing price and, if applicable, the PCRR pricing factor of each CRR;

(d)	The source and sink of each CRR;

(e)	The date and time-of-use block for which the CRR is effective; and

(f)	Total MW of each PTP pair of CRR, awarded, sold or allocated.

(2)	Following each CRR Auction, ERCOT shall post to the ERCOT website the following information for all outstanding or sold CRRs following this auction:

(a)	PTP Options and PTP Options with Refund – the source and sink, and total MWs;

(b)	PTP Obligations and PTP Obligations with Refund – the source and sink and total MWs;

(c)	The identities of the CRR Account Holders that sold, were awarded, or were allocated CRRs in or before the CRR Auction;

(d)	The clearing prices for each strip of CRR Auction bids and CRR Auction offers awarded in the CRR Auction;

(e)	The identity and post contingency flow of each binding directional element based on the CRR Network Model used in the CRR Auction;

(f)	All CRR Auction bids and CRR Auction offers, without identifying the name of the CRR Account Holder that submitted the bid or offer;

(g)	The clearing prices for each strip of CRRs bid or offered in the CRR Auction;

(h)	The Shadow Prices for each Settlement Point in the CRR Auction; and

(i)	The clearing prices for all outstanding CRRs that were previously awarded or allocated for the month(s) in the CRR Auction.

7.5.5.3	Auction Process

(1)	The CRR Auction must be a single-round, simultaneous auction for selling the CRRs available for all auction products.  ERCOT shall enter into the CRR Auction system a credit limit for each Counter-Party that has at least one CRR Account Holder.  A Counter-Party’s CRR Auction credit limit is equal to the lesser of the credit limit as determined in Section 16.11.4.6.1, Credit Requirements for CRR Auction Participation, or, if provided, the Counter-Party’s self-imposed CRR Auction credit limit for the CRR Monthly Auction or for a time-of-use within a CRR Auction held as part of a CRR Long-Term Auction Sequence.

(2)	Prior to the CRR Auction, ERCOT will conduct a two-part pre-auction screening process. First, if the Counter-Party’s CRR Auction credit limit is greater than that Counter-Party’s credit exposure as defined below using the CRR bid volumes rather than awarded volumes, then the Counter-Party’s CRR Auction credit limit will be ignored as the CRR Auction is solved.  Second, for each CRR Account Holder of a Counter-Party, if the CRR Account Holder’s self-imposed credit limit is greater than that CRR Account Holder’s credit exposure as defined below, then the CRR Account Holder’s self-imposed credit limit will be ignored as the CRR Auction is solved.

The calculated exposure for the pre-auction screening for each CRR Account Holder is the sum of the credit exposure for PTP Obligation bids, PTP Obligation offers, and PTP Option bids for that CRR Account Holder.  The calculated exposure for the pre-auction screening for each Counter-Party is the sum of the credit exposure for PTP Obligation bids, PTP Obligation offers, and PTP Option bids for that Counter-Party.  PTP Option offers have zero credit exposure.  Separately, for PTP Obligation bids, PTP Obligation offers, and PTP Option bids, for each source/sink Settlement Point combination, the credit exposure will use the bid price and MW quantity that produces the maximum credit exposure that could result from the CRR Auction for that source/sink Settlement Point combination.

(3)	The credit constraint for each Counter-Party is based on the following calculation:

ACR b 		= 	AOBLCR b  + AOPTCR b  - AOBLCRO b

Where:

AOBLCR b = 	∑m ∑h ∑j, k [(BOBLMW m, h,(j, k), b * (Max(0, BPOBL m, h,(j, k), b) – Min(0,A ci99, m, h,(j, k), b, EACP m, h,(j, k) )))]

AOPTCR b = 	∑m ∑h ∑j, k [(BOPTMW m, h,(j, k), b * BPOPT m, h,(j, k), b)]

AOBLCRO b = 	∑m ∑h ∑j, k (OOBLMW m, h,(j, k), b * Min(0, OPOBL m, h,(j, k), b))

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| ACR b | $ | Auction Credit Requirement—The auction credit requirement for a Counter-Party b. |
| AOBLCR b | $ | Auction PTP Obligation Credit Requirement—The auction credit requirement for all PTP Obligation bids submitted by a Counter-Party b for all Operating Days. |
| BOBLMW m, h, (j, k), b | MW | Awarded Bid PTP Obligation—The awarded bid PTP Obligation with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. |
| BPOBL m, h, (j, k), b | $/MW per hour | Bid Price for PTP Obligation—Bid Price for PTP Obligation with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. |
| A ci 99, m, h, (j, k), b | $/MW per hour | Path-Specific DAM-Based Adder—The path-specific DAM-based adder with the source j and sink k for the hour h, and month m submitted by a Counter-Party b; calculated as 99th percentile of the average rolling consecutive DAM settled price for the reference CRR source/sink over a period that represents a month for each product type (18 days for 5*16, 8 days for 2*16, 28 days for 7*8). The look-back period for DAM settled prices shall be the lesser of Nodal Market go-live to current time and current time minus three years. If historical Day-Ahead Settlement Point Prices (DASPPs) are not available for a Settlement Point for one or more Operating Days, ERCOT will designate a proxy Settlement Point for this purpose, and the DASPPs of the proxy Settlement Point of corresponding Operating Days are used. |
| EACP m, h, (j, k) | $/MW per hour | Effective Auction Clearing Price—The auction clearing price with the source j and sink k for the hour h, and month m. For each CRR PTP Obligation, this value is equal to the auction clearing price of an awarded CRR selected as follows: (a) Awarded CRRs with the source j and sink k containing hour h and operating month m are selected from the set of unexpired awarded PTP Obligations. If no awarded CRRs are found the EACP value is zero. (b) If (a) results in more than one awarded CRR, awarded CRRs with the most recent award date are selected. (c) If (b) results in more than one awarded CRR, then the awarded CRR with the lowest auction clearing price is selected. |
| AOBLCRO b | $ | Auction PTP Obligation Credit Requirement for Offers—The auction credit requirement for all PTP Obligation offers submitted by a Counter-Party b for all Operating Days. |
| OOBLMW m, h, (j, k), b | MW | Awarded Offer PTP Obligation—The awarded offer PTP Obligation with source j and sink k for the hour h, and month m submitted by a Counter-Party b. |
| OPOBL m, h, (j, k ), b | $/MW per hour | Offer Price for PTP Obligation—The offer price for PTP Obligation with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. |
| AOPTCR b | $ | Auction PTP Option Bid Credit Requirement—The auction credit requirement for all PTP Option bids submitted by a Counter-Party b. |
| BOPTMW m, h, (j, k),b | MW | Awarded Bid PTP Option—The awarded bid PTP Option with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. |
| BPOPT m, h, (j, k), b | $/MW per hour | Bid Price for PTP Option—The bid price for PTP Option with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. |
| b | none | A Counter-Party. |
| m | none | An operating month. |
| h | none | An Operating Hour. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| ci99 | none | 99th percentile confidence interval. |

(4)	ERCOT may review preliminary CRR Auction results to ensure that post auction collateral requirements are satisfied for all CRR Account Holders participating in the CRR Auction.  If it is practicable to rerun the applicable CRR Auction, and the post CRR Auction collateral requirements for a Counter-Party are not satisfied, ERCOT:

(a)	Shall promptly notify the Counter-Party of the amount by which its Financial Security must be increased and allow it until 1500 on the next Bank Business Day from the date on which ERCOT delivered Notification to increase the Financial Security.

(b)	If sufficient Financial Security is not received by 1500 on the next Bank Business Day, ERCOT shall void all of the Counter-Party’s bids and offers in the CRR Auction and rerun the CRR Auction without that Counter-Party’s activity.

(c)	ERCOT shall award CRRs in quantities truncated to the nearest tenth MW (0.1 MW).

(d)	The CRR clearing price is equal to the corresponding Shadow Price for that CRR product.

(e)	When a CRR Account Holder is awarded CRRs as a result of a CRR Auction, the CRRs do not become the property of the winning CRR Account Holder, and the CRRs may not be placed in their CRR accounts, until the required CRR Invoice has been paid.

(5)	ERCOT shall use a linear programming auction engine model for each CRR Auction that evaluates all CRR Auction bids and CRR Auction offers submitted, and selects a combination of CRR Auction bids and CRR Auction offers that:

(a)	Makes the solution simultaneously feasible within the limits of the ERCOT network capability over the auction term; and

(b)	Maximizes the objective function, which is equal to the total economic value (as expressed in the CRR Auction bids) of the awarded CRR Auction bids, less the total economic cost (as expressed in CRR Auction offers) of the awarded CRR Auction offers, while observing all applicable constraints.

(6)	The CRR Network Model must, to the extent practicable, reflect the continuous and post-contingency system operating limits and operational procedures (i.e., Remedial Action Schemes (RASs), Automatic Mitigation Plans (AMPs) and Remedial Action Plans (RAPs)) in the Network Operations Model used by ERCOT during Real-Time operations, as discussed below in Section 7.5.5.4, Simultaneous Feasibility Test.

(7)	Once a CRR Auction is complete, ERCOT shall archive and keep the CRR Auction system and all models used to finalize the CRR Auction results under ERCOT’s data retention policy as that policy applies to data that may be needed to resolve requests for billing adjustments under applicable billing adjustment procedures.

(8)	Once a CRR Auction is complete, ERCOT will make available on the MIS Certified Area each active CRR Account Holder’s credit exposure calculated within the CRR Auction process (as defined in paragraph (3) above).

16.11.6.1.4	Repossession of CRRs by ERCOT

(1)	ERCOT, at its sole discretion, may repossess CRRs held by a Market Participant in Default under an Agreement with ERCOT.  ERCOT shall effect that repossession by sending a written notice to the Market Participant of the repossession and by removing the Market Participant’s access to the repossessed CRRs.  The repossessed CRRs will be handled as specified in Section 16.11.6.1.6, Liquidation of Repossessed or Forfeited CRRs.

16.11.6.1.5	Declaration of Forfeit of CRRs

(1)	At ERCOT’s sole discretion, if it does not receive full payment on the due date of a CRR Auction Invoice, it may declare any of the CRR bids cleared and Pre-Assigned Congestion Revenue Rights (PCRRs) allocated to the Market Participant forfeited. ERCOT shall effect that forfeiture by sending a written Notice to the Market Participant of the forfeiture and by not delivering the CRRs or PCRRs to the Market Participant’s CRR account.  The forfeited CRRs or PCRRs will be liquidated as specified in Section 16.11.6.1.6, Liquidation of Repossessed or Forfeited CRRs.

(2)	ERCOT may also, at its sole discretion, honor any of the sell offers that were cleared in the CRR Auction made by a Market Participant who fails to fully pay the CRR Auction Invoice when due.  ERCOT shall offset net revenues due to the Market Participant from CRRs offered and cleared against amounts owed to ERCOT by the Market Participant.

16.11.6.1.6	Liquidation of Repossessed or Forfeited CRRs

(1)	If any repossessed or forfeited CRRs have the following characteristics, they will be irrevocably voided, the capacity will be dissolved, and they will not settle in the DAM beyond the effective date of the repossession or forfeiture nor be offered into any future CRR Auctions:

(a) 	PTP Options with Refund and PTP Obligations with Refund;

(b) 	Awarded PCRRs that have not yet been priced in a CRR Auction and paid for by the breaching or defaulting Market Participant; and

(c)	Partial-month CRRs resulting from bilateral trades.

(2)	The remaining unvoided CRRs in the repossessed or forfeited portfolio will be liquidated in the following manner, based on the characteristics of the CRRs:

(a) 	For CRRs that settle in the same month as the effective date of the repossession or forfeiture, for CRRs where the timing of the Market Participant Payment Breach or Default does not allow for the repossessed or forfeited CRRs to be offered into the CRR Monthly Auction in the calendar month following the Payment Breach or Default, for forfeited CRRs that were awarded in a CRR Monthly Auction, and for repossessed CRRs that have been offered into every available auction but were not fully awarded, the CRRs will settle in the DAM and may result in net payments or charges accruing for Operating Days throughout the month, as follows:

(i)	If the CRR portfolio results in a net charge in the DAM, the payment will, to the extent possible, be made by drawing on any available Financial Security of the Market Participant from whom the CRRs were repossessed or forfeited.  If this is insufficient to pay the charge in full, the remainder due will be short-paid for the Operating Day in accordance with Section 9.7.3, Enforcing the Financial Security of a Short-Paying Invoice Recipient.  Regardless of the short pay, any Market Participant from whom CRRs were repossessed or forfeited shall remain liable for any charges associated with the liquidation of its CRRs in accordance with paragraph (4) of Section 16.11.6.1.7, Revocation of a Market Participant’s Rights and Termination of Agreements.

(ii)	If the CRR portfolio results in a net payment in the DAM, the payment amount will be added to the cash collateral held by ERCOT for the Market Participant from whom the CRRs were repossessed or forfeited.

(b) 	For any remaining unvoided repossessed or forfeited CRRs for one-month or multi-month strips, ERCOT will offer each CRR into the next available auction for the effective time period of the repossessed or forfeited CRR.

(i)	CRRs will be offered into auctions at -$0.01 for PTP Options and -$500.00 for PTP Obligations.

(ii)	If a CRR is offered into an auction but is not fully awarded, the remaining MW of the CRR will be offered into the next available auction for the effective time period of the repossessed or forfeited CRR.  ERCOT may adjust the offer price for a PTP Obligation in subsequent auctions to increase the likelihood of the sell offer being fully awarded.

If another auction will not be held for the effective time period of the repossessed or forfeited CRRs, the remaining unawarded MW of each CRR will settle in the DAM and may result in net payments or charges accruing for Operating Days throughout the month, as noted in paragraph (2)(a) above.

(iii)	Prior to offering repossessed or forfeited CRRs into an auction, ERCOT will calculate the value of all PTP Obligations for each CRR Account Holder associated with the defaulting Counter-Party by summing the value of all existing PTP Obligations for which the most recent auction clearing price for the effective time period of the auction was negative.  The value of each CRR will be calculated per the following formula: (MW amount) x (TOU hours) x (auction clearing price).  The total value of the PTP Obligations for the defaulting Counter-Party will be evaluated to determine if it exceeds 10% of the value of all existing PTP Obligations for which the most recent auction clearing prices for the effective time period of the auction were negative.  If the total value of the PTP Obligations for the defaulting Counter-Party to be offered into an auction exceeds the 10% threshold, ERCOT will prorate the CRR MW amounts to be offered into the auction to reduce the total portfolio value to be below the 10% threshold.  The remaining CRR MW will be offered into the next available auction for the effective time period of the repossessed or forfeited CRRs or will settle in DAM if there will not be another auction for the effective time period.

(iv)	A charge resulting from the sale of the CRRs into an auction will, to the extent possible, be paid by drawing on any available Financial Security of the Market Participant from whom the CRRs were repossessed or forfeited.  If this is insufficient to pay the charge in full, the remainder due will be short-paid in accordance with Section 9.7.3.  Regardless of the short pay, any Market Participant from whom CRRs were repossessed or forfeited shall remain liable for any charges associated with the liquidation of its CRRs in accordance with paragraph (4) of Section 16.11.6.1.7.

(v)	A payment resulting from the sale of the CRRs into an auction will be added to the Financial Security of the Market Participant from whom the CRRs were repossessed or forfeited.

16.11.6.1.7	Revocation of a Market Participant’s Rights and Termination of Agreements

(1)	ERCOT may revoke a breaching Market Participant’s rights to conduct activities under these Protocols.  ERCOT may also terminate the breaching Market Participant’s agreements with ERCOT.

(2)	If ERCOT revokes a Market Participant’s rights or terminates the Market Participant’s agreements, then the provisions of Section 16.2.5, Suspended or Terminated Qualified Scheduling Entity – Notification to LSEs and Resource Entities Represented, and Section 16.2.6.1, Designation as an Emergency Qualified Scheduling Entity or Virtual Qualified Scheduling Entity, apply.

(3)	If a breaching Market Participant is also an LSE (whether or not the Default occurred pursuant to the Market Participant’s activities as an LSE), then:

(a) 	Within 24 hours of receiving notice of the Payment Breach, the Market Participant shall provide to ERCOT all the information regarding its Electric Service Identifiers (ESI IDs) set forth in the ERCOT Retail Market Guide; and

(b)	On revocation of some or all of the Market Participant’s rights or termination of the Market Participant’s agreements and on notice to the Market Participant and the Public Utility Commission of Texas (PUCT), ERCOT shall initiate a Mass Transition of the Market Participant’s ESI IDs pursuant to Section 15.1.3.1, Mass Transition Process, without the necessity of obtaining any order from or other action by the PUCT.

(4)	After revocation of its rights or termination of its Agreement with ERCOT, the Market Participant will remain liable for all charges or costs associated with any continued activity related to the Counter-Party’s relationship with ERCOT and any expenses arising from the consequences of such termination or revocation.

16.11.7 	Release of Market Participant’s Financial Security Requirement

(1)	Following the termination of a Market Participant’s Standard Form Market Participant Agreement, ERCOT shall retain Financial Security to cover potential future obligations of the terminated Market Participant.  These obligations may include, but are not limited to, the Invoices associated with the liquidation of repossessed or forfeited CRRs, Resettlement Statements, Final or True-Up Settlements, and Default Uplift Invoices.

(2)	Regardless of whether a Market Participant’s Agreement with ERCOT has been terminated, ERCOT shall not return or release any Financial Security of a Market Participant from whom CRRs were repossessed or forfeited until all such repossessed or forfeited CRRs have been voided, settled in the DAM, or sold in a CRR Auction in accordance with Section 16.11.6.1.6, Liquidation of Repossessed or Forfeited CRRs, and all Invoices associated with the liquidation have been paid in full.

(3)	Except as specified in paragraph (2) above, required Financial Security for potential future obligations of a terminated Market Participant will be the maximum of the Counter-Party’s TPE, as applicable, or $5,000.

(4)	If a terminated Market Participant elects to withdraw non-cash Financial Security following termination, and ERCOT determines that Financial Security continues to be necessary to cover potential future obligations, then the terminated Market Participant must provide ERCOT with Cash Collateral in the amount determined by ERCOT under this section before ERCOT will return or release the non-cash Financial Security to the terminated Market Participant.

(5)	Upon ERCOT’s sole determination that no sums remain owed or are necessary to cover potential future obligations to ERCOT by the terminated Market Participant, ERCOT shall return or release any Financial Security held by ERCOT to the terminated Market Participant.

16.11.8  	Conversion of Letters of Credit and Surety Bonds to Cash Collateral

(1)	To facilitate Settlement of Market Participant Invoices arising in consequence of a Payment Breach or other Default by a Market Participant, including but not limited to those described in Section 16.11.6.1.4, Repossession of CRRs by ERCOT, Section 16.11.6.1.5, Declaration of Forfeit of CRRs, and Section 16.11.6.1.6, Liquidation of Repossessed or Forfeited CRRs, ERCOT may at its sole discretion initiate conversion of guarantees, letters of credit, or surety bonds held as Financial Security to cash collateral.

16.11.9		Acceleration

(1)	Upon termination of a Market Participant’s rights as a Market Participant and any other agreement(s) between ERCOT and the Market Participant, all sums owed to ERCOT are immediately accelerated and are immediately due and owing in full.  At that time, ERCOT may immediately draw upon the Market Participant’s Financial Security and shall use those funds to offset or recoup all amounts due to ERCOT.