ERCOT Nodal Operating Guides

Section 8

Attachment L

Emergency Operations Plan

February 1, 2017

This attachment provides a template to be used by each Transmission Operator (TO) for the development of its emergency operations plan to mitigate operating emergencies, as required by the applicable North American Electric Reliability Corporation (NERC) Reliability Standard.  The emergency operations plan can be made up of multiple parts and does not need to be a single document.  When multiple parts are used, the TO shall include documentation describing the location of each element required by the applicable NERC Reliability Standard.  Each plan should include each of the elements listed below:

I.	PURPOSE – The purpose statement will address the TO’s operations plan to mitigate operating emergencies.

II.	SCOPE – The scope statement shall provide, in a brief summary, the boundaries of the emergency operations plan and to whom the emergency operations plan applies.

III.	DEFINITIONS – Definitions of terms that are used in the TO emergency operations plan that are not common to the ERCOT Region.  Define what is considered an operating emergency.

IV.	KEY PERSONNEL ROLES AND RESPONSIBILITIES – Identify roles and responsibilities of key personnel that are responsible for activating the plan.

V.	PROCESSES TO PREPARE FOR AND MITIGATE EMERGENCIES – Include the following:

A.	Notification to ERCOT to include current and known projected Real-Time conditions, when experiencing an operating emergency;

B.	Cancellation of Transmission Facility Outages;

C.	Transmission system reconfiguration;

D.	Provisions for operator-controlled manual Load shedding that minimizes the overlap with automatic Load shedding and that is capable of being implemented in a timeframe adequate for mitigating the emergency; and

E.	Reliability impacts of extreme weather conditions.