Texas SET Change Control Request Form Change Control Number: 2020 - 815 Implementation Version: Future

This Section Is Completed by Submitter of Change Control Request Only:

| Submitter Name: Kathy Scott | Submitting Company Name: CenterPoint Energy | Phone Number: 713-582-8654 |
| --- | --- | --- |
| Date of Submission: April 7, 2020 | Affected TX SET Transaction(s): 650_01 | Submitter’s E-Mail Address: Kathy.Scott@CenterPointEnergy.com |
| Texas SET Issue cross-reference number: |  | Protocol Impact (Y/N): N |
| Detailed Description and Reason for Proposed Change(s): This Change Control includes the following changes or clarifications that are the result of market recommendations following Hurricane Harvey Lessons Learned Workshop and several follow-up discussions held during TX SET meetings over the past 18 months: 650_01 Service Order Request Adds or Clarifies the following in the REF~8X (Purpose Code): Add: New Purpose Code “DC006” – “Disconnect Premise Due to Safety, Weather related or Emergency Condition(s)” Clarify: Existing Purpose Code “RC002”, “RC003” and “RC004” Add: New Purpose Code “RC006” – “Reconnect Premise After Safety, Weather related or Emergency Conditions No Longer Exist” Add: New Purpose Code “RC007” – “Reconnect Premise after Corrections Made to Resolve Service Standards Clearance Violation(s)” On March 26, 2020 after reviewing the draft version of this 650_01 redlined changes consensus was gained by TX SET members in attendance to draft this change control that will be discussed during a subsequent TX SET Change Control Conference Call. |  |  |
| NOTE: Requester must complete above fields and include a redlined example of modifications to each impacted implementation guide. This must be included at the time the request form is submitted. Please submit this completed form via e-mail to txsetchangecontrol@ercot.com and RMS Chair. |  |  |

For ERCOT Change Control Manager Use Only:

| Texas SET Recommendation: Recommend for next TX SET release | Recommendation for Emergency (Y/N): N | Date of TX SET Recommendation: 04/15/2020 |
| --- | --- | --- |
| Detailed Description and Reason for Revision: |  |  |
| RMS Decision: | Emergency (Y/N): | Date of RMS Decision: |
| Summary of RMS Discussion: |  |  |

650_01 Service Order Request

Segment:	REF Reference Identification (Purpose Code)

Position:	030

Loop:	HL        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | HL Parent Loop (Service Order Level Information) Segment contains all of the service orders that are available to request. All service orders must be for known ESI IDs, including service orders for Street Lights and Guard Lights. When requesting action on a specific meter on a multi-metered/un-metered ESI ID you must specify in REF~8X the appropriate purpose for the transaction and provide specific information in the REF~MG for meter service request or REF~ADE for un-metered service request. The following Purpose Codes can only be used when requesting the associated Transaction Type Code as noted below: Purpose Codes Beginning With: DC use only when BGN07 = 72 Disconnect FI use only when BGN07 = XZ Facilities Investigation GL use only when BGN07 = AN Lighting SL use only when BGN07 = AN Lighting RC use only when BGN07 = 79 Reconnect ME use only when BGN07 = KH Meter Exchange MT use only when BGN07 = 38 Meter Test MM use only when BGN07 = 13 Meter Maintenance RD use only when BGN07 = RD Read (Out of Cycle) SH use only when BGN07 = SH Switch Hold Indicator TE use only when BGN07 = IN Technical Environmental |
| --- | --- | --- |
|  |  | Required on all 650_01 Service Request, if CR does not provide a Purpose Code TDSP will reject the transaction |
|  |  | REF~8X~DC001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 8X |  | Transaction Category or Type |  |  |  |  |  |
|  |  |  |  |  |  | Purpose Code |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | DC001 |  | Disconnect for Non-Pay |  |  |  |  |  |
|  |  |  |  |  |  | When this purpose code is sent, the Segment YNQ Yes/No Disconnect Premium Location is Required to be sent with a Y(Yes) in the YNQ02 |  |  |  |  |  |
|  |  |  |  | DC002 |  | Disconnect for Customer Requested Clearance |  |  |  |  |  |
|  |  |  |  |  |  | Disconnect for Customer Requested Clearance (DC002) and Reconnect for Customer Requested Clearance (RC002) may be sent to the TDSP at the same time to allow the TDSP to appropriately schedule both Disconnect and Reconnect request. |  |  |  |  |  |
|  |  |  |  | DC003 |  | Remove One Specific Meter on a Multi-Meter Premise/ESI-ID. |  |  |  |  |  |
|  |  |  |  |  |  | This can only be used to Remove One Specific Meter from a Multi-Meter Premise/ESI-ID. Not to be used for Non-Multi-Metered Premises/ESI-ID. This does not close out the Premise/ESI-ID. |  |  |  |  |  |
|  |  |  |  | DC004 |  | Disconnect Due to Denial of Access to the Meter |  |  |  |  |  |
|  |  |  |  |  |  | This code authorizes the TDSP to disconnect service at any location accessible to them, which includes premium disconnect location. |  |  |  |  |  |
|  |  |  |  | DC005 |  | Disconnect for Non-Pay for Charges associated to Tampering |  |  |  |  |  |
|  |  |  |  |  |  | This code authorizes the TDSP to disconnect service at any location accessible to them, which includes premium disconnect location. |  |  |  |  |  |
|  |  |  |  | DC006 |  | Disconnect Premise Due to Safety, Weather related or Emergency Condition(s) |  |  |  |  |  |
|  |  |  |  |  |  | Used by CR to request TDSP disconnect Premise due to safety, weather related or emergency conditions. Example: Fire, flooding, hurricane, tornado and/or any number of safety, weather related or emergency conditions. |  |  |  |  |  |
|  |  |  |  | FI001 |  | Relocation of Service/Facilities |  |  |  |  |  |
|  |  |  |  |  |  | Used when Retail Customer is requesting relocation of Retail Customer and/or TDSP owned equipment for example service pole, drops, metering equipment (meter can, may include breaker box), weatherhead, streetlight/guard light and/or customer owned transformer. |  |  |  |  |  |
|  |  |  |  | FI002 |  | Customer Requesting Information Only concerning Relocation of Services/Facilities |  |  |  |  |  |
|  |  |  |  | FI003 |  | Plan Review |  |  |  |  |  |
|  |  |  |  | FI004 |  | Dead Animal on Facilities |  |  |  |  |  |
|  |  |  |  | FI005 |  | Transformer Open |  |  |  |  |  |
|  |  |  |  | FI006 |  | Fire Ants in/on TDSP owned equipment |  |  |  |  |  |
|  |  |  |  | FI007 |  | Wire |  |  |  |  |  |
|  |  |  |  | FI008 |  | Pole |  |  |  |  |  |
|  |  |  |  | FI009 |  | Transformer |  |  |  |  |  |
|  |  |  |  | FI010 |  | Crossed Meters |  |  |  |  |  |
|  |  |  |  | FI011 |  | Possible Meter Damage |  |  |  |  |  |
|  |  |  |  | FI012 |  | Customer inquiry or installation of some type of Generation device or equipment at the Premise |  |  |  |  |  |
|  |  |  |  |  |  | This code is used by the CR to notify the TDSP of Customer's inquiry or installation of some type of Generation (example: wind, solar, micro-turbines, etc.) at the premise. CR will provide in the MTX segment any additional information of the type of generation, Customer and CR contact information (telephone/cell phone numbers, email address, etc.) needed for TDSP to communicate directly with Customer. |  |  |  |  |  |
|  |  |  |  | FI013 |  | Electric Vehicle |  |  |  |  |  |
|  |  |  |  |  |  | Used when Retail Customer and / or Retail Electric Provider is requesting a load analysis of TDSP owned equipment capacity to serve Electric Vehicle that may be present at the customer's premise. |  |  |  |  |  |
|  |  |  |  | GL001 |  | Guard Light - Lights Out |  |  |  |  |  |
|  |  |  |  | GL002 |  | Guard Light - Dim Light |  |  |  |  |  |
|  |  |  |  | GL003 |  | Guard Light - Light Hanging |  |  |  |  |  |
|  |  |  |  | GL004 |  | Guard Light - Pole Leaning |  |  |  |  |  |
|  |  |  |  | GL005 |  | Guard Light - Light On All Day |  |  |  |  |  |
|  |  |  |  | GL006 |  | Guard Light - Light Goes On and Off |  |  |  |  |  |
|  |  |  |  | GL007 |  | Guard Light - Install Shield |  |  |  |  |  |
|  |  |  |  | GL008 |  | Guard Light - Re-aim |  |  |  |  |  |
|  |  |  |  | GL009 |  | Guard Light - Remove a Specific Lamp |  |  |  |  |  |
|  |  |  |  | ME001 |  | Exchange with AMR (Automated Meter Read) Meter |  |  |  |  |  |
|  |  |  |  | ME002 |  | Exchange with OMR (Off-site Meter Read) Meter |  |  |  |  |  |
|  |  |  |  | ME003 |  | Exchange with IDR |  |  |  |  |  |
|  |  |  |  | ME004 |  | Exchange with TOU |  |  |  |  |  |
|  |  |  |  | ME005 |  | Exchange with Demand Meter |  |  |  |  |  |
|  |  |  |  | ME006 |  | Exchange with kVAR Meter |  |  |  |  |  |
|  |  |  |  | ME007 |  | Exchange with kWh Meter |  |  |  |  |  |
|  |  |  |  | ME008 |  | Exchange with Electrical Pulse Metering |  |  |  |  |  |
|  |  |  |  | ME009 |  | Exchange (like for like) |  |  |  |  |  |
|  |  |  |  | ME010 |  | Upgrade (Demand Capacity, Scale # of Dials) |  |  |  |  |  |
|  |  |  |  |  |  | Upgrade is an exchange of a like meter for like meter where there is a need for a change due to increased functionality or customer request. On upgrades, comments will be used for details. |  |  |  |  |  |
|  |  |  |  | ME011 |  | Damaged |  |  |  |  |  |
|  |  |  |  |  |  | A damaged meter purpose code indicates that the customer knows the meter has been damaged. |  |  |  |  |  |
|  |  |  |  | ME012 |  | Dead Meter |  |  |  |  |  |
|  |  |  |  | ME013 |  | Optional IDR Removal |  |  |  |  |  |
|  |  |  |  |  |  | ESI ID met threshold requirement for Optional IDR Removal |  |  |  |  |  |
|  |  |  |  | ME014 |  | Mandatory IDR Installation |  |  |  |  |  |
|  |  |  |  |  |  | ESI ID met threshold requirement for Mandatory IDR Installation |  |  |  |  |  |
|  |  |  |  | MM001 |  | Install Lock Band |  |  |  |  |  |
|  |  |  |  | MM002 |  | Remove Lock Band |  |  |  |  |  |
|  |  |  |  | MM003 |  | Meter Based Angle Adapter "elbow" |  |  |  |  |  |
|  |  |  |  | MM004 |  | Dual Socket Adapter |  |  |  |  |  |
|  |  |  |  | MM005 |  | Verify Meter Data |  |  |  |  |  |
|  |  |  |  | MM006 |  | Tampering Suspected |  |  |  |  |  |
|  |  |  |  | MM007 |  | Investigate or Verify Premise Type |  |  |  |  |  |
|  |  |  |  | MT001 |  | Meter Test |  |  |  |  |  |
|  |  |  |  | RC001 |  | Reconnect after Disconnect for Non-Pay |  |  |  |  |  |
|  |  |  |  | RC002 |  | Reconnect for Customer Requested Clearance |  |  |  |  |  |
|  |  |  |  |  |  | Disconnect for Customer Requested Clearance (DC002) and Reconnect for Customer Requested Clearance (RC002) may be sent to the TDSP at the same time to allow the TDSP to appropriately schedule both Disconnect and Reconnect request. NOTE: Where applicable City Permit will be required before Premise can be reconnected by the TDSP. |  |  |  |  |  |
|  |  |  |  | RC003 |  | Reconnect Premise |  |  |  |  |  |
|  |  |  |  |  |  | Used by the current REP of Record to restore service at the Premise when the current REP of Record wasn’t the disconnecting REP. NOTE: Where applicable City Permit will be required before Premise can be reconnected by the TDSP. |  |  |  |  |  |
|  |  |  |  | RC004 |  | Reconnect after Disconnect for Denial of Access |  |  |  |  |  |
|  |  |  |  |  |  | Used by the CR to request reconnection after a disconnection for denial of access to the meter. The CR is required to use the MTX segment to explain what the Customer did to resolve the access issue. |  |  |  |  |  |
|  |  |  |  | RC005 |  | Reconnect after Tampering Disconnect |  |  |  |  |  |
|  |  |  |  | RC006 |  | Reconnect Premise After Safety, Weather related or Emergency Conditions No Longer Exist |  |  |  |  |  |
|  |  |  |  |  |  | Used by CR to request TDSP reconnect service after safety, weather related or emergency conditions no longer exist. Example: Fire, flooding, hurricane, tornado and/or any number of safety, weather related or emergency conditions are resolved. NOTE: Where applicable City Permit will be required before Premise can be reconnected by the TDSP. |  |  |  |  |  |
|  |  |  |  | RC007 |  | Reconnect Premise after Corrections Made to Resolve Service Standards Clearance Violation(s) |  |  |  |  |  |
|  |  |  |  |  |  | Used by CR to request reconnection of service after corrections were made to Service Standards Clearance Violation. The MTX segment will be used by CR to explain what the Customer did to resolve the Service Standards Clearance Violation(s). |  |  |  |  |  |
|  |  |  |  | RD001 |  | Special Out of Cycle Read |  |  |  |  |  |
|  |  |  |  | RD002 |  | Re-Read/Potential Error |  |  |  |  |  |
|  |  |  |  | SH001 |  | Add Payment Plan Switch Hold Indicator |  |  |  |  |  |
|  |  |  |  | SH002 |  | Remove Payment Plan Switch Hold Indicator |  |  |  |  |  |
|  |  |  |  | SH003 |  | Remove Tampering Switch Hold Indicator |  |  |  |  |  |
|  |  |  |  | SL001 |  | Street Light - Lights Out |  |  |  |  |  |
|  |  |  |  | SL002 |  | Street Light - Dim Light |  |  |  |  |  |
|  |  |  |  | SL003 |  | Street Light - Light Hanging |  |  |  |  |  |
|  |  |  |  | SL004 |  | Street Light - Pole Leaning |  |  |  |  |  |
|  |  |  |  | SL005 |  | Street Light - Light On All Day |  |  |  |  |  |
|  |  |  |  | SL006 |  | Street Light - Light Goes On and Off |  |  |  |  |  |
|  |  |  |  | SL007 |  | Street Light - Install Shield |  |  |  |  |  |
|  |  |  |  | SL008 |  | Street Light - Needs Tag |  |  |  |  |  |
|  |  |  |  | SL009 |  | Street Light - Pole Paint |  |  |  |  |  |
|  |  |  |  | SL010 |  | Street Light - Remove a Specific Lamp |  |  |  |  |  |
|  |  |  |  | TE001 |  | Radio/TV Interference |  |  |  |  |  |
|  |  |  |  | TE002 |  | EMF (Electro-Magnetic Field) |  |  |  |  |  |
|  |  |  |  | TE003 |  | PCB |  |  |  |  |  |
|  |  |  |  | TE004 |  | Audible Interference |  |  |  |  |  |
|  |  |  |  | TE005 |  | Power Quality |  |  |  |  |  |
|  |  |  |  | TE006 |  | Tree Trimming |  |  |  |  |  |
|  |  |  |  | TE007 |  | Right of Way Cut/Brush Clearing |  |  |  |  |  |
|  |  |  |  | TE008 |  | Tree Removal or Cut Down Tree(s) |  |  |  |  |  |
|  |  |  |  | TE009 |  | Mow Grass in Easement |  |  |  |  |  |
|  |  |  |  | TE010 |  | Property Damaged by Tree Crew |  |  |  |  |  |
|  |  |  |  | TE011 |  | Customer Complaint concerning Tree Crew work |  |  |  |  |  |