| NPRR Number | 1063 | NPRR Title | Dynamic Rating Transparency |
| --- | --- | --- | --- |
| Date |  | April 21, 2021 |  |
| Submitter’s Information |  |  |  |
| Name |  | Seth Cochran |  |
| E-mail Address |  | cochran@dc-energy.com |  |
| Company |  | DC Energy Texas |  |
| Phone Number |  | 512-971-8767 |  |
| Cell Number |  |  |  |
| Market Segment |  | Independent Power Marketing (IPM) |  |

Comments

DC Energy supports ERCOT’s sample report for Dynamic Ratings, as provided to the Reliability Operations Subcommittee (ROS) on April 1, 2021.  Accordingly, DC Energy submits these comments to align Nodal Protocol Revision Request (NPRR) 1063 with the sample report.

Revised Cover Page Language

| Revision Description | This Nodal Protocol Revision Request (NPRR) requires ERCOT to post information related to Dynamic Rating approvals to the Market Information System (MIS) Secure Area. |
| --- | --- |

Revised Proposed Protocol Language

3.10.8.4	ERCOT Responsibilities Related to Dynamic Ratings

(1)	ERCOT shall provide a system to accept and implement Dynamic Ratings or temperatures to be applied to rating tables for each hour in the Day-Ahead and in the Operating Hour.  ERCOT shall also:

(a)	Provide software and processes that allow secure access for TSPs and Market Participants and that maintains a log of data provided and the actions of the TSP and ERCOT, to implement the Dynamic Ratings as described above;

(b)	Use Dynamic Ratings for alarming, compliance with ERCOT and NERC requirements, and SCED purposes in both Real-Time operations and operational planning;

(c) 	Approve or reject the new Dynamic Rating request within 24 hours of receipt;

(d)     Post Dynamic Ratings approved by ERCOT for each planned production load of the Network Operations Model on the MIS Secure Area.  The posting will include the Transmission Element name, approved thermal rating limits, and the planned effective date; and

(e)	Implement the approved Dynamic Rating automatically within 24 hours of approval.

(2)	ERCOT shall provide a system to implement Dynamic Ratings and to obtain monthly expected ambient air temperatures to be applied to rating tables for the CRR Network Models.  Temperatures applied to the rating tables shall be determined using the same method as described in item (3)(f) of Section 7.5.5.4, Simultaneous Feasibility Test.  Transmission Elements that have Dynamic Ratings implemented in the Network Operations Model must have Dynamic Ratings in the CRR Network Models.

(3)	ERCOT shall identify additional Transmission Elements that have a high probability of providing significant added economic efficiency to the ERCOT market through Dynamic Rating and request such Dynamic Ratings from the associated TSP.  ERCOT shall post annually the list of the Transmission Elements and identify if the TSP has agreed to provide the rating on the MIS Secure Area.