TDSP AMS Data Practices Matrix 											v. 06/01/2016

|  | Questions regarding LSE files | AEP | CNP | Oncor | TNMP | Notes/Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Do all TDUs provide a complete set of 96 interval values for each day? | It is not acceptable to provide nulls for interval values and would be rejected at ERCOT. | It is not acceptable to provide nulls for interval values and would be rejected at ERCOT. | It is not acceptable to provide nulls for interval values and would be rejected at ERCOT. | It is not acceptable to provide nulls for interval values and would be rejected at ERCOT. |  |
| 2(a) | Do all TDUs trigger the estimation process when there are missing interval values or if the interval value does not pass VEE? | Yes | Yes | Yes | Yes |  |
| 2(b) | Are missing intervals provided as zero or estimated? | Estimated | Estimated | Estimated | Estimated If there is no data, or negative data, at the time the LSE is exporting it populates with a 0,E (zero labeled as an estimate) |  |
| 2(c) | What is each TDU’s process for estimating Missing interval values? | AEP’s automated systems will attempt to retrieve missing interval data for 5 days. Any intervals retrieved through the automated system go through the VEE process and are sent to the Market. If interval data cannot be recovered, manual estimation may be necessary. Full explanation of AEP’s estimation routine is referenced in 2(f) below. | Methods: The MIM (Missing Interval Monitor) Process will automatically make 5 attempts over 5 calendar days to check for actual data and retrieve the missing data. Manual estimation will be necessary if systematic estimation is not successful. If actual data becomes available greater than 5 calendar days after the consumption date a proactive inquiry is necessary to see if the meter has actual data available. For CNP’s estimation routine see 2(f) CNP note on page 12 of this document. Note: "0" is a valid interval value if there is an associated power outage or de-energized event/flag. | For Residential, estimated interval values are shaped based on ERCOT load profiles. For Commercial, estimated interval values are shaped based on the most recent 3 like days of historical usage (actuals or estimates) available for the premise. | The 2 week like day historical estimation process uses "like-days" from the designated reference week and Like-Day set. For example, if the data needing estimation is Tuesday data, and Tuesday of the preceding week is considered a Like-Day, then the corresponding intervals from Tuesday of the preceding week are used in the estimation. If there is data for both weeks the most recent week’s data will be used. | See “California Historical Estimation” document from TNMP for more details. |
| 2(d) | Under what circumstances do TDUs estimate interval values? | Missing intervals for an active premise or data fails VEE | Missing intervals for an active premise An estimate will also be rendered for an interval during which a time change occurred (clock adjustment). | Missing intervals for an active premise or data fails VEE | Missing intervals for an active premise or data fails VEE |  |
| 2(e) | What triggers the estimation process? | Missing interval value or register read or data does not pass VEE | Any time there is missing interval value or does not pass the VEE process triggers the estimation process CNP’s automated systems will concurrently attempt to retreive actual or missing interval data through CNP’s Missing Interval Monitor (MIM) processes for 5 days Note: CNP will not trigger any estimation based on register read | Missing register read or interval values or the data does not pass VEE | Missing register read or interval values or data does not pass VEE |  |
| 2(f) | Please explain the estimation process/method(s) used for each TDU | - Interval Gap Fill: Used for missing intervals within a day - 2 hours or less utilizes Interpolation - 2 hours or more utilizes “like days” averaging - Full Cut Estimation: Used when all intervals are missing for the day - If register read is available, current day consumption distributed evenly over day’s 96 intervals. - If register read or any intervals for the current day not available, the intervals from yesterday are used to estimate the usage. AEP’s process will look back for up to seven days for data to fulfill the logic for this process. - If register read for the current day or intervals from previous 7 days not available, the current day’s intervals are filled with estimated zeros. | See Uniformed Business Practices (UBP) procedures ** See note at end of matrix for CNP’s detailed estimation routine/process. | Register reads are estimated by using available historical data to determine the day’s consumption. Current day consumption distributed per the load shape over day’s 96 intervals ** See note at end of matrix for Oncor’s detailed estimation routine/process. | 1. Linear Interpolation – Used for intervals totaling 1 hr or less Linear interpolation operates on one or more intervals that need to be estimated, so long as a single contiguous group of intervals requiring estimation does not exceed 1 hour. 2. If (1) is unsuccessful, then '2 week like day historical' estimation process is used, as described below. The 2 week Like day historical estimation process replaces intervals needing estimation with data from "like-days" from the designated reference week and Like-Day set. For example, if the data requiring estimation is Tuesday, and Tuesday of the preceding week is considered a Like-Day, then the corresponding intervals from Tuesday of the preceding week are used in the estimation. If the previous week is not available, the data from two weeks prior is used. 3. If (1) and (2) are unsuccessful, then 'California historical' is used for intervals totaling more than 1 hr. This process calculates average daily load shapes using data from historical reference days. IEE derives interval-by-interval averages from valid intervals from the three nearest days of the same or like day of the week. |  |
| 3(a) | Are Zero (0) values provided for interval data? (Y/N) If so, what circumstances? | Yes Zero is a valid value for both estimates and actuals. - Zero actuals are rendered when received from an active/authorized premise. - Zero estimates are rendered for active/authorized premises when a known outage has been identified or the premise’s load has been disconnected. | Yes Zero is a valid value for both estimates and actuals. - Zero actuals are rendered when received from an active/authorized premise. - Zero estimates are rendered for active/authorized premises when a known outage has been identified or the premise’s load has been disconnected. | Yes Zero is a valid value for both estimates and actuals. - Zero actuals are rendered when received from an active/authorized premise. - Zero estimates are rendered for active/authorized premises when a known outage has been identified or the premise’s load has been disconnected. | Yes Zero is a valid value for both estimates and actuals. - Zero actuals are rendered when received from an active/authorized premise. | NOTE: The term ‘Active/Authorized’ is used for premises with an active or authorized REP of Record. This excludes premises where there is not an active or authorized REP of Record in the TDU’s system for the ESI ID. |
| 3(b) | If utilized, explain the validations performed for Zero value intervals? | AEP does not perform validations on zero value intervals. | The System identifies consecutive zero interval data and will flag intervals with a failure code. But, CNP isn’t performing zero validations at this time. Future plans will be to refine a core Validation Failure Report to be utilized for this purpose. | Oncor performs data analysis checks for Zero consumption after disconnects or MVOs on ESIIDs and initiates additional review where deemed appropriate The day after DNP completed, a validation is done to make sure the register read is the same as when the DNP occurred to determine if tampering/diversion has taken place. | TNMP does not validate on zero interval data. Zero monthly billed consumption and zero consumption after DNP or MVO is validated in the CIS. |  |
| 4(a) | If a TDU receives an Actual value for an interval that was previously Estimated, is the Estimated interval value replaced with the Actual interval value? | Yes, except in the case of stopped (tampered or damaged) meter since these were estimated because the meter was not allowed to register the actual usage. | Yes, except in the case of stopped (tampered or damaged) meter since these were estimated because the meter was not allowed to register the actual usage. | Once Oncor has re-established communication with a non-communicating meter, the gap reconciliation process will attempt to retrieve the interval data for up to the most recent four days. If the gap reconciliation process is successful, Oncor will replace the estimated intervals with the actual intervals for up to the most recent four days. | Any actual received after estimation will replace the estimate with an actual read. Any new/changed data with a read date 270 days old to current will be resent to the market provided profile and account status necessitate that data is available to the market |  |
| 4(b) | What is the timeline for replacing Estimated usage with Actual usage? | Up to 5 business days if new data is recovered through the automated process. Additionally, if a REP initiates a MarkeTrak and AEP is able to locate & re-process the data, then the Actual usage will replace the Estimated usage within 2 business days (manual process). | Up to 6 business days - Normal automated retry retrieval process will replace estimated usage typically within 6 days - Actuals obtained through an on-demand read will also replace an estimate, if one should occur. - Actuals can replace estimates later than 6 days when this manual activity occurs - Manual estimation will occur upon request | Typically, within seven business days. | Upon receipt of actual data. |  |
| 4(c) | How are usage updates reflected in the LSE file? | New LSE files will be posted with the create date and time stamp of the LSE file. | Date and time stamp upon replacement; New LSE files will be posted with the create date and time stamp of the LSE file. | New LSE files will be posted with the create date and time stamp of the LSE file | - Flagging update of actual data with “actual” - Use of actual or estimated flag is used based on the RMG Appendix G: ERCOT Specified File Format for Submission of Interval Data for Advanced Metering Systems. - Date and time stamp upon replacement; new LSE will be posted | See Retail Market Guide Section 7 – located in 7.15.2 (2) Website Link: http://www.ercot.com/mktrules/guides/retail/current |
| 5(a) | Is a Register Read always provided? | Yes. | No. - Will not provide a register reading in the daily LSE file if the register reading has been estimated for more than (2) consecutive register readings. - If the ESI ID has a skipped midnight read, no register reads will be rendered on the LSE until both a beginning and end register read is available | Yes. Oncor’s defines a Register Read as any “5-dial” read of the meter odometer. For example, the start and stop reads on an 867_03 are “register reads”. A “midnight read” is simply a Register Read that is taken at midnight. | Yes |  |
| 5(b) | If provided, under what circumstances do you estimate Register Reads? | When Register Read not received | Criteria: A ‘day end’ register read is only estimated when the midnight interval is gapped out (missing) and will be done only twice consecutively. Additionally, when a meter stops communicating, the MDM will estimate a register read twice. After the 2nd estimate, if the meter is still not communicating, no further register reads are in the MDM until the meter starts back communicating or the meter is exchanged. If the meter starts back communicating and encounter another non-communication period, the same estimation routine is performed. Method: The estimated register read is derived by adding the sum of the day’s actual plus estimated intervals to the beginning day’s register read. | When Register Read not received or fails VEE. - If the collection process successfully passes all the daily interval values, the MDM will calculate a missed Register read using the"96 good" interval values. - Estimated register reads are also provided when a cancel rebill occurs. The rebill register reads are used to manually calculate the interval adjustments. | When Register Read not received |  |
| 5(c) | What is the validation process for Register Reads? | UBP guidelines. Daily VEE is performed by the MDM on the register read. | Industry standards for Validation, Estimation and Editing (VEE) Monthly billing reads are validated prior to bill processing in the CIS. Daily register reads are not validated, however the associated interval data is validated daily in accordance with industry standard VEE, as it is loaded into the MDMS. | UBP guidelines | UBP guidelines Monthly billing reads are validated prior to bill processing in the CIS. Daily register reads are not validated, however the associated interval data is validated daily in accordance with industry standard VEE, as it is loaded into the MDMS. |  |
| 5(d) | How often is the Register Read validated? | During the preparation of each LSE file | Each time data passes through the MDM ‘s cleaner process Monthly billing reads are validated as-billed within the CIS. Midnight register reads are not currently validated. | During the preparation of each LSE file | Monthly billing reads are validated as-billed within the CIS. Midnight register reads are not currently validated. |  |
| 5(e) | What is the allowed variance between the sum of the interval data and the register read? | 2 times the meter multiplier | 2 times the meter multipliers | 2 times the meter multipliers The 2 multiplier variance applies to both IDR and AMS and is an industry standard used for many years. Reasoning is based on the fact that register reads are never rounded up. I.e. A register read of 20.01 and 20.99 would both appear as “20” when the presentation is in integer format. In this scenario, the register read would appear to have ZERO consumption for the period between the start and stop read. | 2 times the meter multipliers | Link for the Uniform Business Practices (UBP) for Unbundled Electric Metering http://www.naesb.org/req/req_form.asp (see notes at bottom) |
| 6(a) | What reading is used to complete a straight MVI? | On-demand read at time of completion | On-demand read at time of completion | Midnight reads (going into the day) when available. This may be an actual register read or the estimated value from the daily VEE process, otherwise on-demand read | Midnight read when available |  |
| 6(b) | What reading is used to complete a MVI Force Off? | Midnight read when available | Midnight read when available | Same as the MVI process | Midnight read when available |  |
| 6(c) | What reading is used to complete a MVO? | On-demand read at time of completion | On-demand read at time of completion | Same as the MVI process | On-demand read at time of completion |  |
| 6(d) | Explain smoothing as it pertains to your entity. | per Tariff Section 4.8.1.4 | per Tariff Section 4.8.1.4 -CNP performs a manual query to determine if any actuals are available in the MDM Note: CNP will look within our MDM system to verify if actual readings are available before invoking this smoothing process to the 867_03 usage transactions. | Smoothing applies to traditional meter estimation processes where a manual daily average is calculated when an assumed quantity of energy is deemed to have occurred and an adjustment is made in the cancel / rebill process. | Interval data for Standard meters is not smoothed. | for reference: Tariff Section 4.8.1.4 – “For Meters other than Standard Meters, AMS-M Meters, and IDR Meters, when an Actual Meter Reading is taken after two or more consecutive months of estimation, Company shall allocate any over or under-estimated usage over the entire estimation period. The allocation shall be based on the average daily consumption for the Retail Customer for the period between Actual Meter Readings. For Standard Meters, AMS-S Meters, and IDR Meters, Company shall consistently use reasonable methodologies to develop Estimated Billing Determinants. When Company must estimate Interval Data, it shall estimate the interval usage based on a methodology that reasonably accounts for the Retail Customer’s consumption and consumption patterns. If requested, Company shall provide the estimation methodology used. A Meter Reading for a Standard Meter, AMS-M Meter, or an IDR Meter shall not be considered an Estimated Meter Reading if an Actual Meter Reading was completed and Company had to estimate a limited number of intervals of data to fill in gaps in the data collected.” |
| 6(e) | Under what circumstances is an 867 cancelled & rebilled due to changes in AMS data (such as receiving an actual for a previous estimate)? | None | None | None | None currently TNMP is implementing a process where we will be internally alerted when Actual data is received for a period that has already been billed with Estimated data. If adjustments are needed, we will cancel/rebill accordingly. |  |
| 6(f) | How does Tampering impact the 867 vs LSE? | LSE will not be updated | LSE data will be updated subsequent to the cancel/rebill of an 867. | LSE data is normally updated. | 867 and LSE data will be updated |  |
| 6(g) | How are Inadvertent Gains handled in regards to the LSE files? | Once the Inadvertent Gain issue has been resolved, and all cancelling and rebilling of usage has been completed, and invoicing has been corrected for the REP of Record service period(s), AEP will update the Daily REP of Record file to SMT to reflect ROR changes from IAGs, and resend all LSE files for the same service period(s) for the corrected REP of Record to SMT for posting into the REP of Record’s FTPS folder. | LSE files are always sent to the CR of Record. LSE files will be resent where restatements are required due to the Inadvertent Gain. [See AMIT Issue 024 - OWG 115 Impacts of an Inadvertent Gain (IAG) on Interval Data (attached) where CNP's process has already been documented ] Once all cancel and rebilling of usage and invoicing has been corrected for the REP of Record service period(s). CNP will also resend all LSE files for the same service period(s) for the corrected REP of Record to SMT for posting into the REP of Record’s FTPS folder. | Oncor does not resend LSE files in the case of an IAG. | LSE files are always sent to the CR of Record. LSE files will be resent where restatements are required due to the Inadvertent Gain | Note: As a standard process at Smart Meter Texas, a REP of Record has access to all historical interval data available for an ESIID once the ESIID as long as they are REP of Record. |
| 6(h) | Under what scenarios is a REP of Record Update sent to SMT? Any differences for MVI, MVO, Switch? Or is the timing the same? | Any REP of record change initiated by a TX SET transaction. Dependent upon when the 867_03Final/ 867_04Initial is processed – which triggers a REP of Record change file to be sent to SMT. | Any REP of record change initiated by a TX SET transaction. Dependent upon when the 867_03Final/ 867_04Initial is processed – which triggers a REP of Record change file to be sent to SMT. Any gap days are provided as an LSE file when the 867 process is complete. | Any REP of record change initiated by a TX SET transaction, except for a backdated MVI. Dependent upon when the 867_03Final/ 867_04Initial is processed – which triggers a REP of Record change file to be sent to SMT. | Any REP of record change initiated by a TX SET transaction. Dependent upon when the 867_03Final/ 867_04Initial is processed – which triggers a REP of Record change file to be sent to SMT. |  |

2(f) CNP Note

CenterPoint Energy’s estimation routine for AMS 15 minute interval data:

Systemic:

If section of data needing estimation is 2 hours or less in length, point-to-point linear interpolation is used to estimate the data.

Can be either:

- Single interval to be estimated:  Estimated value used = ((interval value immediately preceding + interval value immediately following) / 2)

- Greater than 1 interval/less than 8 intervals bound by register reads:  [Difference of the register reads (Ending Register – beginning register) less the sum of the actual intervals] divided by # of missing intervals.

If section of data needing estimation is greater than 2 hours, the first successful pattern found will be used.

[Limitation of 60 day look back; Residential attempts 2 days then 1 before moving onto the next pattern; Commercial attempts 3 days than 2 then 1 before moving onto the next pattern.]

- Average usage pattern of ‘X’ same weekdays all actual data (defined as the same day of week)

- Average usage pattern of ‘X’ like days all actual data (defined as the same day type)

- Average usage pattern of ‘X’ same weekdays allow estimate values (defined as the same day of week)

- Average usage pattern of ‘X’ like days allow estimate values (defined as the same day type)

If none are successful, the systems will exception the missing data for manual remediation.

If the missing gap is bound by two actual register reads, the difference of the registered usage will be applied to the missing intervals with the allocation based on the pattern identified.

If the missing gap is not bound by actual register reads, the pattern values will be used.

Manual:

Will be addressed by use of one of the following:

- Manual estimation using actual historical data (greater than 60 days)

- Manual estimation using estimated historical data (greater than 60 days)

- Manual estimation using actual historical data from a nearby address

- Manual estimation for special conditions (usually for external request)

- Manual scaled with register

2(f) Oncor Note

If 1) TDSP Misses intervals early in the day:

And:

a) TDSP collects Actual Start Read and Actual Stop Read –

What does the TDSP do for the Register Read?  Oncor would have NO change to Actual Reads

What does the TDSP do for the intervals values?  Once Oncor re-establishes communication with the meter, Oncor’s gap reconciliation program will make up to two attempts to replace the missing interval data with actual interval data. If the gap reconciliation process fails, Oncor will estimate the consumption in the intervals utilizing the total consumption for the day from Register reads.

b) TDSP collects Actual start read but missed the Stop Read

What does the TDSP do for the Register Read?  Oncor will estimate the Stop Read which sets the kWhs for the day.

What does the TDSP do for the intervals values?  Assuming no intervals were collected during the day either, Oncor will use the estimated kWhs for the day and shape the interval values according to the ERCOT load profiles for Residential and Like day average shape for Non-residential.  Once Oncor has re-established communication with a non-communicating meter, the gap reconciliation program will attempt to retrieve the interval data for up to the most recent four days.  If the gap reconciliation process is successful, Oncor will replace the estimated intervals with the actual intervals for up to the most recent four days.

C) TDSP Misses the Start read but Collects the Actual Stop Read

What does the TDSP do for the Register Read(s)?  Missed Start Read would be estimated at the conclusion of the prior day’s consumption.  The Start Read (yesterday’s stop read) may be adjusted today once the End of Day Read today is collected.

What does the TDSP do for the interval values?  Once Oncor re-establishes communication with the meter, Oncor’s gap reconciliation program will make up to two attempts to replace the missing or estimated interval data with actual interval data. If the gap reconciliation process fails, the missing interval values will be estimated based on the Register reads at the end of today’s production.  Shaping the interval values will be done according to ERCOT load shapes for Residential and Commercial ESIIDs will be shaped based on the ESIID’s 3 like days prior.

d) Miss start - Miss Stop Read

What does the TDSP do for the Register Read? Oncor Estimates the Stop Register read for the day and uses previously estimated register start read which was the prior day’s “stop read”.  The total consumption for the day is estimated kWh.

What does TDSP do for the intervals values? Oncor uses the estimated kWh and Shape the interval values as described above.

2) Miss intervals in the middle of the day: a) – d) – This will be the same as the scenarios above for Oncor

3) Miss intervals at end of day: a) – d) same as the scenarios above for Oncor.

4) Miss all data for the day:    Oncor will estimate the Register Reads and the Interval Values will be shaped as described earlier

Uniform Business Practices (UBP) for Unbundled Electric Metering – Definition for “Like Days”

2.2. If the section of data needing estimation is more than 2 contiguous hours, use the average of selected reference days to estimate the data.

Rules and definitions for selecting reference days for estimation:

- “Same weekdays” are defined as the same day of week as the day that needs estimation. In the case of holidays, “same weekdays” are holidays.

- “Like days” are defined as the same day type (i.e., weekday, weekend, and perhaps holidays) as the day that needs estimation.

- A standard list of holidays should be used, regardless