# TAC Workshop on Non-Spin Ancillary Service (10-19-2021)

# Non-Controllable Load Resource (NCLR) Grouping - Current Implementation

- RRS Grouping algorithm runs hourly for the next operating day in Day-Ahead at 20 minutes past the top of the hour until

  - DRUC completes, or

  - 18:00

- DRUC completion, or 1800 if no successful DRUC execution by 1800, will trigger RRS Grouping report posting

- RRS grouping

  - Day-Ahead: based on NCLR’s COP status (= ‘ONRL’) and RRS responsibilities, at a randomly selected HE, group qualified NCLRs into two groups

(Group 1 and Group 2) such that each group has similar amount of RRS responsibilities

  - Real Time (once Manual Refresh in AS Manager is clicked): based on telemetry status and RRS schedule,

    - assign any qualified NCLR without an assigned group in Day Ahead to Group 1

    - qualified NCLRs with a group assigned in Day Ahead – no change

# Proposed grouping methodology to group NCLR and Generation Resources awarded Offline Non-Spin

- For a randomly selected hour, the Grouping Algorithm will select all Generation Resources with COP Status of OFFNS and NCLRs with non-zero Non-Spin Responsibility and COP Resource Status of ONRL

  - All NCLRs with COP Status of ONRL and non-zero Non-Spin Responsibility

  - All Generation Resources with COP Status of OFFNS, for Generation Resources only LSL MW value will be considered for deployment

- The Grouping Algorithm will then create groups of approximately 500 MW which will include randomly selected Generation Resource and NCLRs as defined in para 1

- Grouping Algorithm runs hourly for the next operating day in Day-Ahead at 20 minutes past the top of the hour until

  - DRUC completes, or

  - 18:00

- DRUC completion, or 1800 if no successful DRUC execution by 1800, will trigger Non-Spin Grouping report posting

- Real Time (once Manual Refresh in AS Manager is clicked): based on telemetry status and Non-Spin schedule,

    - Assign any qualified NCLR or Generation Resource without an assigned group in Day Ahead to Group 1

    - Qualified Generation Resource or NCLRs with a group assigned in Day Ahead – no change

- ERCOT may have to consider an overflow limit where if Group-1 becomes too large than the remaining Resources that were not assigned group in the day-ahead are considered for Group-2 and hence forth.

- ERCOT proposes online unit providing offline non-spin using power augmentation should be receive its grouping assignment as described in para-5 with limitations stated under para-6 above.  ERCOT also requests stakeholder’s feedback with any alternate solutions for online units providing offline Non-Spin using power augmentation.