| NPRR Number | 784 | NPRR Title | Mitigated Offer Caps for RMR Units |
| --- | --- | --- | --- |
| Date |  | August 2, 2016 |  |
| Submitter’s Information |  |  |  |
| Name |  | Mark Walker, Bill Barnes |  |
| E-mail Address |  | mark.walker@nrg.com, bill.barnes@nrg.com |  |
| Company |  | NRG Texas LLC, Reliant Energy Retail Services LLC (collectively, “NRG”) |  |
| Phone Number |  | 512-691-6261, 512-691-6137 |  |
| Cell Number |  | 512-585-0450, 315-885-5925 |  |
| Market Segment |  | Independent Generator, Independent Retail Electric Provider |  |

Comments

After consideration of stakeholder discussion at the July 14, 2016 Protocol Revision Subcommittee (PRS) meeting and the July 28, 2016 Technical Advisory Committee (TAC) meeting, NRG respectfully submits the following comments to the ERCOT Board of Directors to limit the increase in the Mitigated Offer Caps for RMR Units to $300/MWh under the methodology proposed in NPRR784.  While the uncapped methodology submitted initially in NPRR784 provides a price signal that is appropriate and consistent with the energy-only market design, the changes proposed in these comments seek to strike a balance by addressing stakeholder concerns while still achieving an improvement in locational price signal that can be implemented quickly.

Proposed Protocol Language Revision

4.4.9.4.1	Mitigated Offer Cap

(1)	Energy Offer Curves may be subject to mitigation in Real-Time operations under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Cap.  The Mitigated Offer Cap is:

(a)	For a Resource contracted by ERCOT under paragraph (2) of Section 6.5.1.1, ERCOT Control Area Authority, ERCOT shall increase the O&M cost such that every point on the Mitigated Offer Cap curve (cap vs. output level) is greater than the SWCAP in $/MWh.

(b)	For each RMR Unit contracted by ERCOT under Section 3.14.1, Reliability Must Run, in consideration of the Shadow Price caps of the transmission constraints which the RMR Unit may be required to resolve and variations in transmission system topology, ERCOT shall set the Mitigated Offer Cap curve equal to the highest value (in $/MWh) that is expected to allow SCED to Dispatch the RMR Unit, unless the Independent Market Monitor (IMM) expressly objects to this action.  The value that will be used for the Mitigated Offer Cap curve shall not exceed $300/MWh and will be initially determined and communicated as part of the Market Notice issued for the status of the RMR Unit, as described in paragraph (6) of Section 3.14.1.2, ERCOT Evaluation.  The Mitigated Offer Cap curve may be modified by ERCOT to ensure that the RMR Unit is Dispatched by SCED to help resolve transmission congestion in Real-Time or to allow the RMR Unit to be Dispatched by SCED after other Resources.  Any modification to the Mitigated Offer Cap curve by ERCOT shall be communicated by Market Notice.

- (c)	For a Generation Resource with a Commercial Operations Date after January 1, 2004, ERCOT shall construct an incremental Mitigated Offer Cap curve (Section 6.5.7.3) such that each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	14.5 MMBtu/MWh times the FIP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below; or

[NPRR664: Replace paragraphs (i) and (ii) above with the following upon system implementation:] (i) 14.5 MMBtu/MWh times the FIPRr; or (ii) The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below; or

(iii)	The amount determined by Verifiable Cost Manual Appendix 10, Procedures for Evaluating Costs and Caps for Energy Storage Resources, for energy storage resources.

- (d)	For all other Generation Resources, each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	10.5 MMBtu/MWh times the FIP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below.

[NPRR664: Replace paragraph (d) above with the following upon system implementation:] - (d) For all other Generation Resources, each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of: (i) 10.5 MMBtu/MWh times the FIPRr; or (ii) The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below.

(e)	Notwithstanding paragraphs (c)(ii), (c)(iii), and (d)(ii) above, the Mitigated Offer Cap verifiable variable O&M cost ($/MWh) for Quick Start Generation Resources (QSGRs) shall incorporate the generic or verifiable O&M cost to start the Resource from first fire to LSL including the startup fuel, plus a minimum energy component to account for LSL commitment costs, and consideration of a fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual.

- (f)	The multipliers for paragraphs (c)(ii) and (d)(ii) above are as follows:

(i)	1.10 for Resources running at a ≥ 50% capacity factor for the previous 12 months;

(ii)	1.15 for Resources running at a ≥ 30 and < 50% capacity factor for the previous 12 months;

(iii)	1.20 for Resources running at a ≥ 20 and < 30% capacity factor for the previous 12 months;

(iv)	1.25 for Resources running at a ≥ 10 and < 20% capacity factor for the previous 12 months;

(v)	1.30 for Resources running at a ≥ 5 and < 10% capacity factor for the previous 12 months;

(vi)	1.40 for Resources running at a ≥ 1 and < 5% capacity factor for the previous 12 months; and

(vii)	1.50 for Resources running at a less than 1% capacity factor for the previous 12 months.

- (g)	The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month.  ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

- (h)	The process for developing the Mitigated Offer Cap in paragraphs (a), (b), (c), (d) and (e) above must be described by ERCOT in a procedure approved by the appropriate TAC subcommittee, and posted to the MIS Secure Area within one Business Day after initial approval, and after each approved change.

5.6.1	Verifiable Costs

(1)	The Qualified Scheduling Entity (QSE) is responsible for submitting verifiable costs unless both the QSE and Resource Entity agree that the Resource Entity will have this responsibility, in which case both the QSE and Resource Entity shall submit an affidavit to ERCOT stating this arrangement.  Notwithstanding the foregoing, QSEs that submit Power Purchase or Tolling Agreements (PPAs) do not have the option of allowing Resource Entities to file verifiable costs.

(2)	Make-Whole Payments for a Resource are based on the Startup Offers and Minimum-Energy Offers for the Resource, limited by caps.  Until ERCOT approves verifiable unit-specific costs for that Resource, the caps are the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  When ERCOT approves verifiable unit-specific costs for that Resource the caps are those verifiable unit-specific costs.  A QSE or Resource Entity may file verifiable unit-specific costs for a Resource at any time, but it must file those costs no later than 30 days after five Reliability Unit Commitment (RUC) events for that Resource in a calendar year.  A RUC event begins when a Resource receives a RUC instruction to come or stay On-Line and ends the later of when the Resource shuts down or the end of the Operating Day.  The most recent ERCOT-approved verifiable costs must be used going forward.

(3)	These unit-specific verifiable costs may include and are limited to the following average incremental costs:

(a)	Allocation of maintenance requirements based on number of starts between maintenance events using, at the option of the QSE or Resource Entity, either:

(i)	Manufacturer-recommended maintenance schedule;

(ii)	Historical data for the unit and actual maintenance practices; or

(iii)	Another method approved in advance by ERCOT in writing;

(b)	Startup fuel calculations based on recorded actual measured flows when the data is available or based on averages of historical flows for similar starts (for example, hot, cold, intermediate) when actual data is not available.  Startup fuel will include filing separately the startup fuel required to reach breaker close and fuel after breaker close to Low Sustained Limit (LSL).  Any fuel required to shutdown a Resource will be submitted as the fuel from breaker open to shutdown;

(c)	Operation costs;

(d)	Chemical costs;

(e)	Water costs; and

(f)	Emission credits.

(4)	Standard Operations and Maintenance (O&M) costs pursuant to paragraph (6) below may be used in lieu of the incremental O&M costs set forth in items (3)(a), (c), (d) and (e) above.

(5)	These unit-specific verifiable costs may not include:

(a)	Fixed costs, which are any cost that is incurred regardless of whether the unit is deployed or not; and

(b)	Costs for which the QSE or Resource Entity cannot provide sufficient documentation for ERCOT to verify the costs.

(6)	At their election, QSEs or Resource Entities may receive standard O&M costs for both startup and minimum energy.  This election may be made by submitting an election form to ERCOT.  If a QSE or Resource has received final approval for actual verifiable O&M costs under the verifiable cost process, it may not elect to receive standard O&M costs.

(a)	Until December 31, 2011, standard O&M costs are defined as follows:

| Resource Category Start Year = 2009 | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Aeroderivative simple cycle commissioned after 1996 | 1,000.00 | 1,000.00 | 1,000.00 | 3.94 |
| Reciprocating Engine | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | 5.09 |
| Simple cycle ≤ 90 MW | 2,300.00 | 2,300.00 | 2,300.00 | 3.94 |
| Simple cycle ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 | 3.94 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 3.19 |
| Combustion turbine < 90 MW | 2,300.00 | 2,300.00 | 2,300.00 |  |
| Combustion turbine ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 |  |
| Steam turbine | 3,000.00 | 2,250.00 | 1,250.00 |  |
| Gas-steam non-reheat boiler | 2,310.00 | 1,732.50 | 866.25 | 7.08 |
| Gas-steam reheat boiler | 3,000.00 | 2,250.00 | 1,125.00 | 7.08 |
| Gas-steam supercritical boiler | 4,800.00 | 3,600.00 | 1,800.00 | 7.08 |
| Nuclear, coal, lignite and hydro | 7,200.00 | 5,400.00 | 2,700.00 | 5.02 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 5.50 |

(b)	For the period beginning January 1, 2012 and ending December 31, 2012, standard O&M costs shall be reduced by 10% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 900.00 | 900.00 | 900.00 | 3.55 |
| Reciprocating Engine | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | 4.58 |
| Simple cycle ≤ 90 MW | 2,070.00 | 2,070.00 | 2,070.00 | 3.55 |
| Simple cycle ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 | 3.55 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.87 |
| Combustion turbine < 90 MW | 2,070.00 | 2,070.00 | 2,070.00 |  |
| Combustion turbine ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 |  |
| Steam turbine | 2,700.00 | 2,025.00 | 1,125.00 |  |
| Gas-steam non-reheat boiler | 2,079.00 | 1,559.25 | 779.63 | 6.37 |
| Gas-steam reheat boiler | 2,700.00 | 2,025.00 | 1,012.50 | 6.37 |
| Gas-steam supercritical boiler | 4,320.00 | 3,240.00 | 1,620.00 | 6.37 |
| Nuclear, coal, lignite and hydro | 6,480.00 | 4,860.00 | 2,430.00 | 4.52 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.95 |

(c)	Beginning January 1, 2013 and going forward, standard O&M costs shall be reduced by 20% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 800.00 | 800.00 | 800.00 | 3.15 |
| Reciprocating Engine | $46.40/MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | 4.07 |
| Simple cycle ≤ 90 MW | 1,840.00 | 1,840.00 | 1,840.00 | 3.15 |
| Simple cycle ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 | 3.15 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.55 |
| Combustion turbine < 90 MW | 1,840.00 | 1,840.00 | 1,840.00 |  |
| Combustion turbine ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 |  |
| Steam turbine | 2,400.00 | 1,800.00 | 1,000.00 |  |
| Gas-steam non-reheat boiler | 1,848.00 | 1,386.00 | 693.00 | 5.66 |
| Gas-steam reheat boiler | 2,400.00 | 1,800.00 | 900.00 | 5.66 |
| Gas-steam supercritical boiler | 3,840.00 | 2,880.00 | 1,440.00 | 5.66 |
| Nuclear, coal, lignite and hydro | 5,760.00 | 4,320.00 | 2,160.00 | 4.02 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.40 |

(d)	If the QSE or Resource Entity chooses to utilize the standard O&M costs for O&M, standard O&M costs will be used by ERCOT going forward until either:

(i)	Verifiable variable O&M costs are filed; or

(ii)	ERCOT notifies the QSE or Resource Entity to update its verifiable costs as set forth in either paragraph (9) or (10) below.  If a Resource is receiving standard O&M costs, it may reelect standard O&M costs when resubmitting verifiable costs.

(7)	When submitting verifiable costs for combined cycle Resources, the QSE or Resource Entity must elect standard O&M costs for all Combined-Cycle Configurations or verifiable costs for all Combined-Cycle Configurations within the combined cycle train.

(8)	QSEs submitting PPAs as Resource-specific verifiable costs documentation are subject to the guidelines detailed below and in the Verifiable Cost Manual.

(a)	Only QSEs offering Three-Part Supply Offers for a specific Resource may submit a PPA as verifiable costs documentation.

(b)	A QSE submitting a PPA as verifiable costs documentation must represent 100% of the Resource’s capacity.

(c)	Only PPAs:

(i) 	Signed prior to July 16, 2008; and

(ii)	Not between Affiliates, subsidiaries or partners will be accepted as verifiable cost documentation.

(d)	Verifiable costs for PPAs shall be capped at the level of the highest comparable Resource (referred to as the reference Resource) specific verifiable costs approved by ERCOT without a PPA.  The ERCOT approved verifiable costs for a PPA shall be equal to the lesser of:

(i)	The cap as described in paragraph (d) above; and

(ii)	The costs from the PPA.

(e)	ERCOT shall use the Resource actual fuel costs submitted by the QSE for startup and operation at minimum-energy level (LSL), and shall use the Resource Category Startup Offer Generic Costs as the cap for the O&M portion of the Startup Costs until ERCOT receives and approves comparable Resource specific verifiable costs.

(f)	PPAs will no longer be accepted as verifiable cost documentation after the primary term of the contract expires.

(g)	ERCOT shall produce a report each April that provides the percentage of RUC Make-Whole Payments for Resources with PPAs during the 12 months of the previous calendar year.  If there are no Make-Whole Payments for Resources with PPAs, ERCOT shall not produce the annual report.  The report shall be based on the final Settlements and include the total number of Resources that used a PPA for their most recent verifiable cost submission that was approved by ERCOT.  ERCOT shall present the results of this study to the appropriate Technical Advisory Committee (TAC) subcommittee.

(h)	Notwithstanding anything to the contrary in this Section 5.6.1, QSEs representing PPAs may, at any time, submit data from a Resource as verifiable costs documentation and such documentation will be accepted for consideration by ERCOT.  A QSE submitting verifiable costs documentation pursuant to this paragraph shall not be required to submit a PPA to ERCOT for consideration for verifiable cost recovery.

(9)	ERCOT shall notify a QSE to update verifiable cost data of a Resource when the Resource has received more than 50 RUC instructions meeting the criteria in Section 5.6.2, RUC Startup Cost Eligibility, in a year, but ERCOT may not request an update more frequently than annually.

(10)	ERCOT shall notify a QSE to update verifiable cost data of a Resource if at least five years have passed since ERCOT previously approved verifiable cost data for that Resource.

(11)	Within 30 days after receiving an update Notice from ERCOT under either paragraph (9) or (10) above, a QSE or Resource Entity must submit verifiable cost data for the Resource.  Despite the provisions in paragraph (2) above, if the QSE or Resource Entity does not submit verifiable cost data within 30 days after receiving an update Notice, then ERCOT shall determine payment using the Resource Category Startup Offer Generic Cap, Resource Category Minimum-Energy Offer Generic Cap, and a zeroed value for variable O&M Cost as described in Section 4.4.9.4.1, Mitigated Offer Cap, in accordance with the schedule established in this section until updated verifiable costs are approved.  If the 30-day deadline has been reached before the start of the tenth day before the end of the month, the Resource’s verifiable costs will revert back to generic costs beginning on the first day of the following month.  If the 30-day deadline falls within the last ten days of the month, the Resource’s verifiable costs will revert back to generic costs on the first day of the second month following the deadline month.

(12)	Resource Entities that represent Reliability Must-Run (RMR) Resources shall submit to ERCOT, Startup and variable O&M Cost estimates to be used by ERCOT as proxies for verifiable Startup Cost and minimum-energy verifiable cost and for Settlement.  The ERCOT-approved verifiable Startup Cost estimate will equal the startup fuel estimate times the sum of the appropriate Fuel Index Price (FIP) or Fuel Oil Price (FOP) and the fuel adder, plus the startup O&M.  The ERCOT-approved minimum-energy verifiable cost estimate will equal the heat rate from the RMR Agreement contract times the sum of the appropriate FIP or FOP and the fuel adder, plus the variable O&M.  The startup and minimum-energy O&M cost estimates shall be revised monthly to be consistent with the latest actual costs for the RMR Unit submitted in accordance with Section 3.14.1.12, Reporting Actual Eligible Cost.  The Mitigated Offer Cap curve verifiable variable O&M shall be set in accordance with Section 4.4.9.4.1, Mitigated Offer Cap.

[NPRR664: Replace paragraph (12) above with the following upon system implementation:] (12) Resource Entities that represent Reliability Must-Run (RMR) Resources shall submit to ERCOT, Startup and variable O&M Cost estimates to be used by ERCOT as proxies for verifiable Startup Cost and minimum-energy verifiable cost and for Settlement. The ERCOT-approved verifiable Startup Cost estimate will equal the startup fuel estimate times the sum of the appropriate Fuel Index Price for Resource (FIPRr) or Fuel Oil Price (FOP) and the fuel adder, plus the startup O&M. The ERCOT-approved minimum-energy verifiable cost estimate will equal the heat rate from the RMR Agreement contract times the sum of the appropriate FIPRr or FOP and the fuel adder, plus the variable O&M. The startup and minimum-energy O&M cost estimates shall be revised monthly to be consistent with the latest actual costs for the RMR Unit submitted in accordance with Section 3.14.1.12, Reporting Actual Eligible Cost. The Mitigated Offer Cap curve verifiable variable O&M shall be set in accordance with Section 4.4.9.4.1, Mitigated Offer Cap.