EMERGENCY RESPONSE SERVICE

Procurement Methodology

ERCOT Board approved 11/19/2013

Effective Date of 11/20/2013

Table of Contents

A.	Document Description	4

B.	Change Control Process	4

C.	ERS Capacity Demand Curve	4

D.	ERS Offer Cap	5

E.	ERS Expenditure Limit	5

F.	Capacity Inflection Point	6

G.	Clearing Price	7

H.	ERS Capacity provided through ERS Self Provision	7

Electric Reliability Council of Texas, Inc. (ERCOT) administers Emergency Response Service (ERS) in accordance with Public Utility Commission of Texas (PUCT) Substantive Rule §25.507 and the ERCOT Nodal Protocols.  This document is intended to be consistent with these standards, but to the extent any conflict exists, the PUC Rule or Protocols control.

# Document Description

This document describes the mechanism for procuring ERS and is considered an “Other Binding Document,” as that term is defined in the ERCOT Protocols.

# Change Control Process

ERCOT Staff will provide a period for stakeholder review and comment for proposed revisions to this document as follows:

- ERCOT shall post proposed revisions to the Emergency Response Service Procurement Methodology to the ERCOT website.

- ERCOT shall also electronically notify stakeholders of the proposed revisions via the TAC and Others distribution list and define the comment period which shall be at least 14 days after initial posting.

- To receive consideration, comments should be submitted via email to ERS@ercot.com by the deadline set forth in the notification.

- Upon Market Participant written request, ERCOT will conduct a conference call and online review of the submitted comments.

- ERCOT will review proposed document revisions with the Technical Advisory Committee (TAC).

- ERCOT will submit proposed document revisions for ERCOT Board approval.

- Within three Business Days of ERCOT Board approval, ERCOT shall post the revised document to the ERCOT website.

# ERS Capacity Demand Curve

ERCOT will develop a capacity demand curve for each ERS Time Period, and all ERS products will be procured together within the limits of that curve.  ERCOT shall maximize the MW procured subject to the expenditure limit for the relevant Time Period.  Each demand curve is derived from the three following parameters, which ERCOT will specify in the Request for Proposal (RFP) for ERS procurement:

- ERS Offer Cap

- ERS Time Period Capacity Inflection Point

- ERS Time Period Expenditure Limit

MW

$/MW/Hr

Offer Cap

Expenditure Limit

Capacity

Demand

Curve

Capacity

Inflection

Point

# ERS Offer Cap

The ERS offer cap establishes a maximum possible procurement price of $80/MW/hr for every ERS Time Period during the ERS budget year.  ERCOT will automatically reject any offers above the offer cap.  This cap is consistent with the prices historically paid to Loads participating in TDSP Standard Offer Load Management Programs.

# ERS Expenditure Limit

P.U.C. Substantive Rule 25.507 restricts ERCOT’s ERS expenditures to an annual cost cap of $50 million.  ERCOT will allocate the $50 million available expenditure within its ERS budget year, which starts with the February through May Standard Contract Term and ends with the October through January Standard Contract Term.  No later than 60 days before each new ERS budget year, ERCOT will make an initial allocation of the annual expenditure limit to each ERS Time Period in each ERS Standard Contract Term based on the expected risk of reaching an Energy Emergency Alert (EEA) in that ERS Time Period, in accordance with the formula detailed below.  ERCOT will assign a high (H), moderate (M), or low (L) risk designation to each ERS Time Period and will assign a risk-weighting factor (a value from 1 to 10 with 10 being the highest risk value) for each risk designation.  ERCOT’s risk assessment will consider a number of factors, including, but not limited to, forecasted operating reserves, forecasted Load, and Resource outage information.  60 days before each ERS Standard Contract Term begins, ERCOT will update the allocation for each remaining ERS Time Period in the budget year to reflect updated forecasts and any expected remaining funds from ERS Standard Contract Terms within the same ERS budget year.  Any funds remaining at the end of an ERS budget year will not be carried forward into a new ERS budget year.

For each ERS Time Period, the expenditure limit is calculated as follows:

Where;

# Capacity Inflection Point

The Capacity Inflection Point establishes the point on the capacity demand curve where capacity can only be procured at an offer price less than the ERS Time Period offer cap while respecting the expenditure limit for that ERS Time Period. The Capacity Inflection Point for each time period is calculated as follows:

Table A below provides hypothetical calculations of the expenditure limits and Capacity Inflection Point for each ERS Time Period in each budget year.

Table A: ERS Time Period Expenditure Limit Allocation and Capacity Inflection Point Calculations

# Clearing Price

The highest offer accepted for an ERS Time Period will set the clearing price for all ERS Resources cleared in that ERS Time Period. In the event that multiple ERS product types are implemented in the Protocols, all ERS product types will be procured using a common ERS capacity demand curve for each ERS Time Period and the highest offer accepted for an ERS Time Period will set the clearing price for all ERS product types.

# ERS Capacity provided through ERS Self Provision

For any ERS self-provision, ERCOT will reduce the Time Period expenditure limit for any offers to self-provide part or all of a Qualified Scheduling Entity’s ERS Obligation by the clearing price for ERS.

http://www.puc.state.tx.us/agency/rulesnlaws/subrules/electric/25.507/25.507ei.aspx