Non-Spinning Reserve Deployment and Recall Procedure

Effective June 1, 2026

Version  2.0

Document Revisions

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 03/30/2007 | 0.1 | TAC-approved version |  |
| 08/11/2010 | 0.2 | Updated to reflect changes to Protocol and current system implementation | Colleen Frosch John Dumas Resmi Surendran |
| 10/4/2010 | 0.3 | Included Non-Spinning Reserve Service Deployment and Recall Procedure Revision Process | Market Rules |
| 04/16/2012 | 0.4 | Updated to synchronize with the Protocol requirements introduced by NPRR426, Standing Non-Spin Deployment in the Operating Hour for Generation Resources Providing On-Line Non-Spin, to change the deployment and recall trigger mechanisms, and to remove language covered by Other Binding Documents | Market Operations Support |
| 05/03/2012 | 0.4 | TAC-approved version. Effective 5/14/12 |  |
| 11/07/2013 | 0.5 | TAC-approved version. Effective 12/1/13. Updated to reflect changes to Protocols pursuant to NPRR555, Load Resource Participation in Security-Constrained Economic Dispatch, with new language grey-boxed language pending implementation of NPRR555. | ERCOT |
| 06/01/2014 | 0.6 | Unboxed language in Sections 2.4 and 3 due to implementation of NPRR555. | ERCOT |
| 06/30/2021 | 0.7 | TAC-approved version. Effective 7/1/21. Revisions proposed by OBDRR031, Change Non-Spinning Reserve Service Deployment. | ERCOT |
| 07/12/2021 | 0.8 | Partially unboxed language related to OBDRR031. | ERCOT |
| 08/02/2021 | 0.9 | Unboxed remaining language related to OBDRR031. | ERCOT |
| 10/28/2021 | 1.0 | PUCT-approved version. Effective 11/1/21. Revisions proposed by OBDRR032, Non-Spin Changes Related to NPRR1093, Load Resource Participation in Non-Spinning Reserve. | ERCOT |
| 12/16/2021 | 1.1 | PUCT-approved version. Effective 1/1/22. Revisions proposed by OBDRR035, Related to NPRR1101, Create Non-Spin Deployment Groups made up of Generation Resources Providing Off-Line Non-Spinning Reserve and Load Resources that are Not Controllable Load Resources Providing Non-Spinning Reserve. | ERCOT |
| 05/27/2022 | 1.2 | Unboxed language related to OBDRR032 and OBDRR035. | ERCOT |
| 1/16/2025 | 1.3 | PUCT-approved version. Effective 2/1/25. Revisions proposed by OBDRR053, Alignment with NPRR1131, Controllable Load Resource Participation in Non-Spin, and Minor Clean-Ups | ERCOT |
| 6/1/2026 | 2.0 | Alignment with implementation of RTC+B | ERCOT |

PROTOCOL DISCLAIMER

This Other Binding Document describes ERCOT Systems and the response of these systems to Market Participant submissions incidental to the conduct of operations in the ERCOT Texas Nodal Market implementation and is not intended to be a substitute for the ERCOT Nodal Protocols (available at http://www.ercot.com/mktrules/nprotocols/current), as amended from time to time. If any conflict exists between this document and the ERCOT Nodal Protocols, the ERCOT Nodal Protocols shall control in all respects.

Table of Contents

1.	Overview of Non-Spinning Reserve Deployment and Recall Procedure	5

2.	Non-Spin Deployment Triggers	5

3.	Procedure upon Non-Spin Deployment	7

4.	Recall of Non-Spin Deployment	8

5.	Revision Process	9

## 1.	Overview of Non-Spinning Reserve Deployment and Recall Procedure

ERCOT will deploy Non-Spinning Reserve (Non-Spin) via ERCOT Operator Dispatch Instruction from Off-Line Generation Resources, from those portions of On-Line Generation Resources that are only available through power augmentation, and from Load Resources that are not Controllable Load Resources (CLRs) under the following situations:

- Detection of insufficient capacity for energy dispatch during periodic checking of available capacity;

- Disturbance conditions such as a unit trip, sustained frequency decay, or sustained low frequency operations;

- Security-Constrained Economic Dispatch (SCED) not having enough energy available to execute successfully; and

- When Off-Line Generation Resources or Load Resources that are not CLRs providing Non-Spin are the only reasonable option available to the ERCOT Operator for resolving local issues.

In each of these cases, the ERCOT Operator will make the final decision and initiate the deployment.  The ERCOT Operator shall deploy Non-Spin in amounts sufficient to respond to the operational circumstances.  This means that Non-Spin may be deployed partially over time or may be deployed in its entirety.  If Non-Spin is deployed partially, it shall be deployed in increments of 100% of each Resource’s capacity.

After deployment, ERCOT Operators may initiate a recall at the appropriate time, as further described in Section 4, Recall of Non-Spin Deployment, below.

## 2.	Non-Spin Deployment Triggers

In accordance with the situations addressed in Section 1, Overview of Non-Spinning Reserve Deployment and Recall Procedure, above, ERCOT may deploy Non-Spin under the following conditions:

- When the Non-Spin deployment margin is less than zero megawatts (MW), deploy sufficient Non-Spin capacity so that such margin is greater than 500 MW;

Where,

  - Non-Spin deployment margin = (On-Line capacity available to serve Load at t+30) – (GTBD+GTBD offset) – (IRR curtailed) – (30 Min net load ramp)

  - t+30: current time + 30 minutes;

  - GTBD: Generation to be Dispatched;

  - IRR: Intermittent Renewable Resources; and

  - On-Line capacity available to serve Load at t+30 = On-Line Generation Resources High Sustained Limits (HSLs) + State of Charge (SOC) limited HSL for Energy Storage Resources (ESRs) at t+30 – headroom reserved for up Ancillary Service Plans.

    - SOC limited HSL for ESRs at t+30 =minHSLOn-Line ESR ,HSLOn-Line ESR,   SOC limited

      - HSLOn-Line ESR,   SOC limited = SOC -BP * 30 mins -SOC reserved for Ancillary Service  Awards-minimumSOC5 mins

        - SOC reserved for Ancillary Service Awards = ESR RRSPFR Awards * RRSPFR duration + ESR RRSFFR Awards * RRSFFR duration + ESR Regulation Up Awards * Regulation Up duration + ESR ECRS Awards * ECRS duration + Non-Spin awarded to ESRs to cover the Non-Spin Plan * Non-Spin duration;

    - When SOC limited HSL for ESRs at t+30 is not limited by SOC:

      - Headroom reserved for up Ancillary Service Plans = (ECRS Plan not awarded to Load Resources + RRS Plan not awarded to Load Resources + Regulation Up Plan not awarded to Load Resources) + Non-Spin awarded to ESRs to cover the Non-Spin Plan;

    - When SOC limited HSL for ESRs at t+30 is limited by SOC:

      - Headroom reserved for up Ancillary Service Plans = ECRS Plan not awarded to Load Resources and ESRs + RRS Plan not awarded to Load Resources and ESRs + Regulation Up Plan not awarded to Load Resources and ESRs;

    - Non-Spin awarded to ESRs to cover the Non-Spin Plan = Non-Spin Plan - min(Non-Spin Plan, On-Line thermal Non-Spin Awards + Off-Line thermal Non-Spin Awards + Load Resource Non-Spin Awards);

    - ECRS Plan not awarded to ESRs and Load Resources = ECRS Plan – min(ECRS Plan, ESR ECRS Awards + Load Resource ECRS Awards);

    - RRS Plan not awarded to ESRs and Load Resources = RRS Plan – min(RRS Plan, ESR RRS Awards + Load Resource RRS Awards);

    - Regulation Up Plan not awarded to ESRs and Load Resources = Regulation Up Plan – min(Regulation Up Plan, ESR Regulation Up Awards + Load Resource Regulation Up Awards); and

    - Ancillary Service Plans in equations above are at t+30.

- When Physical Responsive Capability (PRC) < 3200 MW and is not expected to recover within 30 minutes without deploying reserves, deploy all or a portion of the available Non-Spin capacity;

- When PRC < 2500 MW, deploy all of the available Non-Spin capacity;

- When the North-to-Houston (N_H) Voltage Stability Limit Reliability Margin < 300 MW, deploy Non-Spin (all or partial) in the Houston area as needed to restore reliability margin;

- When Off-Line Generation Resources providing Non-Spin are the only reasonable option available to the ERCOT Operator for resolving local issues, deploy available Non-Spin capacity on only the necessary individual Resources; or

- Load Resources that are not CLRs and Generation Resources providing Off-Line Non-Spin can be deployed individually, as part of a subgroup using a random sampling of Resources needed to meet the target deployment amount, or as an entire block providing Non-Spin.  Deployments to individual Resources may only be done to manage inertia, congestion, or for other local needs.

If a condition other than those listed above indicates that additional capacity may need to be brought On-Line to manage reliability, ERCOT Operators will evaluate the system condition and deploy Non-Spin as needed if no other better options are available to resolve the system condition.  Under an emergency situation, the emergency process will govern the deployment of Non-Spin.

## 3.	Procedure upon Non-Spin Deployment

Upon a Non-Spin deployment, the following steps shall be taken:

3.1	Off-Line Generation Resource with Non-Spin award

- An Off-Line Generation Resource awarded Non-Spin, while Off-Line and before the receipt of any deployment instruction, shall be capable of being Dispatched to the Non-Spin award within 30 minutes of a Dispatch Instruction.

- The Qualified Scheduling Entity (QSE) for the Resource will be sent a Resource-specific Dispatch Instruction deployment indicating a time and date stamp, QSE, Dispatch Asset Code, and Deployed MW.

- The Dispatch Instruction for an Off-Line Generation Resource must include the expected amount of capacity that will be available for SCED and the anticipated duration of the deployment.

- The QSE must have the Off-Line Generation Resource On-Line with an Energy Offer Curve and the telemetered net generation must be greater than or equal to the Resource’s telemetered Low Sustained Limit (LSL) multiplied by P1, where P1 is defined in the “ERCOT and QSE Operations Business Practices During the Operating Hour,” within 25 minutes of the Dispatch Instruction.

- SCED will respond to the changes in Resource Status that are received by telemetry from the QSE.

- Once the Resource is On-Line it is Dispatched as any other Generation Resource, including any provisions for processing generation less than the Resource’s LSL.

- The Resource must, at a minimum, be capable of providing all the Non-Spin energy to SCED within 30 minutes of the Dispatch Instruction.

- The Load Resource must, at a minimum, be capable of remaining deployed until recalled.

3.2	On-Line Resource that will use power augmentation capacity to provide a specific MW portion of its Non-Spin award:

- The QSE will be sent a Resource-specific Dispatch Instruction for the portion of the On-Line Generation Resource that is available through power augmentation capacity.  The Dispatch Instruction will include the expected amount of capacity that will be available for SCED and the anticipated duration of the deployment.

- Upon receipt of the Dispatch Instruction, the QSE must take actions to make the deployed Non-Spin capacity On-Line and available for SCED Dispatch within 30 minutes.  Further, the QSE must update the Resource’s power augmentation capacity telemetry to reflect the Non-Spin deployment.

- The QSE must, at a minimum, ensure that the Normal Ramp Rate represented by the Resource’s ramp rate curve is sufficient to allow SCED to fully Dispatch the Resource’s Non-Spin Resource award within 30 minutes, regardless of whether or not the Resource uses power augmentation to provide the service.

3.3	Load Resource that is not a CLR with Non-Spin award

- The QSE must show the Load Resource’s telemetered Resource Status as On-Line (ONL) and, if equipped with an under-frequency relay, the relay should not be armed and the status should indicate Disabled.

- Load Resources that are not CLRs and Generation Resources providing Off-Line Non-Spin will be deployed as a subgroup using a random sampling of Resources as defined in paragraph (7) of Nodal Protocol Section 6.5.7.6.2.3, Non-Spinning Reserve Service Deployment.

- The QSE will be sent a Resource-specific Dispatch Instruction for the Non-Spin deployment indicating a time and date stamp, QSE, Dispatch Asset Code, and Deployed MW.

- The Dispatch Instruction must include the expected amount of capacity that will be expected to be dropped by the Load Resource within 30 minutes.

- The Load Resources that are not CLRs shall be capable of being dispatched to their Non-Spin award within 30 minutes of a Dispatch Instruction and must, at a minimum, be capable of remaining deployed until recalled.

## 4.	Recall of Non-Spin Deployment

The deployed Non-Spin may be recalled in a manner that is expected to maintain the Non-Spin deployment margin described in Section 2, Non-Spin Deployment Triggers, such that it is greater than 1000 MW and PRC greater than or equal to 3200 MW.  Non-Spin deployments may be recalled partially or all at once, at ERCOT’s discretion.

Upon the recall of a Non-Spin deployment, the following steps should be taken:

- After recall, the QSE for a Generation Resource will be allowed to use normal shutdown procedures to take the Generation Resource Off-Line if the QSE wants to shut down the Resource.

- A QSE for a Generation Resource that was previously Off-Line will be allowed to keep the Generation Resource On-Line after the minimum On-Line time.

- A QSE for a Generation Resource with power augmentation capacity may continue to keep the ducts fired upon recall of Non-Spin deployment or restore the power augmentation capacity.

- If Non-Spin has been deployed in the Houston area to help manage the N_H Voltage Stability Limit, the deployments will be recalled once reliability margins have been restored to a manageable level.

## 5.	Revision Process

Revisions to the Non-Spinning Reserve Deployment and Recall Procedure shall be made according to the approval process as prescribed in Protocol Section 6.5.7.6.2.3, Non-Spinning Reserve Service Deployment.