ERCOT Nodal Protocols

Section 13:  Transmission and Distribution Losses

January 1, 2021

13	Transmission and Distribution Losses	13-1

13.1	Overview	13-1

13.1.1	Responsibility for Transmission and Distribution Losses	13-1

13.1.2	Calculation of Losses for Settlement	13-2

13.2	Transmission Losses	13-2

13.2.1	Forecasted Transmission Loss Factors	13-2

13.2.2	Deemed Actual Transmission Loss Factors	13-2

13.2.3	Transmission Loss Factor Calculations	13-2

13.2.4	Seasonal Transmission Loss Factor Calculation	13-3

13.2.5	Loss Monitoring	13-4

13.3	Distribution Losses	13-4

13.3.1	Loss Factor Calculation	13-5

13.3.2	Loss Monitoring	13-5

13.4	Special Loss Calculations for Settlement and Analysis	13-5

13.4.1	Deemed Actual Transmission Losses for NOIEs	13-5

# Transmission and Distribution Losses

13.1	Overview

(1)	This section sets forth the method for calculating Transmission and Distribution Losses (T&D Losses) and responsibilities of ERCOT, Qualified Scheduling Entities (QSEs), Transmission Service Providers (TSPs) and Distribution Service Providers (DSPs) with respect to T&D Losses.

13.1.1	Responsibility for Transmission and Distribution Losses

(1)	T&D Losses are the responsibility of each QSE representing Load.  ERCOT shall allocate T&D Losses to Load at the appropriate aggregate level as part of the data aggregation process to calculate the Load obligation of QSEs for settlement purposes.

(2)	ERCOT shall forecast Transmission Loss Factors (TLFs) and post them to the ERCOT website by 0600 of the Day-Ahead period.  ERCOT shall forecast the ERCOT-wide TLFs as a percentage of Load for each Settlement Interval of the Operating Day.  By the close of business on the day following the Operating Day, ERCOT shall also calculate TLFs for each Settlement Interval using the actual system Load for that Settlement Interval and shall post the resulting deemed actual TLFs to the settlement system and the ERCOT website.

(3)	ERCOT shall forecast Settlement Interval Distribution Loss Factors (DLFs) and post them to the ERCOT website by 0600 of the Day Ahead period.  ERCOT shall forecast the Settlement Interval DLFs as a percentage of Load for each Settlement Interval of the Operating Day.  On the day following the Operating Day, ERCOT shall also calculate Settlement Interval DLFs using actual system Load for that Settlement Interval and post the resulting deemed actual Settlement Interval DLFs to the settlement system and the ERCOT website.

(4)	Distribution loss coefficients, and the calculation methodology from which they are derived, will be subject to audit by ERCOT for accurate and consistent application.  Non-Opt-in Entities (NOIEs) with Interval Data Recorders (IDRs) at the settlement point of delivery are not required to provide Distribution loss coefficients and calculation methodology.

(5)	In the special case where there are distribution facilities upstream from a wholesale NOIE or External Load Serving Entity (ELSE) settlement IDR, that settlement IDR will be compensated for line and transformer losses between the IDR and the ERCOT Transmission Grid to account for the Distribution Losses.  The NOIE or ELSE will be then treated as a transmission level NOIE or ELSE.  Calculations are subject to review by ERCOT.  Since loss compensation is included in the wholesale settlement IDR, the TSP and/or DSP providing upstream wheeling facilities may need to offer wholesale wheeling tariffs excluding the losses that have already been compensated for.

13.1.2	Calculation of Losses for Settlement

(1)	ERCOT shall use the deemed actual Settlement Interval DLFs applicable to each ESI ID and the deemed actual Settlement Interval TLFs when adjusting aggregated Load for losses to determine the QSE total Load obligations.

13.2	Transmission Losses

13.2.1	Forecasted Transmission Loss Factors

(1)	The forecasted Transmission Loss Factor (TLF) for each interval in the Operating Day shall be a linear interpolation or extrapolation using the on-peak and the off-peak TLFs and the corresponding forecast of ERCOT System Load during the same interval to calculate the loss factors.

(2)	At 0600 of the Day-Ahead period, ERCOT shall forecast a TLF for each Settlement Interval of the Operating Day and post on the ERCOT website the forecasted TLFs which correspond to the Operating Day forecast.  The source of the on-peak and off-peak losses are the ERCOT load flow base cases for the applicable season.  For the purpose of Section 13.2, Transmission Losses, “season” is defined as those set forth in item (1) of Section 13.2.4, Seasonal Transmission Loss Factor Calculation.

13.2.2	Deemed Actual Transmission Loss Factors

(1)	ERCOT shall determine the deemed actual TLF for each interval in the Operating Day, by use of a linear interpolation or extrapolation using the on-peak and the off-peak TLFs corresponding to the actual ERCOT System Load during the interval.

(2)	The day after the Operating Day, ERCOT shall calculate deemed actual TLFs for each Settlement Interval of the Operating Day and publish the TLFs to be used in Settlement calculations.

(3)	ERCOT shall use the TLFs corresponding to the on-peak and off-peak base case ERCOT System Loads during the applicable seasons as the basis for the ERCOT-wide deemed actual TLFs.  ERCOT will post TLFs to the ERCOT website by 0600 two days after the Operating Day.

13.2.3	Transmission Loss Factor Calculations

(1)	The following formulas shall be used to translate the seasonal on-peak and off-peak TLFs into Settlement Interval TLFs.

TLFi	=	(SSC * SIELi) + SIC

| Variable | Unit | Description |
| --- | --- | --- |
| i | none | Interval |
| TLFi | none | Transmission Loss Factor for a Settlement Interval |
| SIELi | MWh | Settlement Interval ERCOT System Load (forecasted or actual) |
| SSC | none | Seasonal Slope Coefficient |
| SIC | none | Seasonal Intercept Coefficient |

And

SSC 	=	(SONLF – SOFFLF)/(SONL-SOFFL)

SIC	=	[(SOFFLF*SONL)-(SONLF*SOFFL)]/(SONL-SOFFL)

| Variable | Unit | Description |
| --- | --- | --- |
| SONLF | none | Seasonal on-peak percent loss factor |
| SOFFLF | none | Seasonal off-peak percent loss factor |
| SONL | none | Seasonal on-peak Load value |
| SOFFL | none | Seasonal off-peak Load value |

13.2.4	Seasonal Transmission Loss Factor Calculation

(1)	Seasonal on-peak and off-peak TLFs are derived from the annually updated ERCOT on-peak and off-peak load flow base cases analysis by ERCOT.  Base cases reflect the most current data on the transmission system and Generation Resource Dispatch.  The ERCOT Transmission Grid topology and related Generation Resource Dispatch in the base cases are the critical factors in calculating losses.  Seasonal time periods are defined as follows:

(a)	Spring (March – May)

(b)	Summer (June – September)

(c)	Fall (October – November)

(d)	Winter (December – February)

(2)	ERCOT shall calculate seasonal TLFs by dividing ERCOT seasonal case Transmission Losses (60 kV system and higher) by the ERCOT seasonal base Load adjusted (reduced) for self-serve Load modeled in the case.  The resulting TLFs are expressed as a percentage of Load.

(3)	ERCOT shall post the seasonal TLFs to the ERCOT website prior to the start of the year for the next four seasons beginning with the Spring season.

13.2.5	Loss Monitoring

(1)	ERCOT shall monitor Transmission Losses annually and will investigate any abnormal loss factors.  ERCOT and TSPs shall use the cost of losses as one criterion in evaluating the need for transmission additions.

13.3	Distribution Losses

(1)	By October 30th of each year for the next calendar year, or two months prior to the posting of any update to the approved Distribution loss coefficients, codes, or calculation, each Distribution Service Provider (DSP), except Non-Opt-In Entities (NOIEs), shall calculate and provide ERCOT the annual Distribution loss coefficients to be applied to distribution voltage level Loads in its area of certification.  ERCOT shall review and approve the Distribution Loss Factor (DLF) calculation methodology used by each DSP prior to use of the loss coefficients for settlement purposes.  If the DLF calculation methodology does not conform with ERCOT’s interpretation of the Protocol criteria in this subsection, ERCOT will work with the DSP to correct the deficiency.  Until deficiencies are resolved, the last approved Distribution loss coefficients and the calculation methodology will be posted, and the last approved Distribution loss coefficients shall be used for settlement.  A DSP may only submit a change to the DLF calculation methodology annually or when a change in a DSP service area warrants an update to the approved DLF methodology based on the DSP internal evaluation.

(2)	The DSP shall assign a Distribution loss code to each Electric Service Identifier (ESI ID).  A maximum of five Distribution loss codes may be submitted for each DSP based upon ERCOT approved parameters, such as service voltages or number of transformations.

(3)	The following standards will be used to identify the Distribution loss code applicable to each ESI ID:

- T = Transmission connected Customers (no Settlement Interval DLF applied)

- A through E = Transmission and/or Distribution Service Provider (TDSP) defined Customer segment(s)

(4)	The DSPs, except NOIEs, are obligated to provide Distribution loss coefficients to ERCOT.  ERCOT will post the Distribution loss coefficients and calculation methodology, for each DSP.

(5)	Distribution loss information submitted by the DSP shall include:

(a)	The annual Distribution loss coefficients (F1, F2, and F3) for each Distribution loss code; and

(b)	The methodology upon which the calculation of the coefficients (F1, F2, and F3) was made.

(6)	A NOIE may provide ERCOT with the information detailed in paragraph (5) above.  If such information is provided, ERCOT shall calculate and post NOIE DSP DLFs using the same processes for the calculation and posting of competitive DSP DLFs.

13.3.1	Loss Factor Calculation

(1)	ERCOT shall use the Distribution loss coefficients submitted by the DSP to calculate the Settlement Interval DLFs.  Settlement Interval DLFs will be calculated from the data provided by DSPs as follows using the following equation:

SILFi = F1 * (SIELi/AAL) + F2 + F3 / (SIELi/AAL)

| Variable | Unit | Description |
| --- | --- | --- |
| i |  | interval |
| SILFi |  | Settlement Interval DLF |
| SIELi |  | Settlement Interval ERCOT System Load (forecasted or actual) |
| AAL |  | Annual Interval Average ERCOT System Load. The AAL is calculated using the total ERCOT Load stated in the most recent settlement during the period beginning on September 1 and ending August 31. ERCOT will provide the AAL to DSPs that are obligated to provide Distribution loss coefficients and calculation methodology to ERCOT, by September 15th of each year. |
| F1 , F2 , F3 |  | Distribution Loss coefficients determined by the Distribution Service Provider to allow calculation of its SILF from ERCOT System Load |

(2)	ERCOT shall use the deemed actual Settlement Interval DLFs calculated for each Settlement Interval of the Operating Day for settlement purposes.

13.3.2	Loss Monitoring

(1)	Distribution loss coefficients and the calculation methodology from which they are derived for all DSPs, except for NOIEs, will be submitted to ERCOT and will be subject to audit for accuracy and consistency of application.

13.4	Special Loss Calculations for Settlement and Analysis

13.4.1	Deemed Actual Transmission Losses for NOIEs

(1)	All Qualified Scheduling Entities (QSEs) representing Load, including Non-Opt-In Entities (NOIEs), will be responsible for Transmission Losses allocated in the manner described in these Protocols.  Those Entities using transmission tie line meters to determine Load will adjust the net meter readings to remove calculated Transmission Losses behind the meter in order to determine the Load responsibility of the Entity.  ERCOT will provide to Settlement the calculation of the losses behind the meters, for each interval, using actual system conditions for that interval.

(2)	The deemed actual Transmission Losses for NOIEs shall be a linear interpolation or extrapolation between the seasonal on-peak and the seasonal off-peak NOIE Transmission Loss Factors (TLFs) posted pursuant to paragraph (3) of Section 13.2.4, Seasonal Transmission Loss Factor Calculation, corresponding to the actual NOIE metered Load in the interval.