| NOGRR Number | 243 | NOGRR Title | Implementation of Seasonal Load Shed Tables |
| --- | --- | --- | --- |

| Date | November 2, 2022 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Eric Easton; Dennis Kunkel; Chris Gerety; Collin Martin |
| E-mail Address | eric.easton@centerpointenergy.com; dkkunkel@aep.com; christopher.gerety@tnmp.com; Collin.Martin@oncor.com |
| Company | “Joint TOs”: CenterPoint Energy Houston Electric (CEHE); AEP Service Corporation (AEPSC); Texas-New Mexico Power (TNMP); Oncor Electric Delivery (Oncor) |
| Phone Number | 713-207-2840; 361-813-4140; 505-280-3310; 817-215-6174 |
| Cell Number | N/A |
| Market Segment | IOU |

Comments

The Joint TOs respectfully submit the revisions below on top of the original NOGRR243 language. These Joint TOs comments delete references to removing transmission connected industrial Load from Transmission Operator (TO) Load shed obligation calculations, clarifies how ERCOT will calculate Seasonal Load shed obligation percentage values, aligns the winter Season months with Public Utility Commission of Texas (PUCT) defined winter Season months, and adds timeline requirements for ERCOT to update anticipated revisions to the Seasonal Load shed obligation percentages no later than March 31st of each year for the summer Load shed table and August 31st of each year for the winter Load shed table.

| Revised Cover Page Language |  |
| --- | --- |
| Revision Description | This NOGRR modifies the Load shed table of the Nodal Operating Guide to include separate Load shed obligations for the winter and summer Seasons to align with Senate Bill 3 directives. |
| Business Case | This NOGRR aligns the ERCOT Load shed obligation tables with the requirements established within Senate Bill 3 which directs ERCOT to modify the Load shed obligations to include winter and summer Seasons. The modifications proposed in this NOGRR adds a second Load Shed Table for the purposes of reflecting Seasonal differences and as a means to accommodate the intent of requirements associated with Senate Bill 3 for a Seasonal Load shed process. Seasonal differences in Load allocation percentages can result in disproportionate Load shedding for a ERCOT Energy Emergency Alert (EEA) Level 3 Load shed event. The amount of Load each TO/DSP is obligated to shed is identified in the Load shed table of Section 4.5.3.4, Load Shed Obligation, by entity based on peak Loads. |

Proposed Guide Language Revision

- 4.5.3.4	Load Shed Obligation

(1)	Each TO shall take and direct actions to ensure that ERCOT Load shed instructions are effectuated.  Each DSP shall comply with any reasonable instruction given by its TO to effectuate Load shed obligations.

(2)	Load shed obligation percentages for ERCOT EEA Level 3 Load shedding will be determined by calculating each TO’s Load as a percentage of the ERCOT System summer and winter peak 15 minute Demand interval.  For the purposes of this paragraph, TO Load will be the amount of Load being served by all of the Transmission and/or Distribution Service Providers (TDSPs) that the TO represents.  The calculations for summer and winter Load shed obligation percentage are as follows:

(a) 	The calculated Load shed obligation percentage for the summer Season will be based on the single highest coincident ERCOT System peak 15 minute Demand interval for the summer months of June through September as reflected in the 4-Coincident Peak (4-CP) data submitted by ERCOT to the Public Utility Commission of Texas (PUCT) for that year.  Anticipated revisions to the summer Load shed table shall be posted as described in paragraph (4) below no later than March 31st of each year based on data from the previous calendar year.

(b) 	The calculated Load shed obligation percentage for the winter Season will be based on the single highest coincident ERCOT System peak 15 minute Demand interval for the winter months of December through February as reflected at the time that ERCOT extracts the Load data for the winter Season from its settlement system.  Anticipated revisions to the winter Load shed table shall be posted as described in paragraph (4) below no later than August 31st of each year based on data from December of the previous calendar year and January through February of the current year.

(3)	The summer Load shed table will be used during a hot weather Load shed event and the winter Load shed table will be used during a cold weather Load shed event.  However, ERCOT will determine, in its sole discretion, whether an EEA event will be treated as a hot weather or cold weather Load shed event based on the weather conditions.  When ERCOT directs TOs to shed Load, it will specify which Seasonal Load shed table applies for the Load shed event.

(4) 	ERCOT shall maintain the Seasonal Load shed tables reflecting each TO’s total Load shed obligation on the ERCOT website.  The Load shed obligation percentages will be reviewed by ERCOT and revised as described above, or as otherwise deemed appropriate by ERCOT, to reflect any new or changed TO designation by a DSP.  Adjustments to the Load shed obligations due to changes in TO designations will be performed using the same Load data upon which the table was based.  Following ERCOT’s Seasonal peak Load reviews or ERCOT’s receipt of any new or changed TO designation, ERCOT shall post any anticipated revisions to the Load shed tables on the ERCOT website. ERCOT shall issue a Market Notice announcing the posting of the revisions at least ten days prior to the effective date of the revisions or as soon as practicable if ERCOT determines there is a need to correct the Market Notice less than ten days before the effective date.