| NPRR Number | 1138 | NPRR Title | Communication of Capability and Status of Online IRRs at 0 MW Output |
| --- | --- | --- | --- |
| Date |  | October 18, 2022 |  |
| Submitter’s Information |  |  |  |
| Name |  | Ian Haley |  |
| E-mail Address |  | Ian.Haley@VistraCorp.com |  |
| Company |  | Luminant Generation Company LLC |  |
| Phone Number |  | 512-673-9655 |  |
| Cell Number |  |  |  |
| Market Segment |  | Independent Generator |  |

Comments

Luminant Generation Company LLC (Luminant) appreciates ERCOT’s work and the discussions on this Nodal Protocol Revision Request (NPRR).  Providing ERCOT with accurate information should be the goal of every Qualified Scheduling Entity (QSE).  However, providing accurate information on the reliability functions of the grid is an absolute.  Luminant is very supportive of the requirements to correctly update Resources’ Reactive Power capabilities to ERCOT.

Through discussions at various stakeholder meetings, ERCOT has stated that they support an Inverter-Based Resource’s (IBR’s) ability to “open their breakers” if the Resource is unwilling to or is not currently capable of providing Reactive Power.  Luminant provides these comments to capture the concept of allowing IBRs to open their virtual or pseudo breakers.  The opening of these breakers will allow for an accurate representation of the IBR’s capabilities.  Additionally, Luminant has provided guidelines to ensure that this is correctly communicated to ERCOT, the Resource is not producing real power, and the Resource is not being compensated for carrying Ancillary Services.

These comments are filed on top of the 10/13/22 ERCOT comments.  Luminant looks forward to additional discussion on this important NPRR.

Revised Cover Page Language

| Revision Description | This Nodal Protocol Revision Request (NPRR) requires each Resource Entity to ensure the reactive capability curve for any Intermittent Renewable Resource (IRR) accurately reflects the IRR’s reactive capability when it is not providing real power or is operating at lower levels of real power output. |
| --- | --- |
| Business Case | This NPRR introduces requirements to improve consistency in the reporting of IRR reactive capabilities. ERCOT has observed Market Participants reporting reactive capabilities at low levels of output and at no real power output in a variety of inconsistent ways. For example, Resource Entities for some IRRs have reported no reactive capability below 10% of nameplate capacity. While not required to have reactive capability at that level of real power output, IRRs generally have some capability below that level and ERCOT needs an accurate understanding of IRR capability for operations and planning purposes. ERCOT also needs to know an IRR’s Reactive Power capability when it is producing no real power. If an IRR can produce Reactive Power when not producing real power and it remains synchronized to the ERCOT System, ERCOT must coordinate that Reactive Power capability with other actions of the Transmission Service Provider (TSP) for real time voltage control in Real-Time Operations. If an IRR cannot produce Reactive Power when not producing real power, ERCOT must know that fact so it does not inadvertently depend on the IRR for Reactive Power for system balance. If an IRR cannot provide Reactive Power when not producing real power and the IRR is contributing to a voltage or stability event, the IRR may be disconnected consistent with paragraph (4)(e) of Section 3.15, Voltage Support. This NPRR requires each Resource Entity to ensure the reactive capability curves for its IRRs accurately reflect reactive capability at the lowest output level. This NPRR also standardizes IRR practices for setting Low Sustained Limit (LSL) values and providing telemetry of Automatic Voltage Regulator (AVR) status when IRR real power output is zero. Some IRRs demonstrate oscillations with unstable Reactive Power control at very low output levels due to equipment limitations, requiring the LSL be set at a slightly higher level to avoid the oscillations. However, in some cases, Resource Entities set the LSL values too high. Resource Entities representing IRRs must minimize equipment limitations to allow them to set the LSL as close to 0 MW as possible, including coordinating with the Original Equipment Manufacturer, if necessary. The standardization of these practices will help ensure ERCOT studies accurately identify voltage and stability issues and will provide greater clarity to ERCOT’s expectations for IRRs in the interconnection process. While this NPRR does not modify reactive capability requirements for IRRs, it requires that an IRR must utilize its reactive capability to maintain voltage at the Point of Interconnection Bus (POIB) even if the IRR is producing no real power when it remains synchronized to the ERCOT System. IRRs capable of providing reactive capability when producing no real power that are not synchronized to the ERCOT System would not have any requirements to maintain voltage at the POIB. |

Revised Proposed Protocol Language

- 3.15.3	Generation Resource Requirements Related to Voltage Support

(1)	Generation Resources required to provide VSS shall have and maintain Reactive Power capability at least equal to the Reactive Power capability requirements specified in these Protocols and the ERCOT Operating Guides.

(2)	Generation Resources providing VSS shall be compliant with the ERCOT Operating Guides for response to transient voltage disturbance.

(3)	Generation Resources providing VSS must meet technical requirements specified in Section 8.1.1.1, Ancillary Service Qualification and Testing, and the performance standards specified in Section 8.1.1, QSE Ancillary Service Performance Standards.

(4)	Each Generation Resource providing VSS shall operate with the unit’s Automatic Voltage Regulator (AVR) in the automatic voltage control mode unless specifically directed to operate in manual mode by ERCOT, or when the unit is telemetering its Resource Status as STARTUP, SHUTDOWN, or ONTEST, or the QSE determines a need to operate in manual mode due to an undue threat to safety, undue risk of bodily harm, or undue damage to equipment at the generating plant.

(5)	Each Generation Resource providing VSS shall maintain the Voltage Set Point established by ERCOT, the interconnecting TSP, or the TSP’s agent, subject to the Generation Resource’s operating characteristic limits, voltage limits, and within tolerances identified in paragraph (4) of Nodal Operating Guide Section 2.7.3.5, Resource Entity Responsibilities and Generation Resource Requirements.

(6)	The reactive capability required must be maintained at all times that the Generation Resource is On-Line.

(7)	Each QSE shall send to ERCOT, via telemetry, the AVR and Power System Stabilizer (PSS) status for each of its Generation Resources providing VSS.  For AVRs, an “On” status will indicate the AVR is on and set to regulate the Resource’s terminal voltage in the voltage control mode, and an “Off” status will indicate the AVR is off or in a manual mode.  For PSS, an “On” status will indicate the service is enabled and ready for service, and an “Off” status will indicate it is off or out of service.  Each QSE shall monitor the status of its Generation Resources’ regulators and stabilizers, and shall report status changes to ERCOT.

(8)	Each Resource Entity shall provide information related to the tuning parameters, local or inter-area, of any PSS installed at a Generation Resource.

(9)	The Resource Entity for an IRR synchronized to the ERCOT System and not capable of providing Reactive Power when not producing real power shall:

(a)	When capable of providing real power, set the IRR’s Low Sustained Limit (LSL) at the lowest MW level, not to exceed 1 MW, at which the IRR can provide stable Reactive Power after appropriate tuning of settings;

(b)	Ensure the lowest MW point on the submitted reactive capability curve reflects 0 MVAr leading and lagging reactive capability at 0 MW;

(c)	Ensure it sets the second-lowest MW point on the submitted reactive capability curve to accurately reflect the IRR’s leading and lagging reactive capability at its LSL when the LSL is not 0 MW; and

(d)	Send to ERCOT, via telemetry, an AVR status of “Off” when the IRR is synchronized to the ERCOT System and not producing Reactive Power.

(10)	The Resource Entity for an IRR synchronized to the ERCOT System and capable of providing Reactive Power when not producing real power shall:

(a)	Provide stable Reactive Power output at all MW levels at which the IRR has Reactive Power capability;

(b)	When capable of providing real power, set the IRR LSL to 0 MW;

(c)	Ensure the lowest MW point on the submitted reactive capability curve accurately reflects the IRR’s MVAr leading and lagging reactive capability when not producing real power;

(d)	Send to ERCOT, via telemetry, an AVR status of “On” when the IRR is synchronized to the ERCOT System, not producing real power and reactive control is working properly; and

(e)	Meet the requirements in paragraphs (2), (4), (5), and (7) above when the IRR is synchronized to the ERCOT System and not producing real power.

(11)	An Inverter-Based Resource (IBR) that is unwilling to or is not currently capable of providing any reactive capability may open its virtual or pseudo breakers provided that the following conditions are met:

(a)	The IBR is not carrying Ancillary Services;

(b)	The IBR is not producing real power; and

(c)	The IBR is telemetering its Resource Status as OFF.

[NPRR989 and NPRR1026: Replace applicable portions of Section 3.15.3 above with the following upon system implementation:] - 3.15.3 Generation Resource and Energy Storage Resource Requirements Related to Voltage Support (1) Generation Resources and ESRs required to provide VSS shall have and maintain Reactive Power capability at least equal to the Reactive Power capability requirements specified in these Protocols and the ERCOT Operating Guides. (2) Generation Resources and ESRs providing VSS shall be compliant with the ERCOT Operating Guides for response to transient voltage disturbance. (3) Generation Resources and ESRs providing VSS must meet technical requirements specified in Section 8.1.1.1, Ancillary Service Qualification and Testing, and the performance standards specified in Section 8.1.1, QSE Ancillary Service Performance Standards. (4) Each Generation Resource and ESR providing VSS shall operate with the unit’s Automatic Voltage Regulator (AVR) in the automatic voltage control mode unless specifically directed to operate in manual mode by ERCOT, or when the unit is telemetering its Resource Status as STARTUP, SHUTDOWN, or ONTEST, or the QSE determines a need to operate in manual mode due to an undue threat to safety, undue risk of bodily harm, or undue damage to equipment at the generating plant. (5) Each Generation Resource and ESR providing VSS shall maintain the Voltage Set Point established by ERCOT, the interconnecting TSP, or the TSP’s agent, subject to the Generation Resource’s or ESR’s operating characteristic limits, voltage limits, and within tolerances identified in paragraph (4) of Nodal Operating Guide Section 2.7.3.5, Resource Entity Responsibilities and Generation Resource Requirements. (6) The reactive capability required must be maintained at all times that the Generation Resource or ESR is On-Line. (7) Each QSE shall send to ERCOT, via telemetry, the AVR and Power System Stabilizer (PSS) status for each of its Generation Resources providing VSS. Each QSE shall send to ERCOT via telemetry the AVR status for each of its ESRs providing VSS. For AVRs, an “On” status will indicate the AVR is on and set to regulate the Resource’s terminal voltage in the voltage control mode, and an “Off” status will indicate the AVR is off or in a manual mode. For PSS, an “On” status will indicate the service is enabled and ready for service, and an “Off” status will indicate it is off or out of service. Each QSE shall monitor the status of its Generation Resources’ and ESRs’ regulators and stabilizers, and shall report status changes to ERCOT. (8) Each Resource Entity shall provide information related to the tuning parameters, local or inter-area, of any PSS installed at a Generation Resource. (9) If any individual Resource within a Self-Limiting Facility is incapable of meeting its Reactive Power requirement at the POI, the QSE must bring On-Line additional Resource(s) within the Self-Limiting Facility to provide VSS as specified in paragraph (4) of Section 3.15, Voltage Support, while respecting the limit on MW Injection. (10) The Resource Entity for an IRR synchronized to the ERCOT System and not capable of providing Reactive Power when not producing real power shall: (a) When capable of providing real power, set the IRR’s Low Sustained Limit (LSL) at the lowest MW level, not to exceed 1 MW, at which the IRR can provide stable Reactive Power after appropriate tuning of settings; (b) Ensure the lowest MW point on the submitted reactive capability curve reflects 0 MVAr capability leading and lagging at 0 MW; (c) Ensure it sets the second-lowest MW point on the submitted reactive capability curve to accurately reflect the IRR’s leading and lagging reactive capability at its LSL when the LSL is not 0 MW; and (d) Send to ERCOT, via telemetry, an AVR status of “Off” when the IRR is synchronized to the ERCOT System and not producing Reactive Power. (11) The Resource Entity for an IRR synchronized to the ERCOT System and capable of providing Reactive Power when not producing real power shall: (a) Provide stable Reactive Power output at all MW levels at which the IRR has Reactive Power capability; (b) When capable of providing real power, set the IRR LSL to 0 MW; (c) Ensure the lowest MW point on the submitted reactive capability curve accurately reflects the IRR’s MVAr leading and lagging reactive capability when not producing real power; (d) Send to ERCOT, via telemetry, an AVR status of “On” when the IRR is synchronized to the ERCOT System, not producing real power and reactive control is working properly; and (e) Meet the requirements in paragraphs (2), (4), (5), and (7) above when the IRR is synchronized to the ERCOT System and not producing real power. (12) An Inverter-Based Resource (IBR) that is unwilling to or is not currently capable of providing any reactive capability may open its virtual or pseudo breakers provided that the following conditions are met: (a) The IBR is not carrying Ancillary Services; (b) The IBR is not producing real power; and (c) The IBR is telemetering its Resource Status as OFF.

6.5.5.1	Changes in Resource Status

(1)	Each QSE shall notify ERCOT of a change in Resource Status via telemetry and through changes in the Current Operating Plan (COP) as soon as practicable following the change.

(2)	Each QSE shall promptly inform ERCOT when the operating mode of its Generation Resource’s Automatic Voltage Regulator (AVR) or Power System Stabilizer (PSS) is changed while the Resource is On-Line.  The QSE shall also provide the Resource’s AVR or PSS status logs to ERCOT upon request.

(3)	Each QSE shall immediately report to ERCOT and the TSP any inability of the QSE’s Generation Resource required to meet its reactive capability requirements in these Protocols.

(4)	For each Intermittent Renewable Resource (IRR) synchronized to the ERCOT System and not capable of providing real power due to a lack of fuel, the Resource Entity and QSE shall send ERCOT, via telemetry, a Real-Time On-Line status and HSL and LSL of 0.