| NOGRR Number | 243 | NOGRR Title | Implementation of Seasonal Load Shed Tables |
| --- | --- | --- | --- |
| Date of Decision |  | December 5, 2022 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Urgent – to allow NOGRR243 implementation prior to the 2022/2023 winter Season, thereby enabling Seasonal obligations to be reflected in Transmission Operator (TO)/Distribution Service Provider (DSP) winter 2022/2023 Load shed instructions from ERCOT |  |
| Proposed Effective Date |  | Upon system implementation |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Nodal Operating Guide Sections Requiring Revision |  | 4.5.3.4, Load Shed Obligation |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Operating Guide Revision Request (NOGRR) modifies the Load shed table to include separate Load shed obligations for the winter and summer Seasons to align with Senate Bill 3 directives. |  |
| Reason for Revision |  | Addresses current operational issues Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This NOGRR aligns the ERCOT Load shed obligation tables with the requirements established within Senate Bill 3 which directs ERCOT to modify the Load shed obligations to include winter and summer Seasons. The modifications proposed in this NOGRR adds a second Load shed table for the purposes of reflecting Seasonal differences and as a means to accommodate the intent of requirements associated with Senate Bill 3 for a Seasonal Load shed process. Seasonal differences in Load allocation percentages can result in disproportionate Load shedding for a ERCOT Energy Emergency Alert (EEA) Level 3 Load shed event. The amount of Load each TO/DSP is obligated to shed is identified in the Load shed table of Section 4.5.3.4 by entity based on peak Loads. |  |
| ROS Decision |  | On 11/7/22, ROS voted unanimously to grant NOGRR243 Urgent status; to recommend approval of NOGRR243 as amended by the 11/2/22 Joint TOs comments; and to forward NOGRR243 to TAC. All Market Segments participated in the vote. |  |
| Summary of ROS Discussion |  | On 11/7/22, ROS reviewed the 11/2/22 Joint TOs comments, 11/2/22 Golden Spread comments, and 11/7/22 PUCT Staff comments. Participants expressed concern that without advanced notice of which Seasonal Load shed table applies to the Load shed event, systems may not be able to respond as quickly. |  |
| TAC Decision |  | On 12/5/22, TAC voted to recommend approval of NOGRR243 as recommended by ROS in the 11/7/22 ROS Report as revised by TAC and the 11/22/22 Impact Analysis. There was one abstention from the Independent Retail Electric Provider (IREP) (Reliant) Market Segment. All Market Segments participated in the vote. |  |
| Summary of TAC Discussion |  | On 12/5/22, TAC reviewed the ERCOT Opinion, ERCOT Market Impact Statement, and Independent Market Monitor (IMM) Opinion for NOGRR243. Participants discussed additional edits to address concerns regarding the timing of when TOs are notified which Load shed table applies to a Load shed event and the effect this may have on system response times and the reliability implications. Participants also commented that a broader discussion on considering Real-Time conditions in the process needs to take place at ROS. |  |

| Opinions |  |
| --- | --- |
| Credit Work Group Review | Not applicable |
| Independent Market Monitor Opinion | IMM has no opinion on NOGRR243. |
| ERCOT Opinion | ERCOT supports approval of NOGRR243. |
| ERCOT Market Impact Statement | ERCOT Staff has reviewed NOGRR243 and believes the market impact for NOGRR243 aligns the ERCOT Load shed obligation tables with the requirements established in Senate Bill 3. |

| Sponsor |  |
| --- | --- |
| Name | Eric Easton; Dennis Kunkel; Chris Gerety |
| E-mail Address | eric.easton@centerpointenergy.com; dkkunkel@aep.com; christopher.gerety@tnmp.com |
| Company | “Joint TOs”: CenterPoint Energy Houston Electric (CEHE); AEP Service Corporation (AEPSC); Texas-New Mexico Power (TNMP) |
| Phone Number | 713-207-2840; 361-813-4140; 505-280-3310 |
| Cell Number | N/A |
| Market Segment | Investor Owned Utilities (IOUs) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Erin Wasik-Gutierrez |
| E-Mail Address | Erin.Wasik-Gutierrez@ERCOT.com |
| Phone Number | 413-886-2474 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Joint TOs 110222 | Deleted references to removing transmission connected industrial Load from TO Load shed obligation calculations, clarified how ERCOT will calculate Seasonal Load shed obligation percentage values, aligned with Public Utility Commission of Texas (PUCT) defined winter Season months, and added timeline requirements for ERCOT to update anticipated revisions to the Seasonal Load shed obligation percentages |
| Golden Spread 110222 | Supported two Load shed tables without excluding transmission Load from the calculation or Load shed obligation |
| PUCT Staff 110722 | Expressed support of limiting the issues to be addressed as contemplated in the 11/2/22 Joint TOs comments; and of aligning the Nodal Operating Guide with Senate Bill 3 Seasonal Load shed obligation directive |

Proposed Guide Language Revision

- 4.5.3.4	Load Shed Obligation

(1)	Each TO shall take and direct actions to ensure that ERCOT Load shed instructions are effectuated.  Each DSP shall comply with any reasonable instruction given by its TO to effectuate Load shed obligations.

(2)	Load shed obligation percentages for ERCOT EEA Level 3 Load shedding will be determined by calculating each TO’s Load as a percentage of the ERCOT System summer and winter peak 15 minute Demand interval.  For the purposes of this paragraph, TO Load will be the amount of Load being served by all of the Transmission and/or Distribution Service Providers (TDSPs) that the TO represents.  The calculations for summer and winter Load shed obligation percentage are as follows:

(a) 	The calculated Load shed obligation percentage for the summer Season will be based on the single highest coincident ERCOT System peak 15 minute Demand interval for the summer months of June through September as reflected in the 4-Coincident Peak (4-CP) data submitted by ERCOT to the Public Utility Commission of Texas (PUCT) for that year.  Anticipated revisions to the summer Load shed table shall be posted as described in paragraph (4) below no later than March 31st of each year based on data from the previous calendar year.

(b) 	The calculated Load shed obligation percentage for the winter Season will be based on the single highest coincident ERCOT System peak 15 minute Demand interval for the winter months of December through February as reflected at the time that ERCOT extracts the Load data for the winter Season from its settlement system.  Anticipated revisions to the winter Load shed table shall be posted as described in paragraph (4) below no later than August 31st of each year based on data from December of the previous calendar year and January through February of the current year.

(3)	The summer Load shed table will be used during a hot weather Load shed event and the winter Load shed table will be used during a cold weather Load shed event.  ERCOT will determine, in its sole discretion, whether an EEA event will be treated as a hot weather or cold weather Load shed event based on the weather conditions.  The summer and winter Load shed time periods will be published annually with the updated obligation tables in paragraph (2) above.  In addition, if ERCOT issues an Operating Condition Notice (OCN), it will notify Market Participants which Load shed table would apply to the potential Load shed event.  When ERCOT directs TOs to shed Load, it will specify which Load shed table applies for the Load shed event.   ERCOT shall use the same Load shed table for the duration of a Load shed event.

(4) 	ERCOT shall maintain the Seasonal Load shed tables reflecting each TO’s total Load shed obligation on the ERCOT website.  The Load shed obligation percentages will be reviewed by ERCOT and revised as described above, or as otherwise deemed appropriate by ERCOT, to reflect any new or changed TO designation by a DSP.  Adjustments to the Load shed obligations due to changes in TO designations will be performed using the same Load data upon which the table was based.  Following ERCOT’s Seasonal peak Load reviews or ERCOT’s receipt of any new or changed TO designation, ERCOT shall post any anticipated revisions to the Load shed tables on the ERCOT website. ERCOT shall issue a Market Notice announcing the posting of the revisions at least ten days prior to the effective date of the revisions or as soon as practicable if ERCOT determines there is a need to correct the Market Notice less than ten days before the effective date.

The above variables are defined as follows: