| # NPRR Number | 1132 | NPRR Title | Communicate Operating Limitations during Cold and Hot Weather Conditions |
| --- | --- | --- | --- |
| Date of Decision |  | January 26, 2023 |  |
| Action |  | Approved |  |
| Timeline |  | Normal |  |
| Effective Date |  | February 1, 2023 |  |
| Priority and Rank Assigned |  | Priority – 2023; Rank – 340 |  |
| Nodal Protocol Sections Requiring Revision |  | 3.9, Current Operating Plan (COP) 3.10.6, QSE and Resource Entity Responsibilities 6.5.9.3.2, Advisory |  |
| Related Documents Requiring Revision/Related Revision Requests |  | Resource Registration Glossary Revision Request (RRGRR) 032, Related to NPRR1132, Communicate Operating Limitations during Cold and Hot Weather Conditions |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) specifies that during local cold weather conditions, each Qualified Scheduling Entity (QSE) must update its Generation Resources’ and Energy Storage Resources’ (ESRs’) Current Operating Plan (COP), Real-Time telemetry, and Outage and derate reporting to reflect any cold-weather limitations. This NPRR also requires each Resource Entity to provide Resource-specific cold weather minimum temperature limits, hot weather maximum temperature limits, and alternate fuel capability information in its Resource Registration data submitted pursuant to Planning Guide Section 6.8.2, Resource Registration Process, and update this information as necessary. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | North American Electric Reliability Corporation (NERC) Reliability Standards IRO-010-4, Reliability Coordinator Data Specification and Collection, and TOP-003-5, Operational Reliability Data, will become effective April 1, 2023. These NERC Reliability Standards will require responsible Entities to communicate generating unit operating limitations during forecasted cold weather conditions to ERCOT (the Reliability Coordinator, Balancing Authority, and a Transmission Operator (TO) for the ERCOT interconnection) for use in its operational planning analysis, Real-Time monitoring, Real-Time assessments, and other analysis functions. These NERC Reliability Standards will also require responsible Entities to provide ERCOT each generating unit’s minimum design temperature, minimum historical operating temperature, or minimum operating temperature determined by an engineering analysis. ERCOT is including these requirements in the ERCOT Protocols, which are part of ERCOT’s documented data specification. ERCOT is requiring that the minimum historical operating temperature be provided for every Generation Resource or ESR that has operated at least one complete winter. ERCOT is also requiring that each Resource Entity provide at least one of the other two minimum temperatures — design temperature or ambient temperature limit determined by an engineering analysis — for each Generation Resource or ESR. This will help support additional ERCOT analysis of risks of Resource unavailability due to the cold weather. Given the impact of this information on ERCOT’s situational awareness, it is important that Resource Entities ensure this information is as accurate as possible. Beyond the cold-weather operating information required by NERC Reliability Standards IRO-010-4 and TOP-003-5, ERCOT is also supplementing its documented data specification to include hot weather maximum temperatures and information concerning alternate fuel capabilities. The proposed dates for annual review and updates are meant to allow time for this information to potentially inform ERCOT’s seasonal studies and assessments. Before this NPRR is implemented, ERCOT intends to issue a request for information (RFI) to Resource Entities to obtain this information for purpose of assessing vulnerabilities ahead of winter 2022-23. |  |
| PRS Decision |  | On 5/11/22, PRS voted unanimously to table NPRR1132 and refer the issue to ROS and WMS. All Market Segments participated in the vote. On 10/13/22, PRS voted to recommend approval of NPRR1132 as amended by the 10/11/22 Luminant comments. There was one abstention from the Consumer (Occidental) Market Segment. All Market Segments participated in the vote. On 11/11/22, PRS voted unanimously to endorse and forward to TAC the 10/13/22 PRS Report as amended by the 10/14/22 ERCOT comments and 11/1/22 Revised Impact Analysis for NPRR1132 with a recommended priority of 2023 and rank of 340. All Market Segments participated in the vote. |  |
| Summary of PRS Discussion |  | On 5/11/22 participants debated the usefulness of historical temperature limits, particularly outside of other factors such as humidity and wind direction. On 10/13/22, participants discussed the 10/10/22 LCRA comments and the 10/11/22 Luminant comments. On 11/11/22, participants reviewed the 10/14/22 ERCOT comments and the 11/1/22 Revised Impact Analysis. |  |
| TAC Decision |  | On 12/5/22, TAC voted to recommend approval of NPRR1132 as recommended by PRS in the 11/11/22 PRS Report as amended by the 12/2/22 ERCOT comments. There was one abstention from the Independent Retail Electric Provider (IREP) Market Segment. All Market Segments participated in the vote. |  |
| Summary of TAC Discussion |  | On 12/5/22, TAC reviewed the ERCOT Opinion, ERCOT Market Impact Statement, and Independent Market Monitor (IMM) Opinion for NPRR1132; and the 12/2/22 ERCOT comments. |  |
| Board Decision |  | On 12/20/22, the ERCOT Board voted unanimously to recommend approval of NPRR1132 as recommended by TAC in the 12/5/22 TAC Report. |  |
| PUCT Decision |  | On 1/26/23, the PUCT approved NPRR1132 and accompanying ERCOT Market Impact Statement as presented in Project No. 54445, Review of Rules Adopted by the Independent Organization. |  |

| Opinions |  |
| --- | --- |
| Credit Review | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR1132 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |
| Independent Market Monitor Opinion | IMM has no opinion on NPRR1132. |
| ERCOT Opinion | ERCOT supports approval of NPRR1132. |
| ERCOT Market Impact Statement | ERCOT Staff has reviewed NPRR1132 and believes the market impact for NPRR1132 fulfills NERC Reliability Standards IRO-010-4 and TOP-003-5 for cold weather conditions, which will become effective April 1, 2023, and further supplements data specifications to include hot weather maximum temperature and information concerning fuel capabilities. |

| Sponsor |  |
| --- | --- |
| Name | Nitika Mago / Stephen Solis |
| E-mail Address | Nitika.Mago@ercot.com / Stephen.Solis@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6601 / 512-248-6772 |
| Cell Number |  |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Brittney Albracht |
| E-Mail Address | Brittney.Albracht@ercot.com |
| Phone Number | 512-225-7027 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| WMS 060622 | Requested PRS continue to table NPRR1132 for further review by the Wholesale Market Working Group (WMWG) |
| ROS 060622 | Requested PRS continue to table NPRR1132 for further review by the Operations Working Group (OWG) |
| ERCOT 090922 | Incorporated feedback from recent requests for information to limit historical ambient temperatures to a finite period of time; added language to include “hot” weather temperature limits; separated the minimum and maximum ambient temperatures to include one maximum/minimum temperature without a Forced Outage or failure to start and a second maximum/minimum temperature without a Forced Derate greater than 10 MW and 5% of the summer/winter Seasonal net max sustainable rating |
| ERCOT 092322 | Addressed various consistency and language errors in the 9/9/22 ERCOT comments |
| ROS 100422 | Endorsed NPRR1132 as amended by the 9/23/22 ERCOT comments as revised by ROS |
| LCRA 101022 | Proposed language clarifying that QSEs shall perform an update when ERCOT issues an Advisory due to forecasted or actual cold or hot weather, and that QSEs update the impact due to relevant weather conditions |
| Luminant 101122 | Proposed language to address concerns for the costs associated with engineering analyses requirements beyond the NERC requirement; suggested moving new paragraph (9) in Section 3.9 to Section 6.5.9.3.2 |
| WMS 101222 | Endorsed NPRR1132 as amended by the 10/11/22 Luminant comments |
| ERCOT 101422 | Proposed language to afford that if a temperature determined by an engineering analysis was never provided, and only a design temperature was provided, the design temperature must be reviewed at least once every seven years; proposed new paragraph (9) remain in Section 3.9 |
| ERCOT 120222 | Clarified the periodicity of review of the hot and cold weather temperature limits in paragraph (3) of Section 3.10.6; also clarified that any updates to Resource Registration data based on these reviews must be provided within 30 days of identifying any change in the data, rather than by a specified date each year |

Market Rules Notes

Please note the baseline Protocol language in the following sections have been updated to reflect the incorporation of the following NPRRs into the Protocols:

- NPRR1120, Create Firm Fuel Supply Service

  - Section 3.9 (incorporated 10/14/22)

Proposed Protocol Language Revision

3.9	Current Operating Plan (COP)

(1)	Each Qualified Scheduling Entity (QSE) that represents a Resource must submit a Current Operating Plan (COP) under this Section.

(2)	ERCOT shall use the information provided in the COP to calculate the High Ancillary Service Limit (HASL) and Low Ancillary Service Limit (LASL) for each Resource for the Reliability Unit Commitment (RUC) processes.

[NPRR1007: Replace paragraph (2) above with the following upon system implementation of the Real-Time Co-Optimization (RTC) project:] (2) ERCOT shall use the information provided in the COP to calculate operating limits and Ancillary Service capabilities for each Resource for the Reliability Unit Commitment (RUC) processes.

(3)	ERCOT shall monitor the accuracy of each QSE’s COP as outlined in Section 8, Performance Monitoring.

(4)	A QSE must notify ERCOT that it plans to have a Resource On-Line by means of the COP using the Resource Status codes listed in paragraph (5)(b)(i) of Section 3.9.1, Current Operating Plan (COP) Criteria.  The QSE must show the Resource as On-Line with a Resource Status of ONRUC, indicating a RUC process committed the Resource for all RUC-Committed Intervals.  A QSE may only use a RUC-committed Resource during that Resource’s RUC-Committed Interval to meet the QSE’s Ancillary Service Supply Responsibility if the Resource has been committed by the RUC process to provide Ancillary Service.

(5)	To reflect changes to a Resource’s capability, each QSE shall report by exception, changes to the COP for all hours after the Operating Period through the rest of the Operating Day.

(6)	When a QSE updates its COP to show changes in Resource Status, the QSE shall update for each On-Line Resource, either an Energy Offer Curve under Section 4.4.9, Energy Offers and Bids, or Output Schedule under Section 6.4.2, Output Schedules.

(7)	Each QSE, including QSEs representing Reliability Must-Run (RMR) Units, Firm Fuel Supply Service Resources (FFSSRs), or Black Start Resources, shall submit a revised COP reflecting changes in Resource availability as soon as reasonably practicable, but in no event later than 60 minutes after the event that caused the change.

(8)	Each QSE representing a Qualifying Facility (QF) must submit a Low Sustained Limit (LSL) that represents the minimum energy available, in MW, from the unit for economic dispatch based on the minimum stable steam delivery to the thermal host plus a justifiable reliability margin that accounts for changes in ambient conditions.

(9)	When ERCOT issues a communication in the form of an Operating Condition Notice (OCN), Advisory, Watch, or Emergency Notice due to forecasted or actual cold or hot weather, for each Generation Resource and Energy Storage Resource (ESR) a QSE represents, the QSE shall update the COP, Real-Time telemetry, and Outage or derate reporting to reflect any Resource-specific operating limitations based on: (i) capability and availability; (ii) fuel supply or inventory concerns, including fuel switching capabilities; or (iii) environmental constraints and the impact on the Generation Resource or ESR due to the weather conditions.  QSEs shall provide these updates in accordance with Sections 3.1.4, Communications Regarding Resource and Transmission Facility Outages; 3.10.7.5, Telemetry Requirements; 3.9, Current Operating Plan (COP); 3.9.1, Current Operating Plan (COP) Criteria; and Nodal Operating Guide Section 7.3, Telemetry.

3.10.6	QSE and Resource Entity Responsibilities

(1)	Resource Entities shall provide Resource Registration data pursuant to Planning Guide Section 6.8.2, Resource Registration Process, to ERCOT and to TSPs upon request.  The Resource Registration data will contain information describing each Generation Resource, SOG, and Load Resource that it represents under Section 3.10.7.2, Modeling of Resources and Transmission Loads.

[NPRR995: Replace paragraph (1) above with the following upon system implementation:] (1) Resource Entities shall provide Resource Registration data pursuant to Planning Guide Section 6.8.2, Resource Registration Process, to ERCOT and to TSPs upon request. The Resource Registration data will contain information describing each Generation Resource, SOG, SOESS, and Load Resource that it represents under Section 3.10.7.2, Modeling of Resources and Transmission Loads.

(2)	QSEs shall ensure availability of telemetry to generation and transmission equipment its Resource Entity owns at ERCOT’s request to maintain observability and redundancy requirements as specified herein, and under Section 3.10.7.5, Telemetry Requirements.  ERCOT shall request such additions when a lack of data telemetry has caused, or can be demonstrated to result in, inaccuracies between Real-Time measurements and modeling outcomes that could result in incorrect LMP prices or potential reliability problems.

(3) 	For each Generation Resource and Energy Storage Resource (ESR), Resource Entities shall provide ERCOT the following temperature data:

(a) 	Cold weather temperature limits:

(i) 	Minimum historical ambient dry bulb temperature in degrees Fahrenheit at which the Resource has operated without a Forced Outage or Startup Loading Failure due to cold weather after at least one complete winter Peak Load Season following the Resource’s Initial Synchronization date based on the previous five calendar years of historical data; and

(ii) 	Minimum historical ambient dry bulb temperature in degrees Fahrenheit at which the Resource has operated without experiencing a Forced Derate greater than 10 MW and 5% of its winter Seasonal net maximum rating due to cold weather after at least one complete winter Peak Load Season following the Resource’s Initial Synchronization date based on the previous five calendar years of historical data; and

(iii)  	At least one of the following:

(A)	Minimum ambient dry bulb temperature in degrees Fahrenheit at which the Resource was designed to operate without a Forced Derate greater than 10 MW and 5% of its winter Seasonal net maximum sustainable rating; or

(B)	Minimum ambient dry bulb temperature in degrees Fahrenheit at which the Resource can operate without a Forced Derate greater than 10 MW and 5% of its winter Seasonal net maximum sustainable rating determined by an engineering analysis; and

(iv)  	At least one of the following:

(A) 	Minimum ambient dry bulb temperature in degrees Fahrenheit at which the Resource was designed to operate without a Forced Outage or Startup Loading Failure; or

(B)	Minimum ambient dry bulb temperature in degrees Fahrenheit at which the Resource can operate without a Forced Outage or Startup Loading Failure determined by an engineering analysis.

(b) 	Hot weather temperature limits:

(i)	Maximum historical ambient dry bulb temperature in degrees Fahrenheit at which the Resource has operated without experiencing a Forced Outage or Startup Loading Failure due to hot weather after at least one complete summer Peak Load Season following the Resource’s Initial Synchronization date based on the previous five years of historical data; and

(ii)	Maximum historical ambient dry bulb temperature in degrees Fahrenheit at which the Resource has operated without experiencing a Forced Derate greater than 10 MW and 5% of its summer Seasonal net maximum sustainable rating due to hot weather after at least one complete summer Peak Load Season following the Resource’s Initial Synchronization date based on the previous five calendar years of historical data; and

(iii)	At least one of the following:

(A)	Maximum ambient dry bulb temperature in degrees Fahrenheit at which the Resource was designed to operate without a Forced Derate greater than 10 MW and 5% of its summer Seasonal net maximum sustainable rating; or

(B)	Maximum ambient dry bulb temperature in degrees Fahrenheit at which the Resource can operate without a Forced Derate greater than 10 MW and 5% of its summer Seasonal net maximum sustainable rating, determined by an engineering analysis; and

(iv)	At least one of the following:

(A)	Maximum ambient dry bulb temperature in degrees Fahrenheit at which the Resource was designed to operate without a Forced Outage or Startup Loading Failure; or

(B) 	Maximum ambient dry bulb temperature in degrees Fahrenheit at which the Resource can operate without a Forced Outage or Startup Loading Failure, determined by an engineering analysis.

(4)  	Each Resource Entity shall review at least annually the temperatures described in paragraphs (3)(a)(i), (3)(a)(ii), (3)(b)(i), and (3)(b)(ii) above and shall update each Resource’s Registration data within 30 days of identifying any change in these temperatures.

(5)	Each Resource Entity shall review at least once every seven years the temperatures described in paragraphs (3)(a)(iii), (3)(a)(iv), (3)(b)(iii), and (3)(b)(iv) above and shall update each Resource’s Registration data within 30 days of identifying any change in these temperatures.

(6)	Resource Entities shall update each Generation Resource’s alternate fuel information within 30 days of any changes to the alternate fuel information.

6.5.9.3.2	Advisory

(1)	An Advisory is the second of three levels of communication issued by ERCOT in anticipation of a possible Emergency Condition.

(2)	ERCOT shall issue an Advisory for reasons such as, but not limited to, the following:

(a)	When it recognizes that conditions are developing or have changed and more Ancillary Services will be needed to maintain current or near-term operating reliability;

(b)	When weather or ERCOT System conditions require more lead-time than the normal DAM allows;

(c)	When communications or other controls are significantly limited; or

(d)	When ERCOT Transmission Grid conditions are such that operations within security criteria as defined in the Operating Guides are not likely or possible because of Forced Outages or other conditions unless a Constraint Management Plan (CMP) exists.

(3)	The Advisory must communicate existing constraints.  ERCOT shall notify TSPs and QSEs of the Advisory, and QSEs shall notify appropriate Resources and Load Serving Entities (LSEs).  ERCOT shall communicate with TSPs as needed to confirm their understanding of the condition and to determine the availability of Transmission Facilities.  For the purposes of verifying submitted information, ERCOT may communicate with QSEs.

[NPRR857: Replace paragraph (3) above with the following upon system implementation and satisfying the following conditions: (1) Southern Cross provides ERCOT with funds to cover the entire estimated cost of the project; and (2) Southern Cross has signed an interconnection agreement with a TSP and the TSP gives ERCOT written notice that Southern Cross has provided it with: (a) Notice to proceed with the construction of the interconnection; and (b) The financial security required to fund the interconnection facilities:] (3) The Advisory must communicate existing constraints. ERCOT shall notify TSPs, DCTOs, and QSEs of the Advisory, and QSEs shall notify appropriate Resources and Load Serving Entities (LSEs). ERCOT shall communicate with TSPs and DCTOs as needed to confirm their understanding of the condition and to determine the availability of Transmission Facilities. For the purposes of verifying submitted information, ERCOT may communicate with QSEs.

(4)	Although an Advisory is for information purposes, ERCOT may exercise its authority, in such circumstances, to increase Ancillary Service requirements above the quantities originally specified in the Day-Ahead in accordance with procedures.  ERCOT may require information from QSEs representing Resources regarding the Resources’ fuel capabilities.  Requests for this type of information shall be for a time period of no more than seven days from the date of the request.  The specific information that may be requested shall be defined in the Operating Guide.  QSEs representing Resources shall provide the requested information in a timely manner, as defined by ERCOT at the time of the request.

(5)	When an Advisory is issued for PRC below 3,000 MW and ERCOT expects system conditions to deteriorate to the extent that an EEA Level 2 or 3 may be experienced, ERCOT shall evaluate constraints active in SCED and determine which constraints have the potential to limit generation output.

(a)	Upon identification of such constraints, ERCOT shall coordinate with the TSPs that own or operate the overloaded Transmission Facilities associated with those constraints, as well as the Resource Entities whose generation output may be limited, to determine whether:

[NPRR857: Replace paragraph (a) above with the following upon system implementation and satisfying the following conditions: (1) Southern Cross provides ERCOT with funds to cover the entire estimated cost of the project; and (2) Southern Cross has signed an interconnection agreement with a TSP and the TSP gives ERCOT written notice that Southern Cross has provided it with: (a) Notice to proceed with the construction of the interconnection; and (b) The financial security required to fund the interconnection facilities:] (a) Upon identification of such constraints, ERCOT shall coordinate with the TSPs and DCTOs that own or operate the overloaded Transmission Facilities associated with those constraints, as well as the Resource Entities whose generation output may be limited, to determine whether:

(i)	A 15-Minute Rating is available to allow for additional transmission capacity for use in congestion management, if an EEA Level 2 or 3 is declared, and post-contingency actions can be taken within 15 minutes to return the flow to within the Emergency Rating.  Such actions may include, but are not limited to, reducing the generation that increased output as a result of enforcing the 15-Minute Rating rather than the Emergency Rating;

(ii)	Post-contingency loading of the Transmission Facilities is expected to be at or below Normal Rating within two hours; or

(iii)	Additional transmission capacity could allow for additional output from a limited Generation Resource by taking one of the following actions:

(A)	Restoring Transmission Elements that are out of service;

(B) 	Reconfiguring the transmission system; or

(C)	Making adjustments to phase angle regulator tap positions.

If ERCOT determines that one of the above-mentioned actions allows for additional output from a limited Generation Resource, ERCOT may instruct the TSPs to take the action(s) during the Advisory to allow for additional output from the limited Generation Resource.

(b) 	ERCOT shall also coordinate with TSPs who own and operate the Transmission Facilities associated with the double-circuit contingencies for the constraints identified above to determine whether the double-circuit failures are at a high risk of occurring due to system conditions, which may include: severe weather conditions forecasted by ERCOT in the vicinity of the double circuit, weather conditions that indicate a high risk of insulator flashover on the double circuit, repeated Forced Outages of the individual circuits that are part of the double circuit in the preceding 48 hours, or fire in progress in the right of way of the double circuit.

(c)	The actions detailed in this Section shall be supplemental to the development and maintenance of CMPs as otherwise directed by the Protocols or Operating Guides.