| NPRR Number | 1150 | NPRR Title | Related to NOGRR230, WAN Participant Security |
| --- | --- | --- | --- |
| Date Posted |  | September 28, 2022 |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Sections Requiring Revision |  | 16.2.1, Criteria for Qualification as a Qualified Scheduling Entity |  |
| Related Documents Requiring Revision/Related Revision Requests |  | Nodal Operating Guide Revision Request (NOGRR) 230, WAN Participant Security |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds a requirement in Section 16.2.1 that states if a Qualified Scheduling Entity (QSE) represents a Resource Entity, Emergency Response Service (ERS) Resource, or another QSE and receives or transmits Wide Area Network (WAN) Data, as that term is defined in Nodal Operating Guide Section 1.4, Definitions, then it must maintain connection to the ERCOT WAN and a Secure Private Network (SPN) as described in Nodal Operating Guide Section 7, Telemetry and Communication. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This NPRR accompanies NOGRR230 which addresses requirements relating to the WAN Participants and the use of an SPN. |  |

| Sponsor |  |
| --- | --- |
| Name | Mike Allgeier |
| E-mail Address | Michael.Allgeier@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4750 |
| Cell Number |  |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jordan Troublefield |
| E-Mail Address | Jordan.Troublefield@ercot.com |
| Phone Number | 512-248-6521 |

Market Rules Notes

Please note that the following NPRR(s) also propose revisions to the following

section(s):

- NPRR1067, Market Entry Qualifications, Continued Participation Requirements, and Credit Risk Assessment

  - Section 16.2.1

- NPRR1127, Clarification of ERCOT Hotline Uses

  - Section 16.2.1

Proposed Protocol Language Revision

- 16.2.1	Criteria for Qualification as a Qualified Scheduling Entity

(1)	To become and remain a Qualified Scheduling Entity (QSE), an Entity must meet the following requirements:

(a)	Submit a properly completed QSE application for qualification, including any applicable fee, necessary disclosures, and designation of Authorized Representatives, each of whom is responsible for administrative communications with the QSE and each of whom has enough authority to commit and bind the QSE and the Entities it represents;

(b)	Sign a Standard Form Market Participant Agreement;

(c)	Sign any required Agreements relating to use of the ERCOT network, software, and systems;

(d)	Demonstrate to ERCOT’s reasonable satisfaction that the Entity is capable of performing the functions of a QSE;

(e)	Demonstrate to ERCOT’s reasonable satisfaction that the Entity is capable of complying with the requirements of all ERCOT Protocols and Operating Guides;

(f)	Satisfy ERCOT’s creditworthiness and capitalization requirements as set forth in this Section, unless exempted from these requirements by Section 16.17, Exemption for Qualified Scheduling Entities Participating Only in Emergency Response Service;

(g)	Be generally able to pay its debts as they come due.  ERCOT may request evidence of compliance with this qualification only if ERCOT reasonably believes that a QSE is failing to comply with it;

(h)	Provide all necessary bank account information and arrange for Fedwire system transfers for two-way confirmation;

(i)	Be financially responsible for payment of Settlement charges for those Entities it represents under these Protocols;

(j)	Comply with the backup plan requirements in the Operating Guides;

(k)	Maintain a 24-hour, seven-day-per-week scheduling center with qualified personnel for the purposes of communicating with ERCOT relating to Day-Ahead and Operating Day exchange of market and operational obligations in representing Load, Resources, and market positions.  Those personnel must be responsible for operational communications and must have sufficient authority to commit and bind the QSE and the Entities that it represents;

(l)	Demonstrate and maintain a working functional interface with all required ERCOT computer systems;

(m)	Allow ERCOT, upon reasonable notice, to conduct a site visit to verify information provided by the QSE; and

(n)	If a QSE represents a Resource Entity, Emergency Response Service (ERS) Resource, or another QSE and receives or transmits Wide Area Network (WAN) Data, as that term is defined in Nodal Operating Guide Section 1.4, Definitions, it must maintain connection to the ERCOT WAN and a Secure Private Network (SPN) as described in Nodal Operating Guide Section 7, Telemetry and Communication.

(2)	If a QSE chooses to use Electronic Data Interchange (EDI) transactions to receive Settlement Statements and Invoices, it must participate in and successfully complete testing as described in Section 19.8, Retail Market Testing, before starting operations with ERCOT as a QSE.

(3)	A QSE or QSE applicant must be able to demonstrate to ERCOT’s reasonable satisfaction that none of its Principals were or are Principals of any Entity with an outstanding payment obligation that remains owing to ERCOT under any Agreement or these Protocols.  For purposes of this Section, ERCOT will only consider disqualifying those Principals of the QSE or QSE applicant who were Principals of the other Entity at a time during which the unpaid financial obligation remained owing to ERCOT or during the 120-day period prior to the date on which the unpaid financial obligation first became due and owing to ERCOT.

(4)	If any of a QSE’s or QSE applicant’s Principals were or are Principals of a terminated Market Participant with an obligation for Default Uplift Ratio Share allocated under Section 9.19.1, Default Uplift Invoices, the terminated Market Participant must be current on all payment obligations for Default Uplift Invoices in order for the QSE to remain, or QSE applicant to become, a registered QSE.  For purposes of this Section, ERCOT will only consider as disqualifying those Principals of the QSE or QSE applicant who were Principals of the other Entity at a time during which the other Entity was not current on its payment obligation for Default Uplift Invoices or 120 days prior to the date the other Entity first failed to pay a Default Uplift Invoice.

(5)	A QSE shall promptly notify ERCOT of any change that a reasonable examiner may deem material to the QSE’s ability to continue to meet the requirements set forth in this Section, and any material change in the information provided by the QSE to ERCOT that may adversely affect the reliability or safety of the ERCOT System or the financial security of ERCOT.  This includes any changes in the Principals of the QSE.  If the QSE fails to so notify ERCOT of such change within two Business Days after becoming aware of the change, then ERCOT may, after providing notice to each Entity represented by the QSE, refuse to allow the QSE to perform as a QSE and take any other action ERCOT deems appropriate, in its sole discretion, to prevent ERCOT or Market Participants from bearing potential or actual risks, financial or otherwise, arising from those changes, and in accordance with these Protocols.

(6)	Subject to the following provisions of this paragraph, a QSE may partition itself into any number of subordinate QSEs (“Subordinate QSEs”).  If a single Entity requests to partition itself into more than four Subordinate QSEs, ERCOT may implement the request subject to ERCOT’s reasonable determination that the additional requested Subordinate QSEs will not be likely to overburden ERCOT’s staffing or systems.  ERCOT shall adopt an implementation plan allowing phased-in registration for these additional Subordinate QSEs in order to mitigate system or staffing impacts.  However, ERCOT may not unreasonably delay that registration.

(7)	Each Subordinate QSE must be treated as an individual QSE for all purposes including communications and control functions except for liability, financial security, and financial liability requirements under this Section.  That liability, financial security, and financial liability is cumulative for all Subordinate QSEs for the single Entity signing the QSE Agreement.

(8)	Continued qualification as a QSE is contingent upon compliance with all applicable requirements in these Protocols.  ERCOT may suspend a QSE’s rights as a Market Participant when ERCOT reasonably determines that it is an appropriate remedy for the Entity’s failure to satisfy any applicable requirement.